3GPP TSG-RAN3 Meeting #107bis-e	R3-202342

E-Meeting, 20 – 30 April, 2020

Title: 	(TP for NPN BL CR for TS 38.423): Inactive mobility for NPN

Source: 	Huawei

Agenda item:	16.4

Document for:		Discussion

Annex – TP for TS 38.423 (on the top of BL R3-201593)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

8.2.4	Retrieve UE Context

8.2.4.1	General

The purpose of the Retrieve UE Context procedure is to either retrieve the UE context from the old NG-RAN node and transfer it to the NG-RAN node where the UE RRC Connection has been requested to be established, or to enable the old NG-RAN node to forward an RRC message to the UE via the new NG-RAN node without context transfer.

The procedure uses UE-associated signalling.

8.2.4.2	Successful Operation

Figure 8.2.4.2-1: Retrieve UE Context, successful operation

The new NG-RAN node initiates the procedure by sending the RETRIEVE UE CONTEXT REQUEST message to the old NG-RAN node.

If the old NG-RAN node is able to identify the UE context by means of the UE Context ID, and to successfully verify the UE by means of the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message, and decides to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT RESPONSE message. For PNI-NPN UE, the new NG-RAN node checks that the new NG-RAN node cell is successfully compatible with the PNI-NPN mobility restrictions received from the old NG-RAN node, and respond with the RETRIEVE UE CONTEXT RESPONSE message. For SNPN UE, the new NG-RAN node checks that the supported SNPN(s) of the new NG-RAN node cell successfully matches the serving SNPN included in the Mobility Restriction List from the old NG-RAN node, and responds with the RETRIEVE UE CONTEXT RESPONSE message.

If the Index to RAT/Frequency Selection Priority IE is contained in the RETRIEVE UE CONTEXT RESPONSE message, the new NG-RAN node shall store this information and use it as defined in TS 23.501 [7].

If the Location Reporting Information IE is included in the RETRIEVE UE CONTEXT RESPONSE message, then the new NG-RAN node should initiate the requested location reporting functionality as defined in TS 38.413 [5].

8.2.4.3	Unsuccessful Operation

Figure 8.2.4.3-1: Retrieve UE Context, unsuccessful operation

If the old NG-RAN node is not able to identify the UE context by means of the UE Context ID, or if the integrity protection contained in the RETRIEVE UE CONTEXT REQUEST message is not valid, or, if it decides not to provide the UE context to the new NG-RAN node, it shall respond to the new NG-RAN node with the RETRIEVE UE CONTEXT FAILURE message.

If the old NG-RAN node decides to keep the UE context in case of periodic RNAU, it shall store the Allocated C-RNTI IE and the Access PCI IE in the UE Context ID IE, as described in TS 38.300 [9].

If the Old NG-RAN node to New NG-RAN node Resume Container IE is included in the RETRIEVE UE CONTEXT FAILURE message, the new NG-RAN node should transparently forward the content of this IE to the UE as described in TS 38.300 [9].

For PNI-NPN UE, the old NG-RAN node may respond to the new NG-RAN node with RETRIEVE UE CONTEXT FAILURE message when the supported CAG List related to the serving PLMN of the new NG-RAN node cell is not compatible with the UE’s PNI-NPN mobility restrictions.

The old NG-RAN node may respond to the new NG-RAN node with RETRIEVE UE CONTEXT FAILURE message when the supported SNPN(s) of the new NG-RAN node does not match the serving SNPN included in the Mobility Restriction List from the old NG-RAN node.

8.2.4.4	Abnormal Conditions

Void.

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>