3GPP TSG-RAN3 Meeting #107bis-e	R3-202356

E-Meeting, 20 – 30 April, 2020

Title: 	(TP for NPN BL CR for TS 38.463): Slicing aspects for NPN

Source: 	Huawei

Agenda item:	16.9

Document for:	Discussion

Annex – TP for TS 38.463 (on the top of BL R3-201594)

<<<<<<<<<<<<<<<<<<<< Changes Begin >>>>>>>>>>>>>>>>>>>>

9.3.1.x5	NPN Information

This IE provides information NPN related information.

Editor’s Note 1:	PNI-NPN aspects need to be further discussed.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE NPN Information | M |  |  |  |
| >SNPN Information |  | 1 |  |  |
| >>NID Support List |  | 1..<maxnoofNIDsupported> |  |  |
| >>>NID | M |  | 9.3.1.x1 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofNIDsupported | Maximum no. of NIDs supported. Value is 12 (FFS). |

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.5	Information Element Definitions

<Unchanged Text Omitted>

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofNIDsupported

<Unchanged Text Omitted>

#### -- N

NID	::= OCTET STRING (SIZE (7)) (FFS)

NID-SupportList ::= SEQUENCE (SIZE(1..maxnoofNIDsupported)) OF NID

NPNSupportInfo ::= CHOICE {

sNPN		        NID-SupportList,

choice-extension	ProtocolIE-SingleContainer	{{NPNSupportInfo-ExtIEs}}

}

NPNSupportInfo-ExtIEs E1AP-PROTOCOL-IES ::= {

...

}

SNPN ::= SEQUENCE {

nID	                    NID,

iE-Extensions			ProtocolExtensionContainer { { SNPN-ExtIEs } }	OPTIONAL

}   (FFS)

SNPN-ExtIEs	E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>

9.4.7	Constant Definitions

<Unchanged Text Omitted>

maxnoofNIDsupported								INTEGER ::= 12

<<<<<<<<<<<<<<<<<<<< Changes End >>>>>>>>>>>>>>>>>>>>