3GPP TSG-RAN WG3 #108-e						R3-204163

E-meeting, 1 – 11 June 2020

Agenda Item:  15.3.1.4

Source:       NEC

Title:		(TP for NR_Mob_enh BL CR for TS 38.423) Conditional HO Cancel procedure and HO Success procedure in 38.423

Document for:	discussion

- 1. Introduction

The baseline CR of 38.423 for CHO has included Handover Success procedure.

An editor note say

“Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover.

Editor's note: The impact on the NG-RAN node's behaviour should be verified if the source NG-RAN node receives the HANDOVER SUCCESS message for a UE from the target NG-RAN node after receiving the CONDITIONAL HANDOVER CANCEL message for the same UE from the same target NG-RAN node.

”

After the RAN3#108-e e-mail discussion in CB: # 39_MobEnh_CHOcancel, agree to keep the interaction description, and only to remove the Editor’s note and text.

- 2. TP

- Annex // TP for TS38.423 BLCR (based on R3-202909)

### 8.2.A	Handover Success

#### 8.2.A.1	General

The Handover Success procedure is used during a conditional handover or a DAPS handover, to enable a target NG-RAN node to inform the source NG-RAN node that the UE has successfully accessed the target NG-RAN node.

The procedure uses UE-associated signalling.

#### 8.2.A.2	Successful Operation

Figure 8.2.A.2-1: Handover Success, successful operation

The target NG-RAN node initiates the procedure by sending the HANDOVER SUCCESS message to the source NG-RAN node.

If late data forwarding was configured for this UE, the source NG-RAN node shall start data forwarding using the tunnel information related to the global target cell ID provided in the HANDOVER SUCCESS message.

When the source NG-RAN node receives the HANDOVER SUCCESS message, it shall consider all other CHO preparations accepted for this UE in the target NG-RAN node as cancelled and may initiate Handover Cancel procedure towards other candidate target NG-RAN nodes for this UE, if any.

Interactions with other procedures

If a CONDITIONAL HANDOVER CANCEL message was received for this UE prior the reception of the HANDOVER SUCCESS message, the source NG-RAN node shall consider that the UE successfully executed the handover.

#### 8.2.A.3	Unsuccessful Operation

Not applicable.

#### 8.2.A.4	Abnormal Conditions

If the HANDOVER SUCCESS message refers to a context that does not exist, the source NG-RAN node shall ignore the message.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////