3GPP TSG-RAN3 #108-e	R3-204068

1st June – 12th June 2020

Online

Agenda Item:	09.3.8.1

Source:	Ericsson

Title:	SoD on Measurement gap deactivation over F1AP.

Document for:	Discussion, Decision

# Introduction

In the previous meeting the issue of Measurement Gap deactivation was brought up in [1] and [2]. In this contribution we capture offline discussions on this issue as part of the following CB:

CB: # 90_MeasGapDeactivation

-  need to consider when measgap does not include measgaptoaddlist IE

- further check details

(E/// - moderator)

rev in R3-204066

3385 rev in R3-204067

Summary of offline disc R3-204068

# For the Chairman’s Notes

Following agreements were proposed on the first round of offline discussion:

# Discussion

- The use case we would like to analyse is the following. The gNB-DU has some specific measurement gaps configured. At some point though gNB-CU realizes some of those measurement gaps are no longer needed and wants them to be deactivated. The question we are trying to answer is how the gNB-CU informs the gNB-DU about its decision.

- In TS 38.473 it can be seen that the IE used by the gNB-CU to inform gNB-DU of measurement gaps to be configured is the MeasConfig IE included in the CU to DU RRC Information IE.

MeasConfig IE consists of two types of information, the MeasObjectToAddModList IE, which specifies the list of frequencies for which the gNB-CU requests the gNB-DU to generate gaps and measObjectToRemoveList IE, which specifies a list of Measurement Object IDs for which measurement gaps shall be deactivated.  However, TS38.473 does not provide guidance about which IE to use to deactivate gaps.

The proposal in [3] is to agree to a solution in line with the principles below

• gNB-CU indicates to gNB-DU that measurements gaps are to be activated or deactivated via including MeasConfig IE in CU to DU RRC Information IE

• Measurement gaps for frequencies included in MeasObjectToAddModList IE are to be activated

• Measurement gaps for frequencies previously configured at gNB-DU, if not included in MeasObjectToAddModList IE, are to be deactivated

• If gNB-CU includes MeasConfig IE, but it does not contain MeasObjectToAddModList IE, this is understood by the gNB-DU as a request to deactivate all the existing configured measurement gaps

• If the gNB-DU receives CU to DU RRC Information IE without the MeasConfig IE, it does not apply any change to the existing configured measurement gaps

The solution above allows a gNB-DU to be state-less with respect to measurement gaps. Namely, a gNB-DU does not need to store the frequency and cell for which a gap was assigned. Instead,, the gNB-DU adds/removes/modifies gaps purely on the basis of the information in MeasObjectToAddModList IE.

Companies are invited to express their view on the problem and solutions above

Is the problem acknowledged:

| Company | ACK/NACK | Comments |
| --- | --- | --- |
| Ericsson | ACK |  |
| Nokia | ACK |  |

Comments on solution:

| Company | Comments |
| --- | --- |
| Ericsson | A clarification is needed in terms of how the gNB-CU signals deactivation of measurement gaps. The simplest was to do so is to allow for state-less measurement gap configuration at the gNB-DU as proposed in [1],[2],[3]. |
| Nokia | Change is needed in our view. The item which we believe needs update in the CR in [3], is in order to account the following condition “• If gNB-CU includes MeasConfig IE, but it does not contain MeasObjectToAddModList IE, this is understood by the gNB-DU as a request to deactivate all the existing configured measurement gaps” |

# Conclusion

# References

[1] R3-201909, “Discussion on Measurement Gap Deactivation”, Nokia , Nokia Shanghai Bell

[2] R3-201912, “Measurement Gap Deactivation”,  Nokia , Nokia Shanghai Bell

[3] R3-203384, Measurement gap deactivation over F1AP CR 38.473 (Ericsson)