3GPP TSG-RAN3 Meeting #108-e	R3-203718

E-meeting, 01 - 11 June 2020

Title: 	(TP for NR_IIOT BL CR for TS 38.473): Proposed rapporteur changes to the BL CR

Source: 	Huawei

Agenda item:	17.1

Document for:	Other

1. Annex – TP

Change Begins

9.3.1.XX	Reporting Request Type

This IE indicates the type of accurate reference time information reporting to be handled by the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Event Type | M |  | ENUMERATED (on demand, periodic, stop, …) |  |
| Report Periodicity Value | C-ifEventTypeisPeriodic |  | INTEGER (0..512, …) | Indicates the periodicity of accurate reference time information report. Unit in radio frame. |

| C-ifEventYpeisStop | Explanation |
| --- | --- |
| ifEventTypeisPeriodic | This IE shall be present if the Event Type IE is set to "periodic". |

<Unchanged Texts Omitted>

9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

<Unchanged Texts Omitted>

ReportingPeriodicityValue ::= INTEGER (0..512, ...)

Periodicity ::= INTEGER (0..640000, ...)

<Unchanged Texts Omitted>

ReportingRequestType ::= SEQUENCE {

eventType						EventType,

reportingPeriodicityValue							ReportingPeriodicityValue		OPTIONAL,

-- C-ifEventTypeisPeriodic: This IE shall be present if the Event Type IE is set to "periodic" in the Event Type IE.

iE-Extensions					ProtocolExtensionContainer { {ReportingRequestType-ExtIEs} }	OPTIONAL

}

ReportingRequestType-ExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

End Change