3GPP TSG-RAN WG3 Meeting #108-e	R3-204142

# Online, 1st – 11th June 2020

Agenda Item:	17.4

Source:	Ericsson

Title:	EHC updates on E1AP TP

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we propose to update the EHC configuration IE in E1AP so that it is synched with RAN 2 specifications.

The wording, “shall, if supported” is changed to “may” related to gNB-CU-U action.

# 2	Discussion

A number of EHC related parmeters have been changed in RAN2 [1] associated with the Ethernet Header Compression .configuration.

ethernetHeaderCompression-r16  CHOICE {

notUsed                 NULL,

ehc                     SEQUENCE {

ehc-Common              SEQUENCE {

ehc-CID-Length	          ENUMERATED { bits7, bits15 },

...

},

ehc-Downlink            SEQUENCE {confb

drb-ContinueEHC-DL      ENUMERATED { true }                                     OPTIONAL,   -- Need N

...

}                                                                                   OPTIONAL,   -- Need M

ehc-Uplink              SEQUENCE {

drb-ContinueEHC-UL      ENUMERATED { true }                                     OPTIONAL,   -- Need N

...

}                                                                                   OPTIONAL,   -- Need M

...

},

...

}                                                                                           OPTIONAL    -- Cond DRB2

Among the changes applicable to the RAN3 specifications it can be seen that the “ehc-HeaderSize” has be to “ehc-CID-Length” while the its values set has been changed to ENUMERATED { bits7, bits15 }

These changes should be carried to the EHC BL CR for 38.463

- Update the EHC Parameters IE in 38.463 to sync them  to the latest RAN2 specification

# 3	Conclusion

Agree on the proposals below

- Update the EHC Parameters IE in 38463 to sync them to the latest RAN2 specification

# 4	References

- draft R2-2002703 Correction of NR IIoT 38331_v7

# TP for EHC BL CR for TS 38.463

### 8.3.1	Bearer Context Setup

#### 8.3.1.1	General

The purpose of the Bearer Context Setup procedure is to allow the gNB-CU-CP to establish a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.1.2	Successful Operation

Figure 8.3.1.2-1: Bearer Context Setup procedure: Successful Operation.

The gNB-CU-CP initiates the procedure by sending the BEARER CONTEXT SETUP REQUEST message to the gNB-CU-UP. If the gNB-CU-UP succeeds to establish the requested resources, it replies to the gNB-CU-CP with the BEARER CONTEXT SETUP RESPONSE message.

//skip the unchanged part

For EN-DC, if the Subscriber Profile ID for RAT/Frequency priority IE is included in the UE CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25]. If the Additional RRM Policy Index IE is included in the UE CONTEXT SETUP REQUEST, the gNB-CU-UP may use it to apply specific RRM policies as specified in TS 36.300 [25].

For each QoS flow whose DRB has been successfully established and the QoS Monitoring Request IE was included in the QoS Flow Level QoS Parameters IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP shall store this information, and, if supported, perform delay measurement and QoS monitoring, as specified in TS 23.501 [20].

For each requested DRB, if the EHC Parameters IE is included in the PDCP Configuration IE, the gNB-CU-CP shall, if supported, also include ROHC Parameters IE in the PDCP Configuration IE in the BEARER CONTEXT SETUP REQUEST message, to enable the gNB-CU-UP to perform appropriate header compression.

If the EHC parameters IE is included in the PDCP Configuration IE contained in the BEARER CONTEXT SETUP REQUEST message, the gNB-CU-UP may take these parameters into account to perform appropriate header compression for the concerned DRB.

#### <<<<<<<<<<<<<<<<<<<< Next Changes >>>>>>>>>>>>>>>>>>>>

#### 9.3.1.xx		EHC Parameters

This IE carries the EHC parameters for ethernet header compression.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| EHC Common | M |  |  |  |
| >EHC- CID-Length | M |  | ENUMERATED { bits7, bits15, … } | See description of ehc- CID-Length in TS 38.331 [10] |
| EHC Downlink | O |  |  |  |
| >drb-ContinueEHC-DL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-DL in TS 38.331 [10] |
| EHC Uplink | O |  |  |  |
| >drb-ContinueEHC-UL | M |  | ENUMERATED { true, … } | See description of drb-ContinueEHC-UL in TS 38.331 [10] |

<<<<<<<<<<<<<<<<<<<< End of 1st Change >>>>>>>>>>>>>>>>>>>>

-- TEXT OMITTED –

### 9.4.5	Information Element Definitions

-- TEXT OMITTED –

<<<<<<<<<<<<<<<<<<<< 2nd Change >>>>>>>>>>>>>>>>>>>>

EHC-Common-Parameters ::= SEQUENCE {

ehc-CID-Length						ENUMERATED { bits7, bits15, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Common-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Common-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Downlink-Parameters ::= SEQUENCE {

drb-ContinueEHC-DL					ENUMERATED { true, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Downlink-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Downlink-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Uplink-Parameters ::= SEQUENCE {

drb-ContinueEHC-DL					ENUMERATED { true, ...},

iE-Extensions						ProtocolExtensionContainer { { EHC-Uplink-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Uplink-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

EHC-Parameters ::= SEQUENCE {

ehc-Common							EHC-Common-Parameters,

ehc-Downlink						EHC-DL-Parameters					OPTIONAL,

ehc-Uplink							EHC-UL-Parameters					OPTIONAL,

iE-Extensions						ProtocolExtensionContainer { { EHC-Parameters-ExtIEs } } 		OPTIONAL

}

EHC-Parameters-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

<<<<<<<<<<<<<<<<<<<< End of Changes >>>>>>>>>>>>>>>>>>>>