3GPP TSG-RAN3 Meeting #109-e	R3-205086

E-meeting, 17 - 28 August 2020

Title: 	Direct data forwarding for inter-system handover over E1

Source: 	Huawei

Agenda item:	31.3.3

Document for:	Discussion

1. Introduction

Last RAN3# meeting discussed the direct data forwarding for inter-system handover over E1 with the following summary [1].

Direct data forwarding from 5GS to EPS should be supported in case flow to DRB mapping is different with respect to flow to E-RAB mapping

Open Issue 1: The solution on how to support direct data forwarding from 5GS to EPS in case flow to DRB mapping is different with flow to E-RAB mapping needs to be further discussed.

Open issue 2: Whether/how CU-UP should be aware whether the data forwarding is for intra-system handover or inter-system handover needs further discussion.

Open issue 3: For inter-system HO from EPS to 5GS, whether direct data forwarding should be supported if one DRB contains QoS flows mapped to different E-RABs in the target gNB needs further discussion.

This contribution continues to address this issue.

2. Discussion

2.1 5GC to EPC handover

2.2.1 Problem description

As highlighted above, it comes to the common understanding that the QoS flow to DRB mapping could be different from the QoS flow to E-RAB mapping. For example, before the handover, at the source CU-CP side,

-	DRB 1 (QFI 1, QFI 2), DRB 2 (QFI 3)

-	E-RAB 1 (QFI1), DRB 2 (QFI2, QFI3)

So the question 1 here is: How to transmit and map these three fresh QoS flow packets (in terms of SDAP SDUs) over these two E-RAB tunnels?

As described in [2], another question2 is:  How the source CU-UP should be aware whether the DRB forwarding tunnel is used for intra-system handover, or the inter-system direct data forwarding since the forwarded packets are different for the following cases?

- For 5GC to EPC handover, the source CU-UP should forward the PDCP SDUs without PDCN SN or QFI for each packet via the DRB forwarding tunnel.

- For 5GC intra-system handover, in case of only DRB level data forwarding, the source CU-UP should forward PDCP PDUs including SDAP header which have not been acknowledged by the UE, as described in TS 38.300.

Also as described in TS 23.413, the optional E-RAB ID is provided per QoS flow to the NG-RAN node. Then it is possible that while some QoS flows are not associated with E-RAB ID, so that these QoS flows may not be forwarded during the 5GS to EPS handover. However, the source CU-UP has no knowledge what kinds of QoS flows are not configured with E-RAB ID, so as not to forward these QoS flow packets.

2.2.2 Candidate solutions

During the discussion, basically there are two potential solutions listed in [1].

Solution 1: CU-CP informs CU-UP of the flow to E-RAB mapping information during Bearer Context Setup procedure and informs CU-UP of the data forwarding address per E-RAB after it received HO COMMAND message. - Solution 1.1: CU-CP informs CU-UP of the flow to E-RAB mapping information and the data forwarding address per E-RAB during Bearer Context Modification procedure after it received HO COMMAND message Solution 2: After CU-CP receives HO COMMAND message, it indicates to the CU-UP the per-E-RAB tunnel assigned by the target eNB using the DRB information in Bearer Context Modification Request message.

For solution 2, it requires to setup new DRBs, or modify the new QoS flow to DRB mapping during the handover procedure. This could possible work, but if should further study:

- It can not address the question 2 above. Though it is possible for the target eNB to determine to discard the forwarded packets. But the root problem is that source CU-UP can not know which kind of packets should be forwarded.

- It can not address question 3 above easily. This requires the CU-CP to request to release some QoS flows not configured with the E-RAB ID.

Compared with solution 1, solution 1.1 allows to indicate the necessary information only in case of bearer modification procedure. Hence it is preferred. We have the following proposal.

Solution 1.1 is selected for direct data forwarding for 5GC to EPC handover.

2.2 EPC to 5GC handover

- For 5GC to EPC handover, several examples are provided in [2]. One of example is that:

- In the souce:                    E-RAB1: QFI1 & QFI2,      E-RAB2: QFI3

- The target gNB decides: New DRB1: QFI1,               New DRB2: QFI2 & QFI3

- So the question is that whether the target CU-UP needs to be aware of the QoS flow to E-RAB mapping for data forwarding in the BEARER CONTEXT SETUP REQUEST message.

- The CU-CP can be flexibly to have two options as follows.

- Option 1: QoS flow remapping happens during the HO as indicated in the above example:

  - Two DRBs are setup with QFI1 is mapped to DRB1, and QF2/QFI3 are maped to DRB2, signalled by the CU-CP;

  - The CU-UP can first transmit the forwarded packets on the old two DRBs before transmitting new data from 5GCN on the new DRBs. This is similar to the intra-system handover to ensure lossless delivery.

- Option 2: no QoS flow remapping happens during the HO, but after the HO.

- In above two options, the CU-CP can make final decision of QoS flow to DRB mapping. And there is no need for the CU-UP to be aware of the QoS flow to E-RAB mapping.

No singalong impact for direct data forwarding for EPC to 5GC handover.

3. Conclusion

This contribution continuously discuss the direct data forwarding for inter-system handover with the following proposals.

- Solution 1.1 is selected for direct data forwarding for 5GC to EPC handover.

No singalong impact for direct data forwarding for EPC to 5GC handover.

The CRs in [4] and [5] implement the proposal 1.

4. Reference

- R3-204354 Summary of email discussion of CB: # 85bis_InterSysDataFwd_E1_aspects, CATT (moderator)

- R3-203623 Direct data forwarding for Inter-system HO in CP-UP separation, Samsung

- R3-203699 Futher clarification on support of inter-system direct data forwarding for CP/UP separation scenario, CATT, Nokia, Nokia Shanghai Bell,China Telecom,ZTE

- R3-205087	Direct data forwarding for inter-system handover over E1, Huawei

- R3-205088	Direct data forwarding for inter-system handover over E1, Huawei