3GPP TSG-RAN WG3 #109	R3-205606

# Online, 17th - 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.455 | CR | 0111 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introducing NR beam level measurement in inter-RAT measurement in E-CID measurement over LPPa. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This CR adds per SS beam index measurement in the E-CID measurement NR results. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding per SS beam index measurement in the inter-RAT measurement results for NR. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because this is additional information that may be used by receiving node This CR has no impact from functional point of view. The impact can be considered isolated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | E-CID measurement based positioning accuracy is lower compared with the SS beam index measurement being included in the E-CID measurement NR results. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.13, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: - the CR title is revised. Rev2: - SSB RSRP/Q is revised to SS RSRP/Q - ResultsPerSSB-index-item sequence is revised as extensible - consequences if not approved is revised to be specific |  |  |  |  |  |  |  |  |

START OF CHANGES

9.2.13	Inter-RAT Measurement Result

The purpose of the Inter-RAT Measurement Result information element is to provide the Inter-RAT measurement results.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description | Criticality | Assigned criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Inter-RAT Measured Results |  | 1.. <maxnoMeas> |  |  | - |  |
| >CHOICE Inter-RAT Measured Results Value | M |  |  |  | - |  |
| >>Result GERAN | M | 1..<maxGERANMeas> |  |  | - |  |
| >>>ARFCN of BCCH | M |  | INTEGER (0..1023, ...) |  | - |  |
| >>>Physical CellId GERAN | M |  | INTEGER (0..63, ...) |  | - |  |
| >>>RSSI | M |  | INTEGER(0..63, ...) |  | - |  |
| >>Result UTRAN |  | 1..<maxUTRANMeas> |  |  | - |  |
| >>>UARFCN | M |  | INTEGER (0..16383, ...) |  | - |  |
| >>>CHOICE Physical CellId UTRA | M |  |  |  | - |  |
| >>>>Physical CellId UTRA FDD | M |  | INTEGER (0..511, ...) |  | - |  |
| >>>>Physical CellId UTRA TDD | M |  | INTEGER (0..127, ...) |  | - |  |
| >>>UTRA RSCP | O |  | INTEGER(-5..91, ...) |  | - |  |
| >>>UTRA EcNo | O |  | INTEGER(0..49, ...) | This IE applies to FDD only. | - |  |
| >>Result NR | M | 1..<maxNRMeas> |  |  | - |  |
| >>>NR ARFCN | M |  | INTEGER (0.. 3279165) |  | - |  |
| >>>NR PCI | M |  | INTEGER (0..1007) |  | - |  |
| >>>NR SS-RSRP | O |  | INTEGER (0..127) | Cell level measurement result of NR SS-RSRP | - |  |
| >>>NR SS-RSRQ | O |  | INTEGER (0..127) | Cell level measurement result of NR SS-RSRQ | - |  |
| >>>ResultsPerSSB Index List |  | 0..1 |  |  | YES | ignore |
| >>>>ResultsPerSSB Index Item |  | 1..<maxResultsPerSSBIndex> |  |  |  |  |
| >>>>>SSB Index | M |  | INTEGER (0..63) |  | - |  |
| >>>>>NR SS-RSRP beam value | O |  | INTEGER (0..127) | Beam level measurement result of NR SS-RSRP | - |  |
| >>>>>NR SS-RSRQ beam value | O |  | INTEGER (0..127) | Beam level measurement result of NR SS-RSRQ | - |  |

| Range bound | Explanation |
| --- | --- |
| maxnoMeas | Maximum no. of measured quantities that can be configured and reported with one message. Value is 63. |
| maxGERANMeas | Maximum no. of GERAN cells that can be reported with one message. Value is 8. |
| maxUTRANMeas | Maximum no. of UTRAN cells that can be reported with one message. Value is 8. |
| maxNRMeas | Maximum no. of NR cells that can be reported with one message. Value is 32. |
| maxResultsPerSSBIndex | Maximum no. of NR SSB indices that can be reported with one message. Value is 64. |

NEXT CHANGE

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

LPPA-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

eps-Access (21) modules (3) lppa (6) version1 (1) lppa-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-MeasurementQuantities-Item,

id-ResultsPerSSB-Index-List,

id-ResultsPerSSB-Index-Item,

maxCellineNB,

maxCellReport,

maxNrOfErrors,

maxNoMeas,

maxnoOTDOAtypes,

maxServCell,

id-InterRATMeasurementQuantities-Item,

id-WLANMeasurementQuantities-Item,

maxGERANMeas,

maxUTRANMeas,

maxNRmeas,

maxResultsPerSSBIndex,

maxCellineNB-ext,

maxWLANchannels,

maxMBSFN-Allocations,

maxnoFreqHoppingBandsMinusOne,

maxNrOfPosSImessage,

maxnoAssistInfoFailureListItems,

maxNrOfSegments,

maxNrOfPosSIBs

FROM LPPA-Constants

NEXT CHANGE

UNCHANGED TEXT OMMITED

ResultNR ::= SEQUENCE (SIZE (1.. maxNRmeas)) OF ResultNR-Item

ResultNR-Item ::= SEQUENCE {

nRARFCN				NRARFCN,

nRPCI				NRPCI,

sS-NRRSRP			SS-NRRSRP	OPTIONAL,

sS-NRRSRQ			SS-NRRSRQ	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { { ResultNR-Item-ExtIEs} } OPTIONAL,

...

}

ResultNR-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

{ ID id-ResultsPerSSB-Index-List							CRITICALITY ignore	EXTENSION ResultsPerSSB-Index-List					PRESENCE optional},

...

}

ResultsPerSSB-Index-List ::= SEQUENCE (SIZE(1..maxResultsPerSSBIndex)) OF ResultsPerSSB-Index-Item

ResultsPerSSB-Index-Item ::=SEQUENCE {

sSB-Index					SSB-Index,

sS-NRRSRPBeamValue			SS-NRRSRP,			OPTIONAL,

sS-NRRSRQBeamValue			SS-NRRSRQ, 				OPTIONAL,

iE-Extensions				ProtocolExtensionContainer { { ResultsPerSSB-Index-Item-ExtIEs} }	OPTIONAL,

...

}

ResultsPerSSB-Index-Item-ExtIEs LPPA-PROTOCOL-EXTENSION ::= {

...

}

RSSI ::= INTEGER (0..63, ...)

#### -- S

SFNInitialisationTime ::= BIT STRING (SIZE (64))

SRSConfigurationForAllCells ::= SEQUENCE (SIZE (1.. maxServCell)) OF SRSConfigurationForOneCell

SRSConfigurationForOneCell ::= SEQUENCE {

pci							PCI,

ul-earfcn					EARFCN,

ul-bandwidth				ENUMERATED {n6, n15, n25, n50, n75, n100},

ul-cyclicPrefixLength		CPLength,

srs-BandwidthConfig			ENUMERATED {bw0, bw1, bw2, bw3, bw4, bw5, bw6, bw7},

srs-Bandwidth				ENUMERATED {bw0, bw1, bw2, bw3},

srs-AntennaPort				ENUMERATED {an1, an2, an4, ...},

srs-HoppingBandwidth		ENUMERATED {hbw0, hbw1, hbw2, hbw3},

srs-cyclicShift				ENUMERATED {cs0, cs1, cs2, cs3, cs4, cs5, cs6, cs7},

srs-ConfigIndex				INTEGER (0..1023),

maxUpPts					ENUMERATED {true}			OPTIONAL,	-- Cond ifTDD

transmissionComb			INTEGER (0..1),

freqDomainPosition			INTEGER (0..23),

groupHoppingEnabled			BOOLEAN,

deltaSS						INTEGER (0..29)				OPTIONAL,

sfnInitialisationTime		SFNInitialisationTime,

...

}

Subframeallocation ::= CHOICE {

oneFrame						BIT STRING (SIZE(6)),

fourFrames						BIT STRING (SIZE(24))

}

SS-NRRSRP ::= INTEGER (0..127)

SS-NRRSRQ ::= INTEGER (0..127)

SSB-Index ::= INTEGER (0..63)

NEXT CHANGE

9.3.7	Constant definitions

UNCHANGED TEXT OMMITED

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNrOfErrors								INTEGER ::= 256

maxCellineNB								INTEGER ::= 256

maxNoMeas									INTEGER ::= 63

maxCellReport								INTEGER ::= 9

maxnoOTDOAtypes								INTEGER ::= 63

maxServCell									INTEGER ::= 5

maxGERANMeas								INTEGER ::= 8

maxUTRANMeas								INTEGER ::= 8

maxCellineNB-ext							INTEGER ::= 3840

maxMBSFN-Allocations						INTEGER ::= 8

maxWLANchannels								INTEGER ::= 16

maxnoFreqHoppingBandsMinusOne				INTEGER ::= 7

maxNrOfPosSImessage							INTEGER ::= 32

maxnoAssistInfoFailureListItems				INTEGER ::= 32

maxNrOfSegments								INTEGER ::= 64

maxNrOfPosSIBs								INTEGER ::= 32

maxNRmeas									INTEGER ::= 32

maxResultsPerSSBIndex						INTEGER	::=	64

UNCHANGED TEXT OMMITED

-- **************************************************************

--

#### -- IEs

--

-- **************************************************************

id-Cause																	ProtocolIE-ID ::= 0

id-CriticalityDiagnostics													ProtocolIE-ID ::= 1

id-E-SMLC-UE-Measurement-ID													ProtocolIE-ID ::= 2

id-ReportCharacteristics													ProtocolIE-ID ::= 3

id-MeasurementPeriodicity													ProtocolIE-ID ::= 4

id-MeasurementQuantities													ProtocolIE-ID ::= 5

id-eNB-UE-Measurement-ID													ProtocolIE-ID ::= 6

id-E-CID-MeasurementResult													ProtocolIE-ID ::= 7

id-OTDOACells																ProtocolIE-ID ::= 8

id-OTDOA-Information-Type-Group												ProtocolIE-ID ::= 9

id-OTDOA-Information-Type-Item												ProtocolIE-ID ::= 10

id-MeasurementQuantities-Item												ProtocolIE-ID ::= 11

id-RequestedSRSTransmissionCharacteristics									ProtocolIE-ID ::= 12

id-ULConfiguration															ProtocolIE-ID ::= 13

id-Cell-Portion-ID															ProtocolIE-ID ::= 14

id-InterRATMeasurementQuantities											ProtocolIE-ID ::= 15

id-InterRATMeasurementQuantities-Item										ProtocolIE-ID ::= 16

id-InterRATMeasurementResult												ProtocolIE-ID ::= 17

id-AddOTDOACells															ProtocolIE-ID ::= 18

id-WLANMeasurementQuantities												ProtocolIE-ID ::= 19

id-WLANMeasurementQuantities-Item											ProtocolIE-ID ::= 20

id-WLANMeasurementResult													ProtocolIE-ID ::= 21

id-Assistance-Information													ProtocolIE-ID ::= 22

id-Broadcast																ProtocolIE-ID ::= 23

id-AssistanceInformationFailureList											ProtocolIE-ID ::= 24

id-ResultsPerSSB-Index-List													ProtocolIE-ID ::= 25

id-ResultsPerSSB-Index-Item													ProtocolIE-ID ::= 26

END

-- ASN1STOP

END OF CHANGE