3GPP TSG-RAN WG3 Meeting #109-e 	R3-205610

- 17-28 August 2020, Online

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.422 | CR | 0037 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SCTP association change when current SCTP association is failed (X2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, China Telcom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-07-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For multi-SCTP, it is unclear which node initiates the TNL binding update, e.g. when there is a SCTP failure. There is no description for how the TNL binding update is performed. In addition, the description for “one eNB and en-gNB pair” is already moved to the next paragraph, but forgot to delete the first “one eNB and en-gNB pair”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the support that the NG-RAN node can select a new SCTP association when current SCTP association is failed. Also delete the first “one eNB and en-gNB pair”. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has no impact with the previous version of the specification (same release) because it adds the missing behavior for TNL binding update. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear behaviour for how TNL binding update is performed, and wrong specification for the behaviour for one eNB and en-gNB pair. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 1: resubmitted in RAN3#109e Rev 2: updated WI code … |  |  |  |  |  |  |  |  |

Start of Change

# 7	Transport layer

SCTP (IETF RFC 4960 [5]) shall be supported as the transport layer of X2 signalling bearer. The Payload Protocol Identifier (ppid) assigned by IANA to be used by SCTP for the application layer protocol X2AP is 27. The byte order of the ppid shall be big-endian.

SCTP refers to the Stream Control Transmission Protocol developed by the Sigtran working group of the IETF for the purpose of transporting various signalling protocols over IP network.

There shall be only one SCTP association established between one eNB pair, or one eNB and en-gNB pair. An eNB and an en-gNB shall support a configuration with a single SCTP association per one eNB and en-gNB pair. Configurations with multiple SCTP endpoints per one eNB and en-gNB pair should be supported. When configurations with multiple SCTP associations are supported, en-gNB may request to dynamically add/remove SCTP associations between one eNB and en-gNB pair. Within the set of SCTP associations established between one eNB and en-gNB pair, a single SCTP association shall be employed for X2AP elementary procedures that utilize non-UE-associated signalling with the possibility of fail-over to a new association to enable robustness. An eNB and an en-gNB shall use the Destination Port Number value 36422 assigned by IANA to be used for X2AP and this value shall also be used in Source Port Number by all eNBs and en-gNBs within a network. When the en-gNB request to dynamically add additional SCTP association between one eNB and en-gNB pair, the SCTP Destination Port number value may be 36422, or any dynamic port value (IETF RFC 6335 [7]).

NOTE:	A multi-homed eNB implementation should provide the correspondent eNB with the set of IP addresses supported during SCTP association establishment unless the correspondent eNB already has this information e.g. through IP address management.

An arbitrary eNB or en-gNB shall be able to initiate the INIT procedure towards another en-gNB or eNB for establishing the SCTP association.

Within the SCTP association established between one eNB pair;

-	A single pair of stream identifiers shall be reserved for the sole use of X2AP elementary procedures that utilize non UE-associated signalling.

-	At least one pair of stream identifiers shall be reserved for the sole use of X2AP elementary procedures that utilize UE-associated signalling. However a few pairs (i.e. more than one) should be reserved.

-	A single UE-associated signalling shall use one SCTP stream and the stream should not be changed during the communication of the UE-associated signalling.

Between one eNB and en-gNB pair;

-	A single pair of stream identifiers shall be reserved for the sole use of X2AP elementary procedures that utilize non UE-associated signalling.

-	At least one pair of stream identifiers shall be reserved for the sole use of X2AP elementary procedures that utilize UE-associated signalling. However a few pairs (i.e. more than one) should be reserved.

-	For a single UE-associated signalling, the eNB and the en-gNB shall use one SCTP association and one SCTP stream and the SCTP association/stream should not be changed during the communication of the UE-associated signalling unless TNL binding update is performed by the eNB or the en-gNB). The eNB (or en-gNB) can update the UE TNLA binding by sending the X2AP message for the UE to the peer en-gNB (or eNB) via a different TNLA, and the peer en-gNB (or eNB) shall update the X2AP UE TNLA binding with the new TNLA.

End of Change