3GPP TSG-RAN WG3 #108-e	R3-205712

Online, 17-28 August 2020

Agenda Item:	31.3.7.1

Source:	Huawei (Moderator)

Title:	Summary of Offline Discussion on Additional Duplication Activation over E1

Document for:	Approval

# Introduction

This is the kick off of the following Come Back:

CB: # 79bis_AddDuplActE1

-  check usage

- DL or UL? W.r.t. Rel-15

(HW - moderator)

Summary of offline disc R3-205712

# For the Chairman’s Notes

Propose the following:

Proposal 1:

# Discussion

The section 9.3.1.38 PDCP Configuration in TS 38.473 is copied as follows, where it is clearly specified that the Duplication Activation IE is used for DL PDCP duplication. The update was agreed in [R3-186207] in RAN3-101bis meeting, in which it states that:

- Proposal 1: Introduce a new IE in the BEARER CONTEXT MODIFICATION REQUEST message and BEARER CONTEXT SETUP REQUEST message to indicate the initial state of the DL PDCP duplication for a given bearer.

#### 9.3.1.38 PDCP Configuration This IE carries the PDCP configuration. IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. <Skipped>. PDCP Duplication; O; ENUMERATED (True, …); Indicates whether PDCP duplication is to be configured for the DRB. This IE is ignored when the “Additional PDCP duplication Information” IE is present.; -; -. PDCP Re-establishment; O; ENUMERATED (true,…); Indicates PDCP entity re-establishment to be triggered as defined in TS 38.323 [17]; -; -. PDCP Data Recovery; O; ENUMERATED (true,…); Indicates PDCP data recovery to be triggered as defined in TS 38.323 [17]; -; -. Duplication Activation; O; ENUMERATED ( Active, Inactive, …); Information on the initial state of DL PDCP duplication; -; -. Out Of Order Delivery; O; ENUMERATED (true,…); Indicates whether or not outOfOrderDelivery specified in TS 38.323 [17] is configured. Out of order delivery is configured only when the radio bearer is established.; -; -. PDCP Status Report Indication; O; ENUMERATED (downlink, uplink, both, …); For AM DRB, “downlink” indicates that the PDCP entity is configured to send PDCP status report(s) to the UE, and “uplink” indicates that the UE is configured to send PDCP status report(s), as specified in TS 38.323 [17]. “both” indicates that both “downlink” and “uplink” should be applied.; YES; ignore. Additional PDCP duplication Information; O; ENUMERATED (three, four, …); Indicates the number of PDCP duplication configured when it is more than 2 for the DRB; YES; ignore. EHC Parameters; O; 9.3.1.90; YES; ignore.

## Question description

In Rel-16, the PDCP duplication up to four RLC entities is supported. Follow the Rel-15 principle, the CU-UP needs to obtain additional DL Activation State for DRBs configured with up to four RLC entities to indicate the initial state. Otherwise, the CU-UP will not know how many and which RLC entities should activated as initial state.

Question 1: Do company agree that additional information is needed over E1?

Companies’ views can be provided in the following Table.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes | - Follow the logic of R15, this additional information is needed |

If the answer to the question above is YES, R3-205094 proposes the following IE format:

#### 9.3.1.xx	Additional Duplication Activation

This IE indicates additional duplication activation in case that the indicated DRB is configured with more than two RLC entities as specified in TS 38.331 [8].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Duplication Activation List |  | 1 |  |  |
| >Duplication Activation Item |  | 1 .. < maxnoofDuplicationactivation > |  | This IE indicates information on the initial activation state of DL PDCP duplication. Each position in the list represents a state in the order of UP Transport Layer Information in UP Parameters IE. |
| >>Duplication State | M |  | ENUMERATED (Active, Inactive, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofDuplicationactivation | Maximum no of duplication state. Value is 4. |

Question 2: Do company agree to add the above IE format over E1?

Companies’ views can be provided in the following Table.

| Company | Yes/No | Comment e.g. the exact IE format, and number of maximum value. |
| --- | --- | --- |
| Huawei | Yes | - The above IE not only indicates how many RLC entities can be activated, but also which RLC entities (via the corresponding TNL information) should be activated. - Currently up to four RLC entities are supported for R16. So the maximum 4 is enough, for which the same principle applies when determining the number of additional Tunnels. - But we are open to other alternative ways. |

## Other issues

Companies can provide further issues not covered by above.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-205094 Introducing Additional Duplication Activation over E1 (Huawei, CATT)