3GPP TSG-RAN WG3 #109-e	R3-20xxxx

Online, 17-27 August 2020

Agenda Item:	13.2.2

Source:	Qualcomm (moderator)

Title:	Summary of Offline Discussion on IAB service interruption reduction

Document for:	Approval

# Introduction

CB: # 7_IAB_ServiceInterruptionReduction - CATT MT dual connection or UE dual connection could reduce the service interruption. Whether the dual connection mechanisms can be implemented in IAB node migration should be discussed in RAN 2. F1 setup procedure of migrating IAB node can connect to target CU directly or via source CU. The mechanism of IAB node DU connection with two IAB donor CUs to reduce service interruption should be further studied. - QC Discuss a procedure for parallel F1-C migration of all descendant nodes during intra-donor topology adaptation. Consider means to reduce the number of signaling handshakes for F1 migration. Consider use of MobIKE for the migration of IPsec to a new IP address. Consider mechanisms for packet loss recovery at descendant nodes in intra-donor topology adaptation. Extend the NR-UP protocol to support uplink data delivery status reports to enable recovery of packet loss during intra-donor migration. - LG (5063) Take trains, buses and other vehicles into account as the mobile use cases. Consider the following issues for mobile IAB: - Mobile IAB’s cell specific configuration, i.e., PCI, cell ID and TAI allocation - Group mobility for UEs in the vehicles - DAPS/DC based inter Donor CU’s change - LG (5249) Investigate the impacts on RRC and X2/Xn interface for supporting the MCG/SCG recovery of IAB node in EN-DC and NGEN-DC cases. Enhance the CHO procedure for intra/inter-CU Backhaul RLF recovery for IAB-nodes in SA mode. - ZTE (5163) Rel-16 re-routing mechanism is reused in intra-donor DU migration scenario. Considering backward compatibility and architecture complexity, identical architecture should be used in Rel-16 and Rel-17 IAB, i.e. hop-by-hop RLC ARQ is adopt in R17 IAB. Rel-17 UE could be enhanced to perform re-transmission based on PDCP status report. However, this method is not applicable to legacy UEs. Access IAB node performs the re-transmission using the updated IP addresses and BAP address on the target path after migration. Further study how the access IAB node could determine which packets need to be re-transmitted on the target path after migration. Enhancements should be investigated considering that re-routed F1-U packets ciphered with old key couldn’t be deciphered by the target donor CU. Enhancements should be investigated to prevent that UE packets ciphered with new key be transmitted by the migrating IAB node on the source path and delivered to the source donor CU. - ZTE (5164) Rel-16 mobility enhancements (i.e. CHO and DAPS) be applied for IAB-node migration in R17. Migration-IAB should also send the DAPS-HO required BH RLC channels of the migration-IAB to the target gNB during DAPS-HO in IAB. Migration-IAB-MT create a MAC entity for target, establish an RLC entity and an associated DTCH logical channel for the target for each BH RLC channel configured with DAPS. CU indicates the source parent-IAB to continue the downlink user data transmission to the migration-IAB by F1AP message during DAPS-HO. Further discuss how to inform the access UE of migration IAB node to perform uplink data switching in IAB during DAPS-HO. - E/// discuss how to limit packet losses and unnecessary transmissions at inter-CU migration - HW Take the procedure shown as BL for the inter-CU BH RLF recovery. Study the mechanism for IAB-DU recovery (e.g. F1 connection re-establishment, rather than setup) in inter-donor-CU RLF recovery case, to achieve: - Signaling storm avoidance in F1 interface between IAB-DU and new IAB-donor-CU. - Long-term service interruption avoidance for connected UEs. Discuss with RAN2 how to handle the descendent IAB-nodes and UEs in the scenario that the IAB-node suffers BH RLF recovery to a new IAB-donor-CU via new path, from the following two aspects: - Whether the descendent IAB-nodes and UEs should be aware that the connected IAB-donor-CU changes. - The consequent behavior of the descendent IAB-nodes and UEs. Will they change connection to new IAB-donor-CU with the recovery IAB-node? - SS During the IAB node migration procedure, the default BAP routing ID and default BH RLC CH for F1-U traffic can be configured to the IAB-MT part for F1-U traffic interruption at the target path. During inter-CU migration procedure, the F1-C traffic can be switched to the target IAB donor CU as long as the IAB-MT part accesses to the target IAB donor CU. During inter-CU migration procedure, the F1-U traffic can be anchored to the source IAB donor CU before the transmission path is configured at the target path. If IAB-MT part supports DAPS handover, the configurations for serving the collocated IAB-DU part at source, e.g., BAP configuration, BH RLC CHs, IP address, F1-U tunnels at source, etc., can be kept for a while so that the DL F1-U traffic can be transmitted via source, and the UL F1-U traffic can be transmitted via target. ++ Suggested guidelines/topics for discussion from Chair (looking at possible consensus): + Coordinate with CB 6 (rationale: if a solution for migration also ensures service interruption reduction, it should be prioritized) + Tools to achieve reduction of service interruption: limit packet loss/unnecessary transmission? Dual MT/UE connection (seems out of RAN3 scope?)? Parallel migration of descendants? Reduction of handshakes/signaling? Enhancements of data switching? MobIKE? Connection re-establishment? Others? Down-selection strongly recommended + Start st2 TPs for BL CRs (lots of FFSs) + Attempt st3 TPs for BL CRs (lots of FFSs) + Discussion on mobility, including enhancements, etc., seems out of WI scope (QC - moderator) Summary of offline disc R3-205467

(Qualcomm - moderator)

Summary of offline disc R3-20xxxx

The discussion has two phases:

Phase 1: Enhancements to IAB service interruption (and others) to be discussed in Rel-17

Phase 2: TBD

The deadline for Phase 1 is Wednesday, August 20, 24:00 UTC. This allows us to discuss intermediate stage in Thu online session. We might be able to already achieve some agreements at this stage.

The deadline for Phase 2 is the same as for all email discussions, i.e., Tuesday, August 25, 1300 UTC.

# For the Chairman’s Notes

Propose the following:…

# Discussion

## Enhancements to redundant connectivity

Multiple contributions consider enhancements to redundancy as a mean to avoid service interruption due to parent-node change and/or RLF recovery. The following options have been considered:

- NR DC: [4734][5063]

- DAPS: [5063][5164][5412]

- Multi-MT: [5224]

- CP/UP split using FR1/FR2: [4797][4867][5064][5414]

- Backup routes for control plane via descendant nodes: [4797]

Most of these options can be applied to intra-donor and inter-donor redundancy.

Detailed discussion of these options should occur in CB8.

In this CB, companies should provide their view on which of options (1) - (5) should be considered for the reduction of service reduction.

Q1: Which of redundancy options (1) to (5) should be considered for the mitigation of service interruption? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | Reduction of service interruption should be included as a use case in the discussion on redundancy enhancements in CB8. In CB7 (i.e. this CB), we should discuss other means than redundancy for the reduction of service interruption since it cannot be assumed that the above redundancy features (1)-(5) are available. |
| Samsung | Agree with QC, this CB should focus on the schemes of service interruption mitigation except the redundancy. In this sense, (2) & (5) seems to be in the scope. Between them, we think (2) can be prioritized. In addition, we need figure the targeting scenarios for the service interruption mitigation: - Scenario 1: Intra-CU/intra-donor-DU - Scenario 2: Intra-CU/inter-donor-DU - Scenario 3: Inter-CU The first two scenarios can be the start point since it is based on Rel-16 IAB. For scenario 3, we may need wait for a moment since we should first determine the inter-CU migration procedure. |
| Nokia | Option 1, 2 and 4 should be considered. Option 3 may need to wait for RAN2 (and maybe also RAN1/4). Option 5: unclear about the difference to Rel-16 way, e.g. after the IAB declares RLF, it initiates RRC reestablishment to another IAB supporting cell. |
| Huawei | For option 1, please clarify what is NR-DC, compared to legacy MT’s DC in R16. No need to touch option 3, which was agreed in R16 already as supported by implementation. Option 5 should be general, rather than only “for control plane”. Share same view as Samsung, option 2 and option 5 will be beneficial for the service interruption reduction, can be discussed in R17. |
| CATT | Option 1 and option 2 We consider the F1 dual connection which means that IAB node DU connect with two IAB donor CUs to reduce service interruption. However, the scenario of one DU managed by two CUs is not supported in Rel-15. In our view, the mechanism of migrating IAB node DU managed by two CUs is feasible, since the signallings or data packets have target address and/or source address. Thus, the DU can know the signallings or data packets are from which CU. Furthermore, we also consider the DAPS mechanism. Specifically, it needs the MT/UE to keep dual connection or UE dual connection during migration. |
| ZTE | Option 1, 2, 4 could be considered for service interruption reduction. And the above redundancy options could be discussed in CB8. |
| KDDI | Option 1 ,4. It might be better to check if we could reuse the current mechanism. |

## Use of CHO

CHO was proposed by [5164][5247] as a mean to avoid RLF recovery, which would significantly reduce service interruption. CHO can be applied to intra-donor and inter-donor migration.

Q2: Should CHO be considered for mitigation of service interruption? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | CHO should be supported. It promises significant benefit since it can avoid RLF recovery. The specification/implementation overhead is expected to be small since this has already been done for UEs. CHO is mainly in RAN2 territory. |
| Samsung | To reduce the service interruption after BH RLF, CHO is an effective scheme. We can start the discussion from the intra-CU scenario. RAN2 may be a suitable place to start this discussion. |
| Nokia | Agree with QC. |
| Huawei | Agree with QC and Samsung.R3 can confirm the support of CHO and left R2 to do the work. |
| CATT | CHO should be supported. Whether it’s necessary to specify some more configuration information and trigger conditions specifically for IAB node can be further discussed in RAN 2. |
| ZTE | CHO could be considered in IAB node migration to reduce handover failure and improve handover robustness. |

## Parallelization of signaling procedures

According to [4795] and [4796], the Rel-16 intra-donor parent-node change procedure only works if descendant nodes and UEs are migrated sequentially across tiers. Parallelization of these procedures across tiers was proposed by [4745][4795][4796]. Such parallelization can be applied to intra- and inter-donor parent-node change. The figure below shows an example from [4795].

For inter-donor IAB-node migration, this enhancement should also be discussed in CB6.

Figure: Example of sequential vs. parallel intra-donor parent-node migration (R3-204796)

Q3: Should parallelization of migration procedures across tiers be considered for mitigation of service interruption? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | It is absolutely necessary to establish a proper migration procedure for descendant nodes/UEs for both, intra- and inter-donor migration. Parallelization of signaling should be part of this discussion. The benefits to service interruption may be significant in presence of multiple tiers. Also, specification effort is expected to be small since this is mainly st2. |
| Samsung | Parallelization is an effective method to speed up the migration procedure in case of several descendant nodes. Stage 2 impact is needed. The stage 3 impact may need further discussion. |
| Nokia | The main point may be not about the parallelization, but the reconfiguration to the descendant IABs should be performed over the source path and before the migrating IAB connect to target Parent. |
| Huawei | About the term “parallelization”, which seems indicate 100% overlap for the procedure between parent and descendent nodes, there would be some issues to be solved, e.g. the time alignment of TNL migration for multiple UEs. So, we’d like to clarify it should be “some overlap in time domain”, no exactly parallel. Moreover, as stated in the intra-CU topology update procedure in TS38.401, clause 8.2.3.1: “Based on implementation, these steps can be performed after or in parallel with the handover of the migrating IAB-node.” Sending RRC reconfiguration via source path may be beneficial, so the current specification allow the IAB nodes perform RRC reconfiguration and TNL migration in “parallel”, but should up to implementation. Therefore, from our view, this is some kind of clarification for CU implementation on when trigger the each tier’s migration procedure, no additional specification works are needed. We’d like to ask some clarification about stage 3 impact before we agree something. For the inter-donor IAB-node migration, suggest to discuss the sequential way as the baseline at first. |
| CATT | We think this parallelization‎ mechanism has some benefits to reduce the service interruption. The mechanism can be feasible for intra-CU migration scenario. The gap between current implementation method in R16 specification need to be clarified. However, whether it is feasible for inter-CU migration needs to be further studied after the discussion on baseline inter-CU migration procedure. |
| ZTE | In current TS 38.401, it is stated that “Steps 11, 12 and 15 should also be performed for the migrating IAB-node’s descendant nodes”. however, it is not clear whether the RRC reconfiguration to descendant IAB nodes should be performed and when to perform the RRC reconfiguration. So stage 2 specification in TS 38.401 is needed for intra-donor migration scenario. For inter-donor migration scenario, similar issue exists and should be further studied. |
| KDDI | Our understanding is parallel procedures is not restricted in the current specification. |

## Group mobility

Use of group mobility signaling was proposed by [4460] [4732] [4733] [4796] [4917] [5063]. This refers to the bundling of messages associated with multiple UEs and/or IAB-MTs. Message bundling was already applied in Rel-16 for UL mapping update of F1-U tunnels on E1 and F1 interfaces. For UEs, IAB-MTs, it can be applied to, e.g., handover request/request Ack, SN status transfer and path switch to core network. Group mobility primarily reduces signaling overhead.

Group mobility signaling was also prosed as a facilitator for inter-donor IAB-node migration. This aspect should be included in CB6.

Q4: Should group mobility signaling be considered for reduction of signaling overhead and/or service interruption? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | The main benefit for group mobility is reduction of signaling rather than reduction of service interruption. We should first focus on enhancements that can reduce service interruption. At a later stage of Rel-17, i.e., when we have converged on these enhancements and have the baseline procedures in place, we can discuss opportunities to improve signaling via message bundling. |
| Samsung | We we understand the intention of group mobility is to reduce the signaling. However, this should be considered after the baseline procedure is determined. On the other hand, during the inter-CU migration procedure, all the UE contexts need migration anyway. Thus, we are wondering if the group mobility can really reduce the signaling load. |
| Nokia | Group mobility was also discussed in LTE mobile Relay. The main question is what is saved by the group mobility vs. current per-UE handover procedure. The group HO seems to build a super large message to include the context for all UEs. Please clarify what can be saved. |
| Huawei | Based on our understanding, the “group mobility” anyway need to be supported, since some descendent nodes and UEs will perform migration with the migrating IAB node. While for the “group mobility signaling”, such optimization of signaling design may be beneficial for reduce the number of signaling, but we are fine to start with the baseline procedure using separate signaling. |
| CATT | Since each descendant node and UE should interact with IAB donor CU2 about context, we propose send the UE context list in handover request message to reduce signaling overhead. While the discussion of group mobility can be discussed after the baseline procedure is determined. |
| ZTE | A large number of concurrent mobility signaling could be avoided using group mobility. So group mobility could be considered for the purpose of reduction of signaling overhead. |
| KDDI | Should be discussed within CB#6 |

## Use of MobIKE instead of IKE

Use of MobIKE instead of IKE was proposed by [4795][4796]. This applies to intra-donor parent-node change, and potentially also to inter-donor IAB-node parent-node change in case F1-C is migrated from source to target path. Since MobIKE signaling can be conducted in parallel with other signaling or traffic, it does not consume any additional time.

Use of MobIKE in the context of inter-donor IAB-node migration should also be included in CB6.

Q5: Should MobIKE be considered for mitigation of service interruption? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | MobIKE should be supported. The improvement in service interruption is obvious. The specification effort is minor, i.e., mostly st2. |
| Samsung | MobIKE seems to be in SA3 scope. Would it be better to ask SA3’s advice after we determines the baseline procedure for inter-CU migration? |
| Nokia | MobIKE can be considered as an option. |
| Huawei | Agree with Samsung, this is not purely RAN3’s scope, should be discussed by SA3. |
| CATT | Agree with Samsung and Huawei |
| ZTE | The use of MobIKE should be first discussed in SA3. |
| KDDI | Agree with ZTE. |

## Configuration of F1-U mapping via RRC

F1-U UL mapping can be configured via RRC (like the default mapping) rather than F1AP as proposed by [5412]. This saves a signaling handshake on the target path for UP traffic.

Q6: Should RRC-based configuration of F1-U UL mapping be considered for mitigation of service interruption? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | The benefit of this enhancement is obvious, and the specification effort is minor. We should certainly discuss this further. We may want to include other opportunities, where more flexible configuration using either RRC or F1AP could provide improvements. |
| Samsung | Yes. This enhancement establishes a temporary path for the F1-U traffic during the migration before the F1-U tunnels are redirected. Thus, the F1-U traffic can be continuously transmitted as long as the IAB-MT part accesses to the target parent node. |
| Nokia | Ok. The main point is to configure the IAB the BH information earlier. Using RRC may be one option |
| Huawei | Yes, using RRC based UL mapping configuration may be beneficial to enable the IAB-node transmit UP packets as earlier as possible after migration. We can start with some default configure for F1-U via RRC. |
| CATT | Yes. Default configure for F1-U UL could improve service interruption during the F1 setup procedure and RRC reconfiguration procedure. |
| ZTE | Yes, if F1-U UL mapping is configured via RRC, F1-U data forwarding could start on the target path upon F1 connection is setup with target donor CU, i.e. no need to wait for the F1-U UL mapping configuration after F1 setup as in normal procedure. |
| KDDI | OK. Would like to see the benefit via RRC. |

## Improvement of packet-loss

For parent-node change, in-flight packets sent on the source path to or from descendant nodes are dropped if the switch occurs before these packets have passed the link with the source parent. The recovery of these packets increases service interruption. The figure below shows an example.

The following approaches to packet-loss recovery were proposed [4796][5163][5223]:

- Enhancements to PDCP status report (has impact on UE)

- Rerouting of in-flight packets

- Introduction of UL Data Delivery Status to NRUPP analogue to DDDS.

Figure: Example of packet loss to/from descendent nodes during parent-node migration [4796]

Q7: Should packet-loss recovery be considered for mitigation of service interruption. Please also include views on the proposals (1)-(3) to reduce packet loss. Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | Packet loss recovery is only useful for parent-node migration, not for RLF recovery. We should therefore not spend any time on fixing this problem for RLF recovery. For parent-node migration, the above proposals can be assessed as follows: - Change to PDCP: No way since this affects the UE. - Rerouting: Only helps in limited scenarios. Also, rerouting across different donor-DUs is problematic as discussed in Rel-16. Therefore, there is little value in packet recovery via rerouting. - DDDS is sufficient to recover lost packets on the DL, so we don’t need anything else. We should discuss extension of NRUPP to support UL equivalent of DDDS for the recovery of lost packets on the UL. This should be straightforward from specification perspective. |
| Samsung | The enhancement should avoid the impact to the UE since IAB network is transparent to the UEs. - PDCP enhancement: this is in RAN2 scope, which may have UE impact. So, we need avoid. - Re-routing: For the uplink, the in-flight packets at the source path can be continuously transmitted to the source CU. So, the packet loss at the UL link may not be a big problem. The only possible enhancement is that the source CU should be aware of when all in-flight packets are received. For in-flight packets at the descendant nodes of migrated node, there is no problem for intra-CU/intra-donor-DU migration; while other scenarios may cause some problem (e.g., the source routing at the donor DU may discard the in-flight packets, the inter-CU migration results in that the target CU cannot decipher the in-flight packets). It seems that re-routing cannot solve the problem. Thus, at this moment, we are concerning the benefit of re-routing. - UL DDS This is an effective scheme to avoid the packet loss. We can discuss it further. |
| Nokia | Packet-loss prevention should be studied. - has the drawback of UE impact; - currently only works in intra-donor-DU cases; - DDDS or Uplink-data reception status would only help in intra-donor cases due to the PDCP-key change in the inter-donor case. |
| Huawei | The Packet loss recovery is worth to be studied, for both the IAB migration scenario, and the BH RLF recovery case. And the solution should be backward compatible for legacy UEs. - Will impact UE, should be excluded. - Re-routing works for the intra-DU case so far since the BAP address cannot changed, for inter-donor DU and inter-CU case, the re-routing mechanism should be enhanced to support inter-donor-DU re-routing for packet lossless. This option should be at least supported or solved. - Agree with Nokia, such way does not work for the inter-CU case. We can first agree the scope and issue in this meeting, rather than rush into solution. |
| CATT | - PDCP status report: this is in RAN2 scope and may have UE impact. We may further discuss whether we can support status report in BAP layer and retransmit the BAP packet to perform packet-loss recovery‎. - For inter-CU migration, the target CU cannot decipher the in-flight packets - The DDDS mechanism may not need for UL. For DL, the CU sends data packets to DU and forwarding it to UE, UE has ARQ mechanism to DU, which means that the DU knows the transfer status, however, the CU does not know. Therefore, the DU should send DDDS to CU to indicate the transfer status. For UL, the source CU would know what packet received, so it no needs to introduce a UL equivalent of DDDS. |
| ZTE | - Packet-loss recovery needs to be investigated in IAB node migration scenario. - Enhancements to PDCP status report: this option should be excluded since it is not applicable for legacy UE. - Rerouting of in-flight packets: re-routing is only supported in RLF scenario in Rel-16. It could be reused in intra-donor DU migration scenario. For inter-donor DU and inter-donor CU scenario, similar re-routing mechanism could be investigated considering that the BAP address and IP addresses are changed after migration. - Introduction of UL Data Delivery Status to NRUPP analogue to DDDS. This option could be studied and utilized together with approach 2(i.e. rerouting of in-flight packets) to solve the packet loss issue. |
| KDDI | - HARQ, RLC already have the retransmission mechanism for packet loss, it is not very important to spend time on this topic. |

## Indication of donor-association

According to [4698], if the parent node indicates its donor association, cell-selection by an RLF-recovering IAB-node may give preference to parent node candidates that belong to the same donor over those that belong to a different donor. RLF recovery will therefore be faster if a node is selected that belongs to the same donor.

This, actually, was not proposed by [4698]. However, the idea is neat, so we keep it here. [4698] will be discussed further below.

Q8: Should indication of donor association by parent node be considered for mitigation of service interruption. Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | This is worth considering since it is a minor specification effort. It resides in RAN2 territory. |
| Samsung | This can be discussed in RAN2. |
| Nokia | What is “donor association by parent node”? Do you mean states that Donor information (e.g. gNB ID) is provided to the IAB, so the IAB may know whether it is intra-Donor, or inter-Donor? This may be needed. Also, the IAB node may have more than one neighboring Donors, so the IAB node need to know which Donor is the target, e.g. to use the related cell ID when initiate F1 Setup. |
| Huawei | IAB-node should be aware of whether the HO is intra-CU or inter-CU. R16 agree not to support this. But agree with QC and Samsung that should be discussed by RAN2. |
| CATT | Indication of donor-association relevant the broadcast, priority setting and etc. It is better discussed in RAN 2. |
| ZTE | Yes, it is beneficial if IAB node indicates the identity of the donor CU. And it should be discussed in RAN2. |
| KDDI | Agree with QC, but it might be discussed in CB#6 |

## Enhancements related to inter-donor migration

A few enhancements were proposed that specifically apply to inter-donor parent-node change and RLF recovery:

- Simultaneous connectivity of F1-C to source and target IAB-donor [4734]. This can be achieved by having independent F1-C connections by the IAB-DU to each IAB-donor, or by having one of the IAB-donor-CU’s become an F1-C proxy for the other. It allows signaling to occur preemptively and thereby reduces the overall interruption time.

- Descendant IAB-MTs and UEs retain the connection with the parent-node without performing a full handover procedure [4717]. The UEs and descendant nodes may not even be aware that the IAB-donor has changed [5293].

- Use IAB-DU performs recovery procedure at the new IAB-donor rather than setting up a new connection from scratch [5293].

- According to [4698], the migrating IAB-needs to be signaled if the migration is intra- or inter-donor so that it can establish a new F1-C connection in case it is an inter-donor migration.

Detailed discussion of these options should occur in CB6.

In this CB, companies should provide their views, which of these options (1) - (3) should be considered for the reduction of service reduction.

Q9: Which of inter-donor migration enhancements (1) to (4) should be considered for the mitigation of service interruption? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | Assessing the above enhancements: - Should be discussed as part of inter-donor migration in CB6. - Does not work! The UE and descendant nodes need to be connected to the same CU as the migrating IAB-node. - This procedure is not clear. More detailed discussion should occur in CB6. |
| Samsung | - to (3) should be discussed in CB6. |
| Nokia | 1-3 can be considered. |
| Huawei | - simultaneous F1-C connectivity to two different donor-CUs should be avoided, because the gNB-DU only connects to one gNB-CU since R15. IAB-DU should apply same principle. and (3) can be discussed. |
| CATT | (1) and (3) have some benefit to reduce service interruption, but better to discuss together with CB6. (2) more relevant load balance, It is better to discuss together with CB6. (4) We think the migrating IAB node could tell intra-CU migration or inter-CU migration by some information, e.g., CGI change. |
| ZTE | Enhancement 1: we have concern that one gNB-DU is allowed to connect to one gNB-CU except in RAN sharing scenario in NR. Enhancement 2: we agree that descendant IAB-MTs and UEs could retain the connection with the parent-node, however UEs/descendant nodes should be aware that the IAB-donor has changed since the security key changed after inter donor migration. Enhancement 3: we think F1 setup procedure is still needed. The difference with normal F1 setup is that target donor CU already obtains DU context from source donor CU before the F1 setup procedure. Enhancement 4: it is not necessary since migrating IAB node could implicitly deduce whether it is intra or inter donor migration according to the RRC reconfiguration message, e.g. whether new security key is included. |
| KDDI | (1)-(4)should be considered in CB#6 |

## Change of PCI

For mobile IAB-nodes, change of PCI should to be considered according to [5063]. This would apply to intra-donor and inter-donor migration. PCI change has the goal to reduce PCI collision.

Q10: Should PCI-change be considered for intra-donor and/or inter-donor IAB-node migration to reduce PCI collision? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| QC | PCI reconfiguration should be considered. Mobile IAB is one obvious use case, but there may be others. The specification effort should be minor. In Rel-17, the discussion should not extend to the discovery of PCI conflicts. |
| Samsung | In Rel-17, we can assume PCI is not changed since the PCI conflicts can be avoided by OAM configuration when IAB node is fixed. For mobile IAB, the PCI conflict may be a problem for unchanged PCI. However, it is out of scope of Rel-17 |
| Nokia | PCI-change should be considered. But the goal may be not just to reduce PCI collision, but also allow to perform the HO for the UE, e.g. in case of inter-Donor migration. Also, [5063] also propose to consider the cell ID change. This is needed for inter-Donor migration. I will add it in 3.11. |
| Huawei | Agree with Samsung, PCI change is not necessary for R17 IAB since only fixed IAB will be considered. |
| CATT | PCI collision should be considered. It could be addressed through the re-allocation of PCI by target CU, e.g., the new PCI convey to source CU in handover request ack message and further send to migrating IAB node. |
| ZTE | If mobile IAB is considered in R17 IAB, PCI collision issue should be considered and resolved. |
| KDDI | PCI should be considered. |

## Other enhancements

Q11: Please list other enhancements that should be considered for Rel-17 IAB? Please include assessment of expected benefit, impact on specification, implementation, other WGs.

| Company | Comment |
| --- | --- |
| Nokia | Cell ID change also need to be considered, since cell ID is related to the Donor-CU’s gNB ID. If use OAM to consider the cell ID, then need to discuss when to perform the OAM configuration. The migrating IAB may also need to report target donor gNB ID or cell ID to OAM, so OAM can know the target Donor and assign the related cell ID. |
| ZTE | With regard to the cell ID change issue, we think it is more simple and straight forward that the target donor CU configures new cell ID for migrating/descendant IAB nodes. Reconfigaration of cell ID via OAM may consume a long time which may introduce service interruption during migration. |

# Part II…[if needed]

If needed

# References