3GPP TSG-RAN WG3 #109-e							R3-205479

17-28 August 2020

Online

Agenda Item:	15.2

Source:	ZTE (moderator)

Title:	Summary of NR QoE Services

Document for:	Approval

# Introduction

CB: # NRQoE2-Services

- What kind of services should be covered? Besides Streaming services and MTSI services (already supported in LTE), any new services, e.g., AR/VR, MBMS, URLLC...?

- For Streaming services and MTSI, VR services QoE metric, has already been defined in SA4. How to define the QoE metric for new services, if identified?

- Any additional RAN measurements need to be defined for new services, or associate RAN-related measurements with application layer QoE report for NR QoE management? Associate MDT collected data with QoE collected data?

- TP for TR, if agreeable

(ZTE - moderator)

Summary of offline disc R3-205479

The email discussion would split into two phases:

Phase 1:  Please provides your inputs before end of the first week (Friday(2020/8/21),17:00 UTC).

Phase 2:  Moderator would draft TP content based on input and collects further input before the ON-line session start. ( Before Tuesday (2020/8/25) 12:00 UTC).

# For the Chairman’s Notes

Propose the following:

# Discussion

## : Issue 1:  For UE application layer measurements [1][2][7]

- What kind of services should be covered? Besides Streaming services and MTSI services (already supported in LTE), any new services, e.g., AR/VR, MBMS, URLLC...?

- For Streaming services and MTSI, VR services QoE metric, has already been defined in SA4. How to define the QoE metric for new services, if identified?

Provide your view:

Proposal [1]: NR QoE measurement supports following SA4 defined service types:

- Streaming video: TS 26.247

- VR: TS 26.118

- MTSI: TS 26.114

- MBMS: TS 26.347.

Other service type ?

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm | Yes | These four services should be supported as baseline. In addition, we can study the need of supporting: - general OTT service, e.g. HTTP/TCP - gNB understandable QoE. |
| China Telecom | Yes | The above SA4 defined services should be supported. In our understanding, this SID should provide a generic framework to collect the QOE measurement/performance for all types of services. |
| China Unicom | Yes | The services already defined by SA4 should be supported and we prefer to initial some discussion on new metrics for 5G service (e.g. URLLC service) in RAN side. |
| Huawei | Yes | Similar view as China Telecom. We think that SA4 defined services should be supported as baseline. |
| CATT | Yes | These services shoule be supported. Agree with China Unicom, more services supported need to be considered. |
| CMCC | Yes | Share similar view with CU. |
| Samsung | Yes | The above SA4 defined services should be supported as baseline. The other service types which have not been defined in SA4 yet should also be taken into account. |

## Issue 2: RAN measurements related to NR QoE

[1]:LTE QoE measurement configuration and report are encapsulated in a transparent container and transferred to server, and the information is not accessible by eNB. To 5G new service such as URLLC service, some new QoE metrics related with air interface should be studied, which can be beneficial for optimization for lower layer resource allocation. It is propose some new metrics related with new 5G service for air-interface could be studied for better supporting network optimization for resource allocation.

[4] Proposal 2: Instant adaptive QoE mechanism and post-process adaptive QoE mechanism can be considered for NR QoE.

[10]Proposal 2: New solution should be introduced for latency sensitive service.

[15] What RAN-related measurements are required to be associated with application layer QoE report? A straight-forward way is to reuse MDT measurements, but how to associate MDT collected data with QoE collected data ?

Provide your view:

Proposal : RAN measurements (includes RAN node side and UE side) need to be taken into account to support NR QoE.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm | Yes | Correlation with RAN measurement is the benefit of RAN based QoE measurement. But, the actual standard impact may be little. MDT and QoE can be configured together to a UE. Then, the TCE/MCE can correlate the QoE and MDT together based on: timestamp, cell ID, location etc. |
| China Telecom | Yes | Agree with Qualcomm. How to correlate the measurements results from RAN side and UE side is an implementation issue. |
| China Unicom | Yes | Yes, RAN measurement for both RAN node and UE are needed and the standard impact could be further disucssed. |
| Huawei | Pending | Firstly, we need to understand what kind of RAN measurements or UE measurements are considered here. In this study item, our understanding, some KPI info will be considered, are they part of such measurements? We assume yes, then we could start from here. Secondly, what correlation means here? Seems to us, it is assumed that gNB would understand the content of QoE measurement report, but this assumption is still FFS. In addition, if we are talking about MDT for example, our understanding, both MDT and QoE are triggered from up layer, and should be reported to up layer, i.e. not used in RAN, then seems there is no correlation needed in RAN. |
| CATT | Yes | We should consider the global system level assessment and optimization, include NR QoE and RAN other measurements function. The correlation may be happened in RAN node or server. It is better in RAN node for URLLC like services |
| CMCC | Yes | In principle, RAN measurement needs to be studied to support NR QoE. In our opinion, potential RAN measurement contains: -Existing MDT measurement performed at UE -Existing performance measurement as well as L2 measurement performed at both RAN and UE -Newly defined measurement which can be collected and used by RAN. For those QoE-related services defined by SA4, RAN measurement can help RAN identify whether the performance degradation is caused by RAN. For new 5G services such as URLLC, RAN measurement (including newly defined ones such as delay measurement) may be vital to support QoE for URLLC services. |
| Samsung | Yes | We agree RAN measurements in UE side should be taken into account for quickly identifying network issues. RAN measurements in RAN side is FFS. |

## Issue 3: QoE related RAN measurements:

Is the answer of Issue 2 is yes. What RAN measurements related to QoE need to be take into account in NR QoE?

[3] The question for study is then whether also QMC for new NR services can use the LTE mechanism. E.g. new services based on URLLC may need to measure round-trip time on application level. This comes on top of QoS monitoring (latency measurement).

[4] Observations 3: QoE related RAN performance measurements requirements from various vertical traffic can be categorize into predefined metric.

The metrics includes at least: stringent requirements on (end to end ) latency, communication service availability, determinism of small jitter, very high data rate requirements along with low latency requirements, high density of UEs, seamless mobility support, clock synchronicity between different nodes, Network slicing.

[10] Proposal 2: New solution should be introduced for latency sensitive service.

[15] What RAN-related measurements are required to be associated with application layer QoE report? A straight-forward way is to reuse MDT measurements, but how to associate MDT collected data with QoE collected data ?

Provide your view on the proposal:

Proposal : The RAN measurements may include:

MDT measurements,

Latency measurements for latency sensitive service,

Network slicing related measurements,

...

| Company | Yes/No | Comment |
| --- | --- | --- |
| Qualcomm |  | See answer to last question. We need not to include RAN measurement inside QoE. Postprocessing can correlate the RAN measurement (MDT) and QoE. |
| China Telecom | Yes | We need to study whether to extend the existing RAN measurements metrics. |
| China Unicom | Yes | Latency , network slicing and potential new service measurement metrics could be studied. |
| Huawei |  | Not sure. QoE measurements and RAN measurements should be independent, of course we could study whether RAN measurements could be supplementary to QoE measurement, but as QC commented, no need to include RAN measurement inside QoE report. We suggest to start from investigating potential RAN measurements, e.g. latency… |
| CATT | Yes | We may consider RAN visible QoE report and RAN measurement together. For some service, the RAN node may use it directly. |
| CMCC | Yes | Please find answer to last question. |
| Samsung | Yes | Radio layer configuration information should be considered to identify the causes of QoE degradation. Slice information should be considered to monitor QoE on a specific slice. |

# Conclusion, Recommendations [if needed]

If needed

# References

| 1 | R3-204643 | Time plan for NR QoE Management and Optimizations for Diverse Services (China Unicom) | Work Plan |
| --- | --- | --- | --- |
| 2 | R3-204706 | QoE Configuration and reporting (Qualcomm Incorporated) | discussion |
| 3 | R3-204851 | Framework for QoE measurement collection (Nokia, Nokia Shanghai Bell) | discussion |
| 4 | R3-205181 | Initial Consideration On Study of NR QoE (ZTE) | discussion |
| 5 | R3-205182 | TP for FS_NR_QoE (ZTE) | other |
| 6 | R3-205200 | pCR for TR 38.8xx: NR QoE Management Framework (Ericsson) | other |
| 7 | R3-205201 | pCR for TR 38.8xx: NR QoE Measurement Triggering, Configuration, Collection and Reporting (Ericsson) | other |
| 8 | R3-205281 | NR QoE measurement overview (Huawei) | discussion |
| 9 | R3-205283 | Potential RAN3 impacts about QoE measurement (Huawei) | other |
| 10 | R3-205359 | Discussion on NR QoE solution for various service (CATT) | discussion |
| 11 | R3-205360 | Discussion on NR QoE solution architecture and interface impact (CATT) | discussion |
| 12 | R3-205370 | Some Considerations on QOE Collection in NR (China Telecommunications) | discussion |
| 13 | R3-205402 | Discussion on NR QoE requirements and potential solutions (Samsung) | discussion |
| 14 | R3-205403 | Discussion on NR QoE interface impact and solutions (Samsung) | discussion |
| 15 | R3-205439 | NR QoE management (CMCC) | discussion |

Other:

| 16 | R3-204708 | Interworking with LTE QoE (Qualcomm Incorporated) | discussion |
| --- | --- | --- | --- |
| 17 | R3-205183 | Consideration on slice QoE measurement (ZTE) | discussion |
| 18 | R3-205202 | pCR for TR 38.8xx: Use Cases for NR QoE Management (Ericsson) | other |
| 19 | R3-205282 | Initial discussion on latency measurement (Huawei) | discussion |