3GPP TSG RAN WG3 Meeting # 109-e 	R3-205489

e-meeting, 17-27th August 2020

Title:	CB: # 33_NTN_General

Source: 	Thales (moderator)

Type:	discussion

Document for:	Agreement

Agenda Item:	20.1

Work Item: 	NR_NTN_solutions: Solutions for NR to support non-terrestrial networks (NTN)\

# Introduction

This document aims at discussing and agree on reference scenarios to be considered during the Rel-17 WI NR_NTN_solutions.

Hereunder is recalled the description of the email discussion as defined by the RAN3 chair in its notes:

CB: # 33_NTN_General

- Th (5166)

Six transparent payload based satellite reference scenarios are considered for the Rel-17 work item “NR_NTN_solutions” characterised in the table below:

Key reference scenario parameters can be found in the table. It corresponds to the table 4.2-2 of TR 38.821 in which the scenarios referring to the regenerative payload option have been removed.

User equipment considered for the key reference scenario parameters can be found in the table.

UEs with capability on timing and frequency pre-compensation using their GNSS capabilities are assumed. However the support of UEs without capability on timing and frequency pre-compensation is not precluded in the subsequent release.

NTN GW supports all the necessary functions to forward the NR-Uu signal over the feeder link interface.

In a given tracking area (TAC), the association between physical satellite beams and cell IDs may be continuously reconfigured. A stationary UE on ground will be served by the same set of gNB ID.

- In case of Earth fixed beams, the same gNB ID, cell IDs are always associated to the same tracking area code (TAC) ("Stationary identifiers on ground");

- In case of Earth moving beams, cell IDs may be always associated to the same satellite beams ("Moving identifiers on ground");

Existing (REl-15&16) NG-RAN mobility procedures (Intra and Inter gNB) apply to NTN.

++ Suggested guidelines/topics for discussion from Chair (looking at possible consensus):

+ Note work plan

+ Endorse (if agreeable) reference scenarios

+ proposal on TAC & beam handling: to be discussed in CB 34 (common discussion beneficial)

(Thales - moderator)

Summary of offline disc R3-205489

# For the Chairman’s Notes

It is proposed to agree on the following:

- RAN3 to consider WI scenarios including LEO/GEO, Earth fixed/moving beams

- Mobility procedures in NTN should be based on Existing (Rel-15&16) with possible adaptations if needed

- The work plan described in [2] is considered as basis for work

- RAN3 to further discuss which of the Earth fixed or Earth moving beams scenarios should be addressed first

# Assumptions for Rel-17 NR-NTN-solutions WI

## NTN reference scenarios

### Proposal

“Proposal 1 (in R3-205166): Six transparent payload based satellite reference scenarios are considered for the Rel-17 work item “NR_NTN_solutions” characterised in the table below:

Table 2-1 Reference satellite scenarios for Rel-17 work item “NR_NTN_solutions”

| Scenarios | C1.1 | C1.2 | C2.1 | C2.2 | A1 | A2 |
| --- | --- | --- | --- | --- | --- | --- |
| Orbit | LEO @ 600 km altitude | LEO @ 600 km altitude | LEO @ 1200 km altitude | LEO @ 1200 km altitude | GEO @ 35,786 km altitude | GEO @ 35,786 km altitude |
| Frequency band | Sub 6GHz | Above 6GHz | Sub 6GHz | Above 6GHz | Sub 6 GHz | Above 6 GHz |
| Beams generation | Earth fixed beams (Note 1) | Earth fixed beams (Note 1) | Earth moving beams | Earth moving beams | Earth fixed beams | Earth fixed beams |

NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beam-forming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite

### Discussion

Note: from moderator: Earth moving beams may not be realistic for narrow beams and low altitude, due to excessive Hand-over rate. However, they may be envisaged at higher altitude and wider beams.

The organizations are invited to provide their views on the proposal 1 of R3-205166, recalled above.

| Organizations | View on the proposals above: Agree, Agree with changes, disagree and justify |
| --- | --- |
| Nokia | Agree with changes. We propose to study also earth moving beams for LEO@600 km altitude, while deprioritizing LEO @1200 km altitude. In addition, this has more impact to RAN1/2, and should be discussed in RAN1/2. |
| Intel | Agree |
| Ericsson | Is the list of reference scenarios planned to be captured in some 3GPP specification? is it for information only? Not sure what to do with this input in RAN3. Guess it is sufficient to work based on the approved NTN WID. |
| CATT | Only consider Earth fixed beams in LEO @ 600 km altitude scenarios? And only consider Earth moving beams for LEO @ 1200 km altitude scenarios? From RAN3 perspective, no big difference between the LEO scenarios, more impact should be investigated in RAN1/RAN2. |
| Huawei | What is the immediate standard (stage 2, Stage 3) impact and expectation of this proposal for RAN3? There is already a work task defined by the WID with the RAN3 impacts. |
| Qualcomm | The proposal seems to be to tie LEO earth fixed to 600km, and LEO moving beam to 1200km altitude. From our point of view, it seems this discussion could be left to RAN1/2. |
| Thales | We agree with this scenario selection. Note that Scenarios where Non Geo Synchronous Orbit constellation (e.g. LEO and MEO) generate Earth moving beams may be not realistic when the beams are narrow or the altitude is low, due to excessive Hand-over rate that will be created. Therefore Earth moving beams could be restricted to wide beams and higher altitude. On the other hand, Earth fixed beam scenarios are best suited for narrow beams (for broadband and handheld applications) and applicable at any altitude thanks to well proven phased array technology on board satellites. Note that the difference in altitude for LEO scenarios doesn’t have any impact on RAN3 and hence should be considered as informative for RAN3 |
| ZTE | Agree, just informative for RAN3. Thanks! |
| Fraunhofer IIS/HHI | Agree. Even if some of the parameters does not have impact from a RAN3 perspective, we see the need to have the same scenarios defined for all RAN WGs. |
| Hughes/EchoStar | Discussion should be left to RAN1/2. Assumptions/scenarios should be consistent between all WG |

### Possible way forward

About the proposed

- Agree: 4 organizations (Intel, Thales, ZTE, Fraunhofer)

- Agree with changes:  3 organizations (Nokia, Ericsson, Hughes)

- Disagree: 3 organizations (Huawei, CATT, QC)

About the suggestions:

- Earth moving beams to be considered also @600km

  - Moderator: RAN2 agreed to “stick to WI scenarios” which include both Earth fixed and moving scenarios at all altitude. So RAN3 to also support all WI scenarios

- CATT, HW, ZTE consider that there is no need to agree on this proposal because LEO scenarios characteristics (Earth fixed/moving beam) are transparent to RAN3

Based on the above, the moderator suggests the following approach:

- To just agree that RAN3 will consider all WI scenarios

Based on the above, the moderator suggests to revise the proposals as follow:

Proposal 2.1.1: RAN3 to consider WI scenarios including LEO/GEO, Earth fixed/moving beams

## Key parameters of the NTN scenarios

### Proposal

“Proposal 2 (in R3-205166) : The key reference scenario parameters can be found in table 4.2-2 of this document. It corresponds to the table 4.2-2 of [2] in which the scenarios referring to the regenerative payload option have been removed.”

Table 4.2-2: Reference scenario parameters

| Scenarios | GEO based non-terrestrial access network (Scenario A) | LEO based non-terrestrial access network (Scenario C) |
| --- | --- | --- |
| Orbit type | notional station keeping position fixed in terms of elevation/azimuth with respect to a given earth point | circular orbiting around the earth |
| Altitude | 35,786 km | 600 km 1,200 km |
| Spectrum (service link) | <6 GHz (e.g. 2 GHz) >6 GHz (e.g. DL 20 GHz, UL 30 GHz) |  |
| Max channel bandwidth capability (service link) | 30 MHz for band < 6 GHz 400 MHz for band > 6 GHz |  |
| Payload | Scenario A: Transparent (including radio frequency function only) | Scenario C: Transparent (including radio frequency function only) |
| Inter-Satellite link | No | No |
| Earth-fixed beams | Yes | Scenario C1: Yes (steerable beams), see note 1 Scenario C2: No (the beams move with the satellite) |
| Max beam foot print size (edge to edge) regardless of the elevation angle | 3500 km (Note 5) | 1000 km |
| Min Elevation angle for both sat-gateway and user equipment | 10° for service link and 10° for feeder link | 10° for service link and 10° for feeder link |
| Max distance between satellite and user equipment at min elevation angle | 40,581 km | 1,932 km (600 km altitude) 3,132 km (1,200 km altitude) |
| Max Round Trip Delay (propagation delay only) | Scenario A: 541.46 ms (service and feeder links) | Scenario C: (transparent payload: service and feeder links) 25.77 ms (600km) 41.77 ms (1200km) |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21 ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000 045 ppm/s | 0.27 ppm/s (600km) 0.13 ppm/s(1200km) |
| Maximum Delay variation as seen by the UE (note 7) | Negligible | Up to +/- 48 µs/sec (600 km) Up to +/- 42 µs/sec (1200 km) |
| Service link | 3GPP defined New Radio |  |
| Feeder link | 3GPP defined Radio interface | 3GPP defined Radio interface |
| NOTE 1: Each satellite has the capability to steer beams towards fixed points on earth using beamforming techniques. This is applicable for a period of time corresponding to the visibility time of the satellite NOTE 2: Max delay variation within a beam (earth fixed user equipment) is calculated based on Min Elevation angle for both gateway and user equipment NOTE 3: Max differential delay within a beam is calculated based on Max beam foot print diameter at nadir NOTE 4: Speed of light used for delay calculation is 299792458 m/s. NOTE 5: The Maximum beam foot print size for GEO is based on current state of the art GEO High Throughput systems, assuming either spot beams at the edge of coverage (low elevation). NOTE 6: The maximum differential delay at cell level has been computed considering the one at beam level for largest beam size. It does not preclude that cell may include more than one beam when beam size are small or medium size. However the cumulated differential delay of all beams within a cell will not exceed the maximum differential delay at cell level in the table above. NOTE 7: The delay variation measures how fast the round trip delay (function of UE-satellite-NTN gateway distance) varies over time when the satellite moves towards/away from the UE. It is expressed in µs/s and is negligible for GEO scenario |  |  |

The NTN study results apply to GEO scenarios as well as all NGSO scenarios with circular orbit at altitude greater than or equal to 600 km.

### Discussion

The organizations are invited to provide their views on the proposal 2 of R3-205166, recalled above. RAN3 should focus on the parameters that falls in its area of work.

| Organizations | View on the proposals above: Agree, Agree with changes, disagree and justify |
| --- | --- |
| Nokia | Agree with changes Earth moving beams need to be supported at 600 km altitude as well. In addition, this has more impact to RAN1/2, and should be discussed in RAN1/2. |
| Intel | Agree |
| Ericsson | Is the list of key reference scenario parameters planned to be captured in some 3GPP specification? is it for information only? Not sure what to do with this input in RAN3. Guess it is sufficient to work based on the approved NTN WID. |
| Huawei | Same question as previous one and Ericsson…. |
| Qualcomm | Same comment as Ericsson / Huawei |
| Thales | We agree with these key parameters which are more relevant to RAN1/2 and hence should be considered as informative for RAN3 |
| ZTE | Agree, just informative for RAN3. Thanks! |
| Fraunhofer IIS/HHI | Agree, for information only. |
| Hughes/EchoStar | Agree, for information |

### Possible way forward

About the proposal to consider the key reference scenario parameters in table 4.2-2 of TR 38.821 for Rel-17 normative work.

- Agree: 1 organizations (Intel)

- Agree with changes: 5 organizations (Nokia, Thales, ZTE, Fraunhofer, Hughes)

- Disagree: 3 organizations (Ericsson, Huawei, QC)

About the suggestions:

- The key reference scenario parameters is out of RAN3 scope

Based on the above, the moderator suggests the following approach:

- There is no need to agree something specific wrt to the table 4.2-2 of TR 38.821 in RAN3.

## UE types

### Proposal

“Proposal 3(in R3-205166): The User equipment considered for the key reference scenario parameters can be found in table 4.3-1 of this document.

Table 4.3-1 Reference satellite scenarios: User equipment types

| User equipment characteristics | Handheld | VSAT (Note 1) |
| --- | --- | --- |
| Antenna type | Omnidirectional antenna | Directive antenna |
| Motion on the earth | 500 km/h (e.g. on board a high speed train) | Up to 1200 km/h (e.g. aircraft mounted) |
| Antenna types | Omnidirectional antenna | Directional antenna (up to 60 cm equivalent aperture diameter) |
| Antenna polarisation | Linear: +/-45°X-pol | circular |
| Max transmit power | up to 200 mW (power class 3) | up to 20 W |
| Note 1 : VSAT terminal characteristics could be implemented with phased array antenna. It may be mounted on Moving platforms (e.g., aircrafts, vessels) or building |  |  |

Proposal 4 (in R3-205166): UEs with capability on timing and frequency pre-compensation using their GNSS capabilities are assumed. However the support of Ues without capability on timing and frequency pre-compensation is not precluded in the subsequent release.”

### Discussion

The organizations are invited to provide their views on the proposals 3 (1st antenna type row has been erased because there was a redundancy) of R3-205166 recalled above. RAN3 should focus on the characteristics that falls in its area of work.

| Organizations | View on the proposals above: Agree, Agree with changes, disagree and justify |
| --- | --- |
| Nokia | Agree with changes. Suggestion to add up to 500 km/h under movement. In addition, this has more impact to RAN1/2, and should be discussed in RAN1/2. |
| Intel | Agree |
| Ericsson | Is the list of UE characteristics planned to be captured in some 3GPP specification? is it for information only? Not sure what to do with this input in RAN3. Guess it is sufficient to work based on the approved NTN WID. |
| CATT | The UE characteristics should be further checked and decided in RAN1/RAN2, we do not see any real impact to RAN3 specs. |
| Huawei | Same question as previous There is already task to work on fix and moving beam by the work item which is the main impact for RAN3. |
| Qualcomm | This seems primarily outside RAN3 scope. |
| Thales | We agree with proposal 3. It may be considered as informative for RAN3. We agree with Nokia suggestion to add up to 500 km/h under movement. |
| ZTE | Agree, just informative for RAN3. Thanks! |
| Fraunhofer IIS/HHI | Agree. |
| Hughes/EchoStar | Agree |

The organizations are invited to provide their views on the 4 of R3-205166 recalled above. RAN3 should focus on the characteristics that falls in its area of work.

| Organizations | View on the proposals above: Agree, Agree with changes, disagree and justify |
| --- | --- |
| Nokia | Agree, but the accuracy of the UE-based time and frequency-compensation needs to be defined (by RAN1?). The support of UEs without such capabilities (or Ues that do not currently use the capabilities) is also needed. |
| Intel | Agree |
| Ericsson | Confusion increases, sorry. |
| CATT | Agree, UEs with capability on timing and frequency pre-compensation using their GNSS capabilities are assumed in Rel-17. |
| Hauwei | Same question as previous … Our view is to confirm that all UEs have capability to do time and frequency pre-compensation since it can reduce the difficulty for network to handle it. However, it is a RAN2 issue. |
| Qualcomm | Sorry but this seems entirely a RAN1/2 issue. |
| Thales | We agree with proposal 4. It may be considered as informative for RAN3 since it is more relevant for RAN1/2. |
| ZTE | Agree, just informative for RAN3. Thanks! |
| Fraunhofer IIS/HHI | Agree. The question if timing and frequency compensation could be considered at UE side or not shall be agreed in RAN1. |
| Hughes/EchoStar | Agree for information |

### Possible way forward

About the proposal to define the characteristics of the User equipment types and the UEs capability in terms of GNSS and timing and frequency pre-compensation to be considered for Rel-17 normative work

- Agree: 2 organizations (Intel, Fraunhofer)

- Agree with changes:  4 organizations (Nokia, Thales, ZTE, Hughes)

- Disagree: 4 organizations (Ericsson, CATT, Huawei, QC)

About the suggestions:

- The characteristics of the UE types and their capabilities in terms of GNSS and pre compensation is out of RAN3 scope

Based on the above, the moderator suggests the following approach:

- There is no need to agree something specific wrt to this topic in RAN3.

## NTN GW characteristics

### Proposal

“Proposal 5 (in R3-205166): The NTN GW supports all the necessary functions to forward the NR-Uu signal over the feeder link interface.”

### Discussion

This assumption, recalled above, has already been endorsed during the study and should be assumed for the normative phase and therefore, there is no need to discuss it further.

## Network Identities Handling

### Proposal

“Proposal 6 (in R3-205166): In a given tracking area (TAC), the association between physical satellite beams and cell IDs may be continuously reconfigured. A stationary UE on ground will be served by the same set of gNB ID.

- In case of Earth fixed beams, the same gNB ID, cell IDs are always associated to the same tracking area code (TAC) (“Stationary identifiers on ground”);

- In case of Earth moving beams, cell IDs may be always associated to the same satellite beams (“Moving identifiers on ground”);”

### Discussion

The organizations are invited to provide their views on the proposal 6 of R3-205166, recalled above.

| Organizations | View on the proposals above: Agree, Agree with changes, disagree and justify |
| --- | --- |
| Nokia | Disagree. In case of earth moving beam, the cell ID is always associated with the same gNB, same as earth fixed beam. |
| Intel | This is a big decision, perhaps we need to discuss actual solutions to see how this works. |
| Ericsson | Finally, we arrived on good old RAN3 soil ;-) However, as indicated by Mr. Chairman, this should be handled in another comeback. |
| CATT | It seems this should be further discussed in the CB # 34. |
| Huawei | First point is discus in other CB. Second point introduce a confusion with the “may”, such scenario was not capture in the TR. All thoughts nothing precludes that you switch off a moving cell associated to a satellite beam and switch on it with new some identifier when there is a cross border for example … But this is not a standard matter … Please clarify…. |
| Thales | We agree with this proposal but it should be further discussed as part of the CB#34 |
| ZTE | Share the same view as Nokia’s. |
| Fraunhofer IIS/HHI | Agree. This shall be discussed in the email discussion CB#34. |
| Hughes/EchoStar | This is RAN2 discussion |

### Possible way forward

About the proposed

- Agree: 0 organizations

- Agree with changes:  2 organizations (Thales, Fraunhofer)

- Disagree: 7 organizations (Nokia, Intel, Ericsson, CATT, Huawei, ZTE, Hughes)

About the suggestions:

- Network Identities Handling aspects are being discussed in CB#34 thread in this meeting

Based on the above, the moderator suggests the following approach:

- Take no decision in this email discussion wrt to this topic

## Connected mode mobility

### Proposal

“Proposal 7  (in R3-205166) Existing (Rel-15&16) NG-RAN mobility procedures (Intra and Inter gNB) apply to NTN.”

### Discussion

The organizations are invited to provide their views on the proposal 7 of R3-205166, recalled above.

| Organizations | View on the proposals above: Agree, Agree with changes, disagree and justify |
| --- | --- |
| Nokia | Agree. |
| Intel | Agree |
| Ericsson | Also this doesn’t seem to be the scope of this “General” Agenda Item and Comeback. |
| CATT | Agree, from RAN3 perspective, the existing (REl-15&16) NG-RAN mobility procedures (Intra and Inter gNB) could be applied to NTN. However, Feeder link switch is a special case, which may require some new functions in our interface. This will be further discussed in CB # 38. |
| Huawei | The spirit of the proposal is fine, however it the statement is too large, with impact on RAN2, may be SA2 etc … We prefer to not have agreement on this proposal. |
| Qualcomm | Same view as Huawei and Ericsson |
| Thales | We agree that mobility procedures in NTN should be based on Existing (Rel-15&16) NG-RAN mobility procedures |
| ZTE | We agree that mobility procedures in NTN should be based on Existing (Rel-15&16) NG-RAN mobility procedures |
| Fraunhofer IIS/HHI | Agree. |
| Hughes/EchoStar | Agree |

### Possible way forward

About the proposed

- Agree: 3 organizations (Nokia, Intel, Fraunhofer, Hughes)

- Agree with changes : 3 organizations (CATT, Thales, ZTE)

- Disagree: 3 organizations (Ericsson, Huawei, QC)

About the suggestions:

- Reword proposal as follow “mobility procedures in NTN should be based on Existing (Rel-15&16) with possible adaptations if needed”

Based on the above, the moderator suggests to revise the proposals as follow:

Proposal 2.6.1: mobility procedures in NTN should be based on Existing (Rel-15&16) with possible adaptations if needed

# Work plan

## Proposal

Thales in [2] provided a draft work plan for the NR_NTN_solutions WI applicable to RAN1, 2 and 3.

## Discussion

The organizations are invited to provide enhancement to the RAN3 planned activities if any.

| Organizations | View on the proposals above: Agree, Agree with changes, disagree and justify |
| --- | --- |
| Nokia | ok. BTW, the work plan submitted in RAN3 is mainly for information. There is no agree or disagree. |
| Huawei | Should the moving beam be considered as same priority than fixed beam? Moving beam should be considered as the lower priority since it has much more issues than fixed beam. |
| Qualcomm | Agree with Nokia. On moving vs fixed, potentially it could be argued that moving has priority as it is more general. |
| Thales | The work can be endorsed as is, and further updated in subsequent sessions if required. Both Earth fixed and moving beam scenarios are to be considered. Each having its own challenges. Earth fixed bam should be considered first as it could be the easiest to address. |
| ZTE | Agree with Thales’s views. |
| Fraunhofer IIS/HHI | Agree with Thales. However, we see also benefits for earth moving beams, especially in lower capacity scenarios. |
| Hughes/EchoStar | Moving beam should eb priority OR at a minimum same priority as fixed beam. |

### Possible way forward

About the proposed work plan for RAN3 activities in the WI

- Agree: 3 organizations (Thales, ZTE, Fraunhofer)

- Agree with changes: 2 organizations (Nokia, Hughes)

- Disagree: -

About the suggestions:

- Normative to start with Earth fixed bam and then continue with Earth moving beams

  - Agree:  HW, Thales, ZTE, Fraunhofer because Moving beam create more issues than fixed beam

  - Disagree: QC, Hughes because moving beam is a more general case.

Based on the above, the moderator suggests to revise the proposals as follow:

Proposal 3.1bis: The work plan described in [2] is considered as basis for work

Proposal 3.2bis: RAN3 to further discuss which of the Earth fixed or Earth moving beams scenarios should be addressed first

# Conclusion

To be completed based on the outcome of the email discussion.

# Reference

[1] R3-2005166	NR NTN Reference scenarios definition for Rel-17 normative phase	Thales

[2] R3-2005165	NR_NTN_solutions work plan	THALES

END