3GPP TSG-RAN WG3 #109-e	R3-205116

E-meeting, 17th – 27th August 2020

Source:	CATT

Title:	Discussion on RACH optimisation for 2-step RACH

Agenda Item:	10.2.5

Document for:	Discussion and decision

Introduction

2-step RACH is a new feature introduced by RAN1/2 in Rel-16, and is included in the WI objective for Rel-17 SON. In this contribution, we share our opinion on how to support RACH optimisation for this new feature.

Discussion

Collecting data for RACH optimisation mainly comprises two functions which has impact on 3GPP specifications: delivering data for PRACH coordination in order to prevent PRACH collision, and delivering reports from UEs each of which states how a random access procedure succeeds or fails, or “RACH report” for short. This also applies for the feature 2-step RACH.

For the PRACH coordination part, generally speaking, the PRACHs used for 2-step RACHs were designed following a similar approach as the conventional PRACHs for CBRAs/CFRAs. It may use a PRACH different from the conventional ones, but how it works is the same. Their encoding within TS 38.331 is also essentially the same as conventional ones, with differences only on the field names, making the IEs used to indicate PRACHs (i.e. §9.3.1.140 “NR PRACH Configuration List” in TS 38.473) reusable for 2-step RACH, thanks to the fact that it is already designed as a list. And, PRACH collision will occur regardless of the two PRACHs are used for conventional CBRAs/CFRAs or 2-step RAs, making it no use to distinguish the PRACHs used for 2-step RA from the ones used for any conventional types of RAs.

Therefore, we do not observe any necessary essential revise on RAN3 specs w.r.t. PRACH coordination for 2-step RACH. Changes on IE names, semantic descriptions or stage 2 descriptions are not precluded, e.g. “MSG1-FDM” can be renamed to “PRACH-FDM” in order to cover the new feature semantically.

Observation 1: No necessary essential revise on RAN3 specs is observed w.r.t. PRACH coordination for 2-step RACH, while some changes on IE names, semantic descriptions or stage 2 descriptions may be beneficial.

On the other side, there are some new IE on MAC-layer parameters, e.g. at least the IEs used to indicate when to use 2-step RA, and when to use the conventional CBRA/CFRA, such as msgA-RSRP-Threshold-r16 and msgA-TransMax-r16. These IEs can also be subject of self-optimisation. In order to optimise these IEs, we need RACH reports distinguishing 2-step RAs, with potential fallback by MSGB, form conventional ones as self-optimisation algorithm inputs.

Observation 2: In order to provide inputs to optimise the parameters such as msgA-RSRP-Threshold-r16 and msgA-TransMax-r16, RACH reports should distinguish 2-step RAs, with potential fallback by MSGB, from conventional RAs.

Considering the fact that RACH Optimisation is an SON feature in charge by RAN3 while the RACH report from UE to network is mostly related with Uu and more suitable to be discussed in RAN2, we propose to send an LS to RAN2 to kick off the discussion on RACH reporting for 2-step RAs.

Proposal: We propose to send an LS to RAN2, in order to trigger the discussion of RACH reporting for 2-step RACH.

Conclusion

Observation 1: No necessary essential revise on RAN3 specs is observed w.r.t. PRACH coordination for 2-step RACH, while some changes on IE names, semantic descriptions or stage 2 descriptions may be beneficial.

Observation 2: In order to provide inputs to optimise the parameters such as msgA-RSRP-Threshold-r16 and msgA-TransMax-r16, RACH reports should distinguish 2-step RAs, with potential fallback by MSGB, from conventional RAs.

Proposal: We propose to send an LS to RAN2, in order to trigger the discussion of RACH reporting for 2-step RACH.

Based on the abovementioned proposal, we draft an LS accordingly [1].

Reference

[1] R3-205117; LS to RAN2 on RACH report for 2-step RACH; CATT.