3GPP TSG-RAN WG3 Meeting #110-e	R3-206858

- Online, November 2nd – 12th 2020

Agenda Item:	13.3.1

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on IAB Congestion Mitigation

Document for:	Approval

# Introduction

This is the SoD for the following comeback: CB: # 15_IABcongestionMitigation

The deadline for providing replies to Phase 1 is Thursday, November 5th at 23.59 UTC.

Relevant papers:

- R3-206004 Discussion on UP-based congestion mitigation in IAB (Samsung)

- R3-206210 Congestion Indication to CU-CP (Intel Deutschland GmbH)

- R3-206297 Discuss the improvements to DDDS for IAB UP-based congestion mitigation (CATT)

- R3-206563 Discussion on DL E2E flow and congestion control in R17-IAB (ZTE, Sanechips)

- R3-206589 Downlink End-to-End Flow Control in IAB Networks (Ericsson)

- R3-206590 CR TS 38.425 IAB End-to-End Flow Control Feedback (Ericsson)

- R3-206647 Discussion on using DDDS for IAB congestion mitigation (Nokia, Nokia Shanghai Bell)

- R3-206649 Discussion on CP-based approach for DL congestion mitigation (LG Electronics)

- R3-206671 Discussion on IAB E2E flow control (Huawei)

# For the Chairman’s Notes

TBW

# Discussion

At the RAN3#109-e meeting the following was agreed:

UP-based and CP-based approaches for DL congestion mitigation in IAB networks are complementary.

In IAB DL end-to-end flow control, the access node sends feedback to the donor-CU-UP.

Discuss the improvements to DDDS for IAB UP-based congestion mitigation (e.g. packet marking, highest PDCP SN received from parent node, receiving data rate, received data volume).

The measures taken by the donor-CU-CP based on the CP-based approach are up to implementation.

End-to-end UL flow control is deprioritized in Rel17.

In this Summary of offline discussion, only the proposals that do not contradict the above agreements are considered.

## CP-based congestion mitigation

The contributions, [2], [4], [8] and [9] discuss the CP-based congestion mitigation. In [4], [8] and [9] (as well as in [4], indirectly) it is proposed that the DL congestion indication is sent from an IAB node at the parent side of the congested link to the IAB-donor-CU-CP.

Q1: Do you support sending a DL congestion indication from an IAB node at the parent side of the congested link to the IAB-donor-CU-CP?

Paper [8] proposes that the trigger for congestion indication is a failure to resolve congestion during a given time period. Paper [3] proposes that the congestion indication is sent to the IAB-donor-CU-CP when the BAP flow control indicator has been sent N times, in relation to the same congestion event. Paper [4] proposes to discuss the triggering mechanism for sending the congestion indication. On the other hand, RAN3 usually does not specify triggers for sending various indications. Therefore, the corresponding question is formulated as follows:

Q2: Should RAN3 discuss the triggers for sending the congestion indication?

Paper [4] proposes to discuss the granularity and content of the congestion indication. Meanwhile, paper [9] proposes to report the indication per child link level, by enhancing the existing GNB-DU STATUS INDICATION in F1-AP.

Q3: Should the congestion be reported per child link level and should the GNB-DU STATUS INDICATION be used for this purpose?

| Company | Answer and motivation |
| --- | --- |
| Ericsson | Q1: The use case and the claimed benefits need to be clarified. We think that the donor CU has enough information to take appropriate actions, if necessary. In legacy F1AP, we do not indicate the DU overload per UE, so it needs to be clarified why the DU should indicate which child link is congested. The CP-based measures are expected to kick in if the UP-based approach does not help. The main reason for congestion (i.e. queue build-up) is channel quality, whose deterioration, in general, is not expected to last long. Therefore, the use case is also unclear. Q2: No. Even if the CP-based approach is specified, we should not specify triggers for an action, this is up to implementation. Q3: No, see above. |
| Qualcomm | Q1: Yes. On use case and benefits: Opposed to wireline backhaul, which can be over-dimensioned, the wireless backhaul is a scarce resource. The IAB-donor-CU-CP holds the functionality to dynamically optimize the wireless backhaul by changing topology, routing and DU resource configuration and therefore maximize the effective BH capacity. This is only possible if the IAB-donor-CU-CP obtains refined information on the IAB-DU’s DL load/congestion. That is not the case for Rel-16 IAB. Q2: Yes. Since the IAB-donor-CU-CP has to perform topology-wide optimization it is imperative to have comparable information from all nodes. Therefore, it should be able to determine the trigger conditions for such reports. This may be a novelty for RAN3, as the moderator pointed out, since before IAB, RAN3 was never faced with the problem to optimize a complex multi-hop wireless topology. Q3: More discussion needed. On per-child-link reporting: It is not clear if this is necessary or if per-ingress-RLC-CH reporting may be sufficient. We need more discussion on the information reported. On GNB-DU STATUS INDICATION: We don’t have objections, but this issue is not so critical and can be discussed at later point in time. |

## UP-based congestion mitigation

The submitted contributions propose various enhancements to DDDS:

- Paper [1] proposes to consider the reporting of received date volume and received data rate. Packet marking is also proposed for consideration, with some additional clarifications.

- Paper [5] proposes marking of DL packets that experience an increasing queuing delay at intermediate nodes.

- Paper [7] proposes that, if needed, for UM mode, the DDDS is enhanced with a bitmap of PDUs transmitted to lower layers out of sequence.

- Paper [9] proposes an indication of the highest PDCP SN received from parent node.

Q4: Please state your view on each of the above proposals.

| Company | Answer and motivation |
| --- | --- |
| Ericsson | Packet marking: this is a proactive approach, enabling flow control reaction even before the congestion occurs. Due to the multihop nature of IAB communication, discovering early signs of congestion is highly desirable. IAB is essentially wireless multihop, and we cannot afford reactive solutions. Nevertheless, we are open to discussing additional indications to be included in the DDDS. Regarding the concerns about packet marking expressed in papers: - “Packet marking may cause further delay for mitigation congestion” in [3] - in fact, packet marking is a pre-emptive approach, enabling the donor to react before any congestion occurs. - “How to identify the distribution of the marked packets among intermediate nodes” in [1] – the distribution of marked packets among intermediate nodes is irrelevant, because the reaction to imminent congestion is to slow down the traffic on DRBs, which are e2e. Moreover, the donor-CU-UP knows nothing about the topology, so the info about which node marked the packets is of little to no value. Received volume and Receiving data rate: we are open to considering this, but it needs to be analyzed with respect to the fact that the delivery of DDDS to the donor CU will take some time, during which both the received rate and the received data volume may change significantly. This approach may be considered slightly proactive, but the problem is that the rate changes quite dynamically. Indication of the highest PDCP SN received from parent node: today we have the NR-U SN that indicates the delivery status on the transport network. The delta wrt NR-U SN needs to be clarified. This approach is essentially reactive. Bitmap of PDUs transmitted to lower layers out of sequence: the processing bitmap-based indications is computationally expensive. This approach is essentially reactive. |
| Qualcomm | Received data volume and data rate: This needs more clarification. The data volume and date rate received should be the same as the data volume and data rate transmitted minus the packets that were declared as “lost” in DDDS. Obviously, some packets may be reported by DDDS as “lost” while they got solely delayed due to HARQ/ARQ in the multiple hops. A properly designed congestion control mechanism on the CU-UP should be able to handle this. TCP, for instance, has the RTO timer to determine when to hope that a packet may still be delivered and when to give up. Packet marking: Yes, we support this approach. Packet marking is also used in ECN in TCP/IP and it has been shown many times that it is superior to packet-loss indication. Bitmap indicating sequence order: This discussion tries to fix a problem where F1-U is split among multiple routing paths. RAN3 agreed that such F1-U split should not be supported in Rel-17. Highest PDCP SN report: Agree with Ericsson that NR-U SN should be used for this purpose. PDCP SNs, in fact, may be useless in case UE uses split bearer. |

# Conclusion, Recommendations

TBW