3GPP TSG-RAN WG3 #108-e	R3-206861

Online, 1-11 June 2020

Agenda Item:	20.2.1

Source:	Ericsson, Qualcomm (moderators)

Title:	CB: # 18_NTN_IDhandling - Summary of email discussion

Document for:	Approval

# Introduction

Chair’s reflections copied below

CB: # 18_NTN_IDhandling

- note LS; discuss possible reply and impact to RAN3 protocols

FH 5940

- the list of network identities encompasses gNB ID, NCGI, NCI, NPCI, TAC, TAI, NID, and CAG ID.

- The network identities TAC and TAI have been studied and two new solutions, i.e. multiple TAI and periodic registration area update have been introduced.

- decide whether the previously introduced solutions for TAC/TAI of multiple TAIs and/or periodic registration area update are valid solutions or need further study in coordination with RAN2.

CATT 6275

- Network Identifiers can be decoupled from strictly fixed (constant and exclusive mapping) on geographical areas, although they can be associated to them.

- CGI for a given PLMN should keep constant when a NTN cell is moving over the earth, unless there is any topological shift on the link, e.g. switching to another satellite, another NTN gateway, or another gNB.

- One cell broadcasts only one TAI per PLMN, and the boundary of TAs can float in transparent LEO with earth moving cell scenario.

E/// 6381

- Strive for RAN2 confirming the assumptions concerning SIB content in moving satellite cells, i.e. the necessity of broadcast all Tracking Areas corresponding to the momentary coverage area of a moving satellite beam, to cover Tracking area borders.

- Agree that it is not necessary to inform the 5GC or neighbor gNBs about the fact that Tracking Areas reported to be served by a gNB are not continuously served by that gNB.

- Agree from a RAN3 point of view, that a Cell Global Identity corresponds to a fixed geographical area. This is applicable to fixed as well to earth moving cells. Inform RAN2 and SA2 and ask for confirmation.

- Strive for RAN2 confirming the assumptions that SIB content in moving satellite cells need to correspond to the momentary coverage area of a moving satellite beam with respect to fixed geographical area definition of a Cell ID.

- Agree that in case several Cell IDs would correspond to the same geographical (cell) area, it can be assumed that neighbor gNBs and the 5GC is able to cope with such ambiguity.

- Agree that in case several Cell IDs would correspond to the same geographical (cell) area, neighbor gNBs do not need to be constantly updated by the currently applicable set of Cell-IDs.

Intel 6400

- NTN enhancements standardized in RAN3 should strive to minimize 5GC (and by extension – NG-AP) impacts.

- Rule out solution 4 (CN receives “cell ID” and geographical coordinates).

- NTN does not require changes to the “User Location Information” NG-AP IE.

Chair: Consensus that “autonomous” periodic reconfiguration (OAM configures once and for all, then RAN nodes periodically applies config changes) seems better than signaling-assisted (e.g. periodic Xn signaling exchange)? If so, attempt st2 TP

(E///,QC - moderator)

Summary of offline disc R3-206861

# For the Chairman’s Notes [to be confirmed after discussions]

Propose to capture the following:

Issue 1 - Cell ID maps to a fixed geographical area:

It is agreed that a Cell ID corresponds to a fixed geographical area.

Issue 2 - minimizing 5GC/NGAP impact:

WA: RAN3 strives for minimizing NGAP impact for NTN. [probably with additions along discussions below]

Issue 3 - consequences of network identifiers mapped to fixed geographical areas:

to be added.

Issue 4 - avoid periodic reconfiguration: agree on the following

If solutions to support fixed geographical areas for TAI/Cell ID result in TAIs/Cell IDs being served only part-time by a gNB, this should not result in periodic configuration update on NG/Xn. gNBs are able to apply “autonomous” reconfiguration by means of OAM.

# Discussion

## Cell ID maps to a fixed geographical area (applying the same decision as for TA)

Given the SA2 LS in R3-206842 and input papers to RAN3#110-e, it appears to be straight forward  to confirm SA2’s assumption. Please provide your view on that approach.

| Company | Comment |
| --- | --- |
| Nokia | Agree the cell ID provided to the CN is mapped to a fixed geographical area. It also needs to be clarified that it is the same cell ID that is seen by the UE, and the one sent to the CN. |
| Intel | Agree that the cell provided to the CN in ULI is mapped to a fixed geographical area. We are not sure about other cases, in particular about the cell id seen by a UE – at any rate, that is for RAN2 to decide. |
| InterDigital | Agree |
| Qualcomm | We agree with this because it should minimize impacts on the 5GCN side. A 5GCN can continue to treat a cell ID as an approximate known location for a UE. |
| Thales | Agree. Just like the tracking area, the Cell Id exchanged over the NG interface should corresponds to an Earth fixed geographical area in order to minimize the impact of 5GCN. |
| CATT | We agree with the intension to minimize the impacts to 5GC. But we see some issues to be clarified/fixed in RAN side. Why not to use the LCS service in 5GC instead of provision of a geographical fixed cell id in ULI to 5GC. We understand there’re at least two interpretations on such kind of geographical fixed cell identity, as below: - Interpretation 1: the geographical area fixed cell ID is a kind of new defined “virtual cell id”, it’s only used in the ULI for LEO with earth moving cell scenario. The other functions in NG-RAN still use the real NR cell identity (the ID is mapped to the earth moving cell). - Interpretation 2: the geographical fixed Cell ID is a kind of “real logical cell id”, to be used for all the RAN functions, e.g. cell reselection, handover, etc. For the Interpretation 1, how can NG-RAN know the accurate UE location and map it to the geographical fixed virtual cell id? If gNB could get the GNSS info, why not use it directly? If the virtual ID is acquired by the UE, how to report it to RAN and whether it’s realizable enough? For the Interpretation 2, how to maintain the relationship between the physical NTN cell (earth moving cell) and the geographical fixed cell ids need to be further checked, e.g. how to broadcast and update the cell ids in the SI of a physical cell? We see both of the two interpretations have several questions/issues to be clarified/fixed, RAN2 should also be involved. To make life easier, we prefer not to make NR Cell IDs fixed on the ground, just invalidate the NR-Cell-ID in ULI in earth moving cell scenario, and 5GC could rely on the LCS service if needed. |

## 5GC impact: strive for minimizing 5GC/NGAP impact

This topic has several aspects: impact on ULI, additional provision of more granular location information and maybe others. Please provide your view, and, if applicable, add aspects and your view.

| Company | Comment |
| --- | --- |
| Nokia | Agree to minimize the 5GC/NGAP impact |
| Intel | Agree |
| InterDigital | Agree |
| Qualcomm | Agree |
| Thales | Agree to minimize the 5GC/NGAP impact. A TAI could be defined with vertices coordinates preventing any cross border TAI. It shall still corresponds to a List of Cell Ids. There could be at least 1 TAI per country. Hence Cell Id over NG interface should also correspond to a static geographical area defined (e.g. vertices coordinates). Also no cross border Cell Ids shall be defined |
| CATT | In principle we agree with the intension, but we prefer to invalidate the conventional NR-Cell-ID based ULI mechanism and rely on LCS in case of the earth moving cell case. LCS is nothing new anyhow so no need to change the NGAP either. |

## Consequences of network identifiers mapped to fixed geographical areas.

In order to operate an NTN, basic network identifiers (PLMN ID, TAI, Cell ID) as used on network interfaces (e.g. served cells on Xn, served TAs on NG), Uu (e.g. PLMN index in msg5), in broadcast content and in UE procedures (e.g. ULI on NG) need to have the same characteristics and follow the same assumption, especially their relation to fixed geographically represent.

a)	One possible approach would be to assume that SIB content corresponds to momentary coverage area of a beam related to the geographically fixed areas of PLMNs/TAs/Cells - irrespective of whether the beam is fixed or moving. We could ask RAN2 to confirm this assumption.

b) Another possible approach would be discuss de-coupling (but associating) network identifiers from its fixed geographical relation (6275, CATT).

Please provide your view.

| Company | Comment |
| --- | --- |
| Nokia | Ok to ask RAN2 to clarify for both a and b. |
| Intel | Yes, we should ask RAN2. |
| InterDigital | Ask RAN2 |
| Qualcomm | In general, we agree that it possible to go both ways (i.e. decoupling Uu IDs from e.g. NGAP and providing mapping to CN, or have the SIBs reflect “instantaneous coverage”). For sure the second is highly dependent on RAN2, but even the first may not be possible without RAN2 support (i.e. mapping may require some information on UE positioning). Note that in option b), we still need to discuss what happens in Xn. Our preference would be to answer SA2 accepting the challenge, but involving RAN2 directly now on “how” to make this work. |
| Thales | The challenges are For a) When a specific beam covers several of the geographically fixed areas, what set of SIB(s) (including Cell Id(s) and all other parameters) should be broadcasted ? Note that a beam may well cover more than 2 geographical cells. - This would require to ask RAN2 about the feasibility For b) The need is to define “geographical Cell Id” that corresponds to a fixed geographical area possible defined with vertices coordinates. This “geographical Cell Id” could be the one exchanged with 5GCN over the NG interface. In such case the NGCI would point to this “geographical Cell Id”. However, as in TN network, the Cell Id exchanged over the Uu/Xn interface is associated to a beam or a set of beams resulting onto No impact on Uu/Xn interface. How the NG-RAN can map the “Uu/Xn cell Id” with the “geographical based NG Cell Id” for coarse “cell based” location of UE ? - Such “geographical Cell Id” and its mapping with “Uu/Xn cell Id” could be either defined in RAN3 or possibly left for implementation |
| CATT | For earth-moving beams, the option a) is not feasible unless we make it mandatory that any physical NTN beam is capable to serve for multiple logical cells. The footprint of one NTN beam can still be very large compared with TN cells. No matter what method is used, the coverage of an “actual” cell (or a TA, etc) is never fixed, thus making the concept of any strictly ground-fixed cells “virtual”. |

## Impact on Xn/NG management procedures - avoid periodic re-configuration.

Solutions for fixed geographical areas for TAI/Cell ID may result in gNBs not always serving the same, constant set of TAIs/Cells at all time. In that case Xn and NG exchange of configuration information (Setup/Configuration Update) shall not reflect the momentary served set of Identities but rather the super-set of identities, avoiding periodic re-configuration. This would follow the assumption that gNBs are able to apply “autonomous” reconfiguration by means of OAM.

Please provide your view.

| Company | Comment |
| --- | --- |
| Nokia | Unclear about the impact to NGAP interface. For example, when a satellite broadcasting a specific TAC left, another satellite should serve the area with the same TAC. So from CN perspective, that TAC (area) is always supported by the gNB. For Xn, it may be beneficial to take advantage of the periodic, e.g. a cell may be associated with the timing window indicating when it is available or not available. |
| Intel | Agree about NG. As for Xn, there could be a number of solutions and e.g. Configuration Update, if implemented properly, may not be a huge issue. Furthermore, this is related to what functionalities are supported over the Xn interface to begin with (discussed separately). |
| InterDigital | Agree with NG, agree with Nokia on Xn |
| Qualcomm | We agree with the “super-set of identities” approach where each gNB is assigned a coverage area that includes fixed geographical areas for TAI/Cell IDs, but is also allowed to support UEs outside this coverage area when a radio cell or satellite happens to cover both the gNB’s own coverage area and some area outside this. This would be similar to the case where TN gNBs have overlapping cell coverage areas. We agree it would be good to avoid periodic Xn reconfigurations. |
| Thales | If Cell Id and TAI each corresponds to geographical areas defined by the NG-RAN, then this will be transparent to the 5GCN with no need of frequent Setup/Configuration Update when UE are static The NTN control center is able to predict the satellite coverage of the different satellites and therefore can make the association between the predefined geographical areas and the satellite beams foot print. It could well inform the gNBs in real time the association of beam/TAI and possibly with Cell Id. |
| CATT | Agree with NG, the “super set” should be used regardless of whether network IDs are fixed or not, as in the NG setup and Configuration update, only the supported TACs are exchanged. For the Xn, the served cell list is exchanged between NG-RAN nodes. For the earth moving cell scenario: - If “geographical fixed cell id” is also been used in NG-RAN, the “geographical fixed cell id” could be mapped to the TAC. Thus the “super set” could be used for Xn, without frequent update the TAC for the served cells. - If the “earth moving cell id” is used in NG-RAN, the associated TAC for a cell will change periodically. Whether and how to exchange such kind of info need further investigation. Xn based solution, e.g. Time window based solution a possible way, but this may also lead to frequently configuration update. It’s preferred if the OAM based solution is feasible. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-206842 Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (SA WG2) LS in

- R3-205940 NR-NTN: Network Identities Handling (Fraunhofer IIS, Fraunhofer HHI) discussion

- R3-206275 Discussion on identifier handling in NTN (CATT) discussion

- R3-206381 Network Identifier Handling for moving cells (Ericsson) discussion

- R3-206400 On Cell id in User Location Information (Intel Corporation) discussion

- R3-206261 Handling of cell identity signalling towards the CN (Qualcomm Incorporated) discussion

- R3-206262 [DRAFT] Reply LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (Qualcomm Incorporated) LS out

- R3-206263 Support of NTN RAT identification and NTN RAT restrictions (Qualcomm Incorporated) CR0490r, TS 38.413 v16.3.0, Rel-17, Cat. B

- R3-206264 Support of NTN RAT identification and NTN RAT restrictions (Qualcomm Incorporated) CR0488r, TS 38.423 v16.3.0, Rel-17, Cat. B