3GPP TSG-RAN WG3 #110-e	[draft]R3-206848

Online, 2-12 Nov 2020

Agenda Item:	9.3.2

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on the Overlapping Band Handling over F1

Document for:	Approval

# Introduction

CB: # 5_OverlappingBand_F1AP

ZTE,CATT,SS,Nok 6086:

- Extend the scope of the Selected BandCombinationIndex IE to include CA scenario

E///,Vz 6216:

- the actual band chosen by the gNB-DU is signaled over F1 by the gNB-DU to the gNB-CU.

- The servingCellMO (serving cell measurement object) chosen by the gNB-CU for a given cell remains the same, but it is associated to the band for that cell that the gNB-DU selected

- Chair: propose to agree and consolidate on principle first, before merging/revising CRs

(ZTE - moderator)

Summary of offline disc R3-206848

# For the Chairman’s Notes

Propose the following:

# Discussion

In last RAN3#109e meeting, an LS was sent to RAN2 in R3-205765, the corresponding discussion will be triggered in RAN2.

And the below minutes recorded during the meeting:

5673, 5674 are considered as BLs for this issue; if no technical concerns, agreement on both CRs is to be expected. Whether a solution is needed to be specified in order to address the mis-match between MEasConfig prepared by the gNB-CU (before the UE band was changed) and MeasConfig after the gNB-DU assigns a new band to the UE is an open issue for next meeting. To be continued on this basis...

- Two scenarios were identified in last meeting:

- Scenario1: Single connectivity, PCell supports multiple overlapping bands.

Scenario2:  In CA and DC cases,  the UE may support different band combinations with overlapping bands, and the network shall select the band combination with highest throughput to provide the best performance for the UE.

Two solutions are on the table according to the contributions for this meeting:

Option1) Extend the scope of the Selected BandCombinationIndex IE, CRs in R3-206086/87.

Option2) Introduce a new IE to indicate the selected band per cell from DU to CU, CRs in R3-206217/18.

Question 1: RAN3 should decide the solution based on the the feedback from RAN2. Anything else needs to be discussed before the reply LS received?

| Company | Comment |
| --- | --- |
| ZTE | Nothing else. Let’s wait for the feedback from RAN2. |

Another open issue raised in last meeting, whether a solution is needed to be specified in order to address the mis-match between MEasConfig prepared by the gNB-CU (before the UE band was changed) and MeasConfig after the gNB-DU assigns a new band to the UE.

Two alternatives are on the table from last meeting:

Option1) the gNB-CU triggers another round of UE Context Modification procedure

Option2) the gNB-DU updates the MeasObject and measurement gap when it switches the band before it sends the reply to the gNB-CU.

In R3-206216, it said:

The gNB-DU re-uses the servingCellMO pre-allocated by the gNB-CU and includes that servingCellMO and the chosen bands for Pscell/Scells in cellGroupConfig which is also sent in the DU to CU RRC Information IE in the UE Context Setup Response. Namely, the servingCellMO chosen by the gNB-CU for a given cell remains the same, but it is associated to the band for that cell that the gNB-DU selected.	 
If the gNB-CU has requested gNB-DU to configure measurement gaps for the indicated FR1 or FR2 frequencies, the gNB-DU designs the gap configuration in line with the ARFCN and the corresponding chosen band of serving frequencies.

It seems that Option2) is preferred and the below text was proposed in the corresponding CRs:

Information included in the CellGroupConfig IE, e.g. the MeasObjectId, associated to each cell in the Band Selection IE shall be assumed valid for the band selected by the gNB-DU for that cell.

Question 2: Option2 is selected to solve the mis-match between MEasConfig prepared by the gNB-CU (before the UE band was changed) and MeasConfig after the gNB-DU assigns a new band to the UE. Is the additional procedure text proposed above is needed ?

| Company | Comment |
| --- | --- |
| ZTE | After Q1 has been solved, the proposed additional procedure text can be discussed whether it is needed or just leave it as implementation. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-206086 Correction on Overlapping Band Handling over F1 (ZTE, CATT, Samsung, Nokia, Nokia Shanghai Bell) CR0631r2, TS 38.473 v15.11.1, Rel-15, Cat. F

- R3-206087 Correction on Overlapping Band Handling over F1 (ZTE, CATT, Samsung, Nokia, Nokia Shanghai Bell)  CR0632r3, TS 38.473 v16.3.1, Rel-16, Cat. A

- R3-206216 Overlapping bands handling (Ericsson, Verizon Wireless)

- R3-206217 Overlapping band handling CR 38.473 (Ericsson, Verizon Wireless) CR0584r2, TS 38.473 v15.11.1, Rel-15, Cat. F

- R3-206218 Overlapping band handling CR 38.473 (Ericsson, Verizon Wireless) CR0585r2, TS 38.473 v16.3.1, Rel-16, Cat. A