3GPP TSG-RAN WG3 #110-e	R3-206928

Online, 2-12 November 2020

Agenda Item:	8.1

Source:	Qualcomm (moderator)

Title:	Summary of Offline Discussion on Pending Data / ECT

Document for:	Approval

# Introduction

CB: # 74_PendingDataIndication

-  OK to work on NG HO req

- clarify why mod req, UE info transfer, conn est req?

- “shall if supported” vs. “may”?

- merge/revise if agreeable

- check details

(QC - moderator)

Summary of offline disc R3-206928

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

The CRs under discussion are in [1-4]. In the first round, it is proposed to capture views and arguments on each individual proposal / topic, and then decide how to proceed based on the outcomes.

In the following we use ECT for Extended Connected Time!

## Adding ECT to Handover Request

It seems all companies are agreed with this, and it is consistent with SA2, hence

Proposal 1: ECT is added to Handover Request

If agreed, no need to comment! :)

| Company | Comment |
| --- | --- |

## Adding ECT to Connection Establishment Indication [1]

The legacy Pending Data Indicator was not included in the CEI message, probably because this is to be used in CP-CIoT, for which other means have so far been envisaged (e.g. End Indicator). However

Please provide your view on this.

| Company | Comment |
| --- | --- |

## Adding ECT to UE Context Modification [3]

The legacy Pending Data Indicator was not included in this message, so this requires some analysis and justification (see discussion in R3-206570).

Please provide your view on this.

| Company | Comment |
| --- | --- |

## Adding ECT to UE Information Transfer [4]

Currently this message / procedure is NB-IOT specific, and the legacy Pending Data Indicator was included in this message.

Please provide your view on this.

| Company | Comment |
| --- | --- |

## Defining zero value of ECT as similar to PDI [2]

In [2] it is proposed to interpret the zero value of ECT in a similar way to PDI i.e. CN is aware of pending data, but no specific timer is provided to the RAN.

Please provide your view on this.

| Company | Comment |
| --- | --- |

## Procedural text for ECT [3,4]

In [3,4] it is proposed to change “shall if supported” to “may”. The justification in R3-206570 seems to imply that a supporting implementation may in certain cases not apply the ECT value.

Please provide your view on this.

| Company | Comment |
| --- | --- |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-206119, Inclusion of Extended Connected Time (Huawei)

- R3-206165, Correction of usage of the Extended Connected Time (Qualcomm Incorporated)

- R3-206571, Addition of Extended Connected Time to NG-AP (Ericsson)

- R3-206572, Addition of Extended Connected Time to NG-AP (Ericsson)