3GPP TSG-RAN WG3 #110-e	R3-206928

Online, 2-12 November 2020

Agenda Item:	8.1

Source:	Qualcomm (moderator)

Title:	Summary of Offline Discussion on Pending Data / ECT

Document for:	Approval

# Introduction

CB: # 74_PendingDataIndication

-  OK to work on NG HO req

- clarify why mod req, UE info transfer, conn est req?

- “shall if supported” vs. “may”?

- merge/revise if agreeable

- check details

(QC - moderator)

Summary of offline disc R3-206928

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Interim status and call for further feedback

Based on the discussion and proposed conclusions in section 4, the interim status / proposals are as follows:

Interim Agreements:

-	Add ECT to Handover Request

-	Define zero value of ECT as proposed in [2]

-	Stick with “shall, if supported” for ECT use and ensure this is consistently used

No consensus is reached on the proposals to add ECT to CEI, UE Context Modification and UE Information Transfer. Suggest to do a further check to see if based on the discussion so far, any new arguments can be seen or any companies change their views.

Final round (before considering CRs): regarding addition to these 3 messages, do the proponents have further considerations to add, and do any companies have further comments / changes of view taking into account the discussion below on each of the proposals?

| Company | Comment |
| --- | --- |

# Discussion

The CRs under discussion are in [1-4]. In the first round, it is proposed to capture views and arguments on each individual proposal / topic, and then decide how to proceed based on the outcomes.

In the following we use ECT for Extended Connected Time!

## Adding ECT to Handover Request

It seems all companies are agreed with this, and it is consistent with SA2, hence

Proposal 1: ECT is added to Handover Request

If agreed, no need to comment! :)

| Company | Comment |
| --- | --- |
| Nokia | OK |
| Huawei | ok |
| ZTE | ok |

Conclusion: Agreed to add ECT to Handover Request

## Adding ECT to Connection Establishment Indication [1]

The legacy Pending Data Indicator was not included in the CEI message, probably because this is to be used in CP-CIoT, for which other means have so far been envisaged (e.g. End Indicator). However

Please provide your view on this.

| Company | Comment |
| --- | --- |
| Nokia | Was not in SA2 LS. |
| Huawei | It is needed, used for the following two cases: - Initial UE associated NGAP connection establish case, inform RAN to not release the UE immediately - CP relocation, to inform the new RAN to not release the UE immediately. |
| Ericsson | Was not in EPS. This puts a new requirement to RAN and there can be an overlap with the End indication IE. |
| ZTE | Since End Indication IE has already been included in the Connection Establishment Indication message in NGAP, ECT is not necessary, which is aligned with S1AP message. |
| Qualcomm | Would be ok with adding. We acknowledge the comments from Nokia, Ericsson and ZTE, but we were thinking that the End Indication is primarily aimed at MT-EDT/CP-CIoT to aid decision about releasing vs doing RRC setup, but there could be use cases where the CN wants to send a time indication beyond simply “don’t release”. |

Conclusion: No consensus. Can further check for company’s final views based on discussion so far.

## Adding ECT to UE Context Modification [3]

The legacy Pending Data Indicator was not included in this message, so this requires some analysis and justification (see discussion in R3-206570).

Please provide your view on this.

| Company | Comment |
| --- | --- |
| Nokia | No. Don’t see the need. |
| Huawei | No |
| Ericsson | Question: if the AMF realizes that there is more pending data arriving at the downlink, but the value of the ECT timer is going to expire soon, how can the AMF update the value of the ECT to prevent the RAN from releasing the UE early? |
| ZTE | Yes. We agree the intention to indicate ng-eNB to extend the connection duration. |
| Qualcomm | Tend to no. This seems like an optimization which may be better to discuss in SA2. Prefer to ensure we have the basic functionality (i.e. related to initial message exchange after state transition). |

Conclusion: No consensus. Can further check for company’s final views based on discussion so far.

## Adding ECT to UE Information Transfer [4]

Currently this message / procedure is NB-IOT specific, and the legacy Pending Data Indicator was included in this message.

Please provide your view on this.

| Company | Comment |
| --- | --- |
| Nokia | Was not in SA2 LS |
| Huawei | No need, as the UE information Transfer procedure is used before NGAP UE association establishment. |
| Ericsson | Yes, to align with EPS |
| ZTE | Yes. Similar with that the Pending Data Indication IE can be contained in the UE INFORMATION TRANSFER message in S1AP, the ECT should also can be contained in the UE INFORMATION TRANSFER message in NGAP to indicate the DL data to be transmitted. |
| Qualcomm | Tend to no. We looked at this internally a while back and saw no strong reason even in EPS, since there will be an opportunity for the CN to send a value if needed when the association is established. |

Conclusion: No consensus. Can further check for company’s final views based on discussion so far.

## Defining zero value of ECT as similar to PDI [2]

In [2] it is proposed to interpret the zero value of ECT in a similar way to PDI i.e. CN is aware of pending data, but no specific timer is provided to the RAN.

Please provide your view on this.

| Company | Comment |
| --- | --- |
| Nokia | OK otherwise value 0 could be interpreted as immediate release. |
| Huawei | ok |
| Ericsson | Ok |
| ZTE | Ok |
| Qualcomm | ok |

Conclusion: There is a consensus to define zero value of ECT (in a similar way to PDI).

## Procedural text for ECT [3,4]

In [3,4] it is proposed to change “shall if supported” to “may”. The justification in R3-206570 seems to imply that a supporting implementation may in certain cases not apply the ECT value.

Please provide your view on this.

| Company | Comment |
| --- | --- |
| Nokia | “Shall if supported” seems good enough. It doesn’t force implementations to support it. |
| Huawei | Shall, if supported |
| Ericsson | Prefer “may”. Otherwise, “shall, if supported” should be aligned everywhere. |
| ZTE | Prefer “Shall if supported”, e.g. eNB should mandatorily comply the ECT, if supported. |
| Qualcomm | Prefer “Shall if supported” – agree with Ericsson that we should be consistent and check for all uses of the IE. |

Conclusion: Strong support to keep “shall, if supported”, however final CR should ensure that this is consistently used.

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-206119, Inclusion of Extended Connected Time (Huawei)

- R3-206165, Correction of usage of the Extended Connected Time (Qualcomm Incorporated)

- R3-206571, Addition of Extended Connected Time to NG-AP (Ericsson)

- R3-206572, Addition of Extended Connected Time to NG-AP (Ericsson)