3GPP TSG-RAN WG3 Meeting #110-e	R3-206969

E-meeting, 2 - 12 Nov 2020

Agenda Item:	9.3.7.1

Source:	Huawei (moderator)

Title:	Summary of Offline Discussion on SecondaryNG-RANnodeID

Document for:	Approval

# Introduction

This contribution is to kick off the following discussion.

CB: # 96_SecondaryNG-RANnodeID

-  clarify usage: is this needed?

(HW - moderator)

Summary of offline disc R3-206969

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Background

In TS 37.340, it is already captured the 5GC initiated PDU session resource modify procedure as as follows. Then in this case, the MN can decide to perform SN addition procedure in order for the MN to split the PDU session.

### In TS 37.340 ### 10.14.2 PDU Session Split at UPF during PDU session resource modify (5GC initiated) The 5GC may provide an additional UL TEID address during PDU Session Resource Modify in order to allow the MN to split the PDU session. The MN may perform the SN Addition or the MN-initiated SN Modification procedure. If the MN decides to split the PDU session, the MN provides a DL TEID address to be applied as the additional DL tunnel address and the QoS flows associated with that tunnel. Figure 10.14.2-1: PDU Session Split at UPF during PDU session resource modify

This also means that, the MN can offload the whole PDU session to the SN during the 5GC initiated PDU session resource modify procedure, which is at least supported by our specification.

Observation: the MN may offload the whole PDU session to the SN during the 5GC initiated PDU session resource modify procedure, which is at least supported by our specification.

If companies have different understanding from the above observation, please provide comments.

| Company | Comment |
| --- | --- |

# Discussion

In [1], it is proposed that:

in the PDU SESSION RESOURCE MODIFY RESPONSE message:

-	add the Global RAN Node ID of Secondary NG-RAN Node in PDU Session Resource Modify Response Transfer IE.

Based on the online discussion, the use case should be first clarified.

- Use case 1: SN addition during 5GC initiated PDU session resource modify procedure

  - The UE is served by only one NG-RAN node when the Ethernet PDU session#1 is setup with RSN value#1 and Ethernet PDU session#2 is setup with RSN value#2, when the UE is not in coverage of the NG-RAN node.

  - The 5GC initiates the PDU session resource modify procedure, e.g. requesting to setup a few new QoS flows for PDU session#1 and session#2.

  - The NG-RAN node decides to perform SN Addition, and offload the whole PDU session#2 to the SN, when the UE moves within the SN coverage.

  - The NG-RAN responds the Global RAN Node ID of Secondary NG-RAN Node to the 5GC for PDU session #2.

- Use case 2: SN change during 5GC initiated PDU session resource modify procedure

  - The UE is in MR-DC mode, with the Ethernet PDU session#1 with RSN value#1 is served by the MN, and Ethernet PDU session#2 with RSN value#2 is served by the SN.

  - The 5GC initiates the PDU session resource modify procedure, e.g. requesting to setup a few new QoS flows either for PDU session#1 and session#2.

  - The NG-RAN node decides to perform SN change, and offload the whole PDU session#2 to the new SN#2.

  - The NG-RAN responds the Global RAN Node ID of the new Secondary NG-RAN Node to the 5GC for PDU session #2.

Question 1: do you agree the above use case is valid? Or, please find more comments.

| Company | Yes/No | Comment |
| --- | --- | --- |
| Huawei | Yes |  |
| Ericsson | No | The “SecondaryNG-RANnodeID” is introduced for NR-IIoT for Solution 1 of the KI#1. During PDU session setup, the DC shall be used and if the condition meets, the “SecondaryNG-RANnodeID” is communicated to 5GC. There is no SN additional case or SN change case by 5GC for this. Mobility procedures (NG/XN HO) already support it. |

<Second-round discussion>

Question 2: Do you agree the update proposed in [1]? or any further update is needed?

| Company | Yes/No | Comment |
| --- | --- | --- |

# Conclusion

If needed

# References

- R3-206719 Introducing the secondary NG-RAN node ID in PDU Session Resource Modify Response message, Huawei, CATT, ZTE, Intel Corporation.