3GPP TSG-RAN WG3 #110-e							 R3-206899

2-12 November 2020

Online

Agenda Item:	15.2

Source:	ZTE (moderator)

Title:	Summary of Offline Discussion on NRQoE5-Signalling_flow

Document for:	Approval

# Introduction

CB: # NRQoE5-Signalling_flow

- Reuse Trace mechanism for NR QoE is preferred, enhancements can refer MDT solution in NR?

- Management-based and signalling-based QoE signalling flow?

- The RAN node should know the service type related to QoE measurement configuration and QoE reporting which it performs?

- Send an LS to SA5 (cc RAN2) relative to framework used for LTE QMC?

- Send an LS to SA4 (cc RAN2) to inquire about their plans concerning application client requirements for NR services with respect to mandatory vs. optional support of OMA-DM and QMC?

- Capture agreements as TP for TR

- Send LSout to other groups, if needed

(ZTE- moderator)

Summary of offline disc

Note: The first round email discussion plan to be end before the end of first week (Friday 18:00 UTC 2020-11-6).

# For the Chairman’s Notes

Propose the following:

Propose to capture the following:

# Discussion

## Signalling procedure

Last RAN3 meeting achieved agreement on solutions.

Both the management-based and signalling-based solutions for NR QoE management are supported.

While the correspond signalling is missing. In [2][6][8][16][23], companies share the view that MDT-based signalling can be baseline for NR QMC. Based on MDT/Trace, in this meeting one company provides detail signalling in [3] for TP.

It is noting mobility and RAN triggered aspects covered by other CBs, in this CB only cover general activation and deactivation procedure.

### Management-based signalling

#### Management-based activation

One company provides detail signalling in [3] for TP. The detail description is copied below:

Management-based activation The procedure is used for activating the QoE measurement triggered by OAM shown in figure 1. The OAM sends the QoE measurement configuration to gNB. gNB finds the qualified UEs that meet the criteria (e.g. application layer capability, service type, etc.). gNB sends the QoE measurement configuration to the UE. When a session starts, the application layer in UE checks the criteria (e.g. cell list, service type, etc.), if meets, start QoE measurement and reporting.

Proposal : Can we agree to capture into TR38.890 the Management-based activation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | 1:The step 0 is not necessary and should be removed. 2: Replace “gNB” with “NG-RAN node”. |

#### Management-based deactivation

One company provides detail signalling in [3] for TP. The detail description is copied below:

The procedure is used for deactivating the QoE measurement triggered by OAM shown in figure. The OAM sends the deactivation indication to gNB to indicate which QoE measurement should be deactivated, then gNB sends it to UE, and UE stops corresponding measurement and reporting.

Proposal : Can we agree to capture into TR38.890 the Management-based deactivation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | Replace “gNB” with “NG-RAN node”. |

### Signalling-based signalling

#### Signalling-based activation

One company provides detail signalling in [3] for TP. The detail description is copied below:

The procedure is used for activating the QoE measurement triggered by CN shown in figure. The CN sends the QoE measurement configuration to gNB. gNB checks the criteria (e.g. application layer measurement capability, service type, etc.) for the UE. gNB sends the QoE measurement configuration to the UE. When a session starts, the application layer in UE checks the criteria (e.g. cell list, service type, etc.), if meets, start QoE measurement and reporting.

Proposal : Can we agree to capture into TR38.890 the Signalling-based activation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | 1:The step 0 is not necessary and should be removed. 2: Replace “gNB” with “NG-RAN node”. |

#### signalling-based deactivation

One company provides detail signalling in [3] for TP. The detail description is copied below:

The procedure is used for deactivating the QoE measurement triggered by CN . There are two options to trigger deactivation in this case. Option 1a is that the CN sends the deactivation indication to gNB to indicate which QoE measurement should be deactivated, then gNB sends it to UE, and UE stops corresponding measurement and reporting. Option 1b is that the CN sends UE PDU session or context related message to gNB. gNB finds that the criteria are no longer met for a specific QoE measurement after receiving the message. gNB sends the deactivation indication to indicate which QoE measurement should be deactivated, then UE stops corresponding measurement and reporting.

Proposal : Can we agree to capture into TR38.890 the Signalling-based deactivation signalling in [3] ?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes with modification | 1: Replace “gNB” with “NG-RAN node”. 2: There is no description in LTE QoE that RAN need to consider PDU session/ UE context modification when deactive the QoE task. In order to take LTE solution as baseline, it is proposed to remove option 1b from the figure and description part. |

## Send an LS to SA5 (cc RAN2) relative to framework used for LTE QMC ?

In [8], the company observed that SA5 has developed LTE QoE measurements in TS 28.405 without dependency on MDT/Trace. The company observed that Coexistence between NR QMC and LTE QMC will have to be ensured independently of the framework chosen for NR QMC. Therefore, the LTE QMC framework needs to be clarified as part of the Rel-17 NR QoE. A draft LS is prepared for this purpose.

Proposal : Whether the draft LS in [9] is needed?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Yes |  |

## Send an LS to SA4 (cc RAN2) to inquire about their plans concerning application client requirements for NR services with respect to mandatory vs. optional support of OMA-DM and QMC

Based on observation from [8], the contribution raised a concern on CP based MDT mechanism due to OAM-DM mechanism defined in SA4.

However, SA4 specification mandates application clients in the UEs, or at least the MTSI client, to support QoE report configuration and reporting based on OMA-DM. OMA-DM is based on user plane transport and QoE reports based on such transport will not be visible at the RAN. OMA-DM enables activation of QoE reports for individual UEs (similar to MDT subscriber-based activation) but would not to our knowledge be suitable for area-based activation

Therefore , a draft LS can be found in [8] to ask SA4 provide clarification.

RAN3 would like to inform SA4 about discussions relative to visibility of QoE reports in the RAN and potential access stratum level actions relying on these QoE reports. RAN3 also observes that TS 26.114 specifies that an MTSI client that supports the QoE metrics feature shall support OMA-DM for activation/deactivation and gathering of QoE metrics in the MTSI client. The OMA-DM mechanism relies on user plane, and corresponding reports are therefore not visible in the RAN. LTE QMC functionality, based on control plane, is optional in the MTSI client. The UE provides its LTE QMC capability to the E-UTRAN, and the E-UTRAN further forwards this capability information to the EPC. RAN3 would therefore like to ask SA4 about current and planned status concerning application client requirements for NR services with respect to mandatory vs. optional support of QoE activation/deactivation and reporting based on OMA-DM and QMC.

Proposal X: Whether the draft LS in [8] to SA4 is needed?

| Company | Yes/NO | Comment |
| --- | --- | --- |
| ZTE | Probably not. | Pending to the discussion in CB#2. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-206491 time plan update for NR QoE SI (China Unicom)

- R3-206036 Discussion on NR QoE solutions (Samsung)

- R3-206037 pCR for TR 38.890 NR QoE solutions, procedures and interface impacts (Samsung)

- R3-206173 QoE metrics correlation with radio (Qualcomm Incorporated)

- R3-206174 QoE aware by gNB (Qualcomm Incorporated)

- R3-206360 Discussion on NR QoE solutions (CATT)

- R3-206361 Discussion on RAN involved NR QoE (CATT)

- R3-206369 Open points on QMC framework (Nokia, Nokia Shanghai Bell)

- R3-206370 [Draft] LS on Framework for QoE Measurement Collection (Nokia, Nokia Shanghai Bell)

- R3-206397 pCR for TR 38.890 Radio-related Measurements and Information in NR QoE Management (Ericsson)

- R3-206398 pCR for TR 38.890 Mobility Support for NR QoE Management (Ericsson)

- R3-206399 pCR for TR 38.890 Way Forward on Remaining Issues in NR QoE SI (Ericsson)

- R3-206492 Further discussion on NR QoE solution (China Unicom)

- R3-206493 Discussion on requirements and mechanisms for per slice QoE measurement (China Unicom, ZTE)

- R3-206535 Further Consideration on QOE Configuration and Report (China Telecommunications)

- R3-206713 Further Consideration On Study of NR QoE (ZTE)

- R3-206714 TP for TR 38890 (ZTE)

- R3-206732 Discussions on QoE report visibility at the RAN (Huawei)

- R3-206733 Discussions on the radio related measurements and information as assistance to the NR QoE management functionality (Huawei, China Unicom, China Mobile)

- R3-206734 Discussions on potential RAN3 impacts about the QoE measurement configuration, reporting and releasing under SA, NSA and MR-DC operation (Huawei, China Unicom)

- R3-206787 Further discussion on NR QoE management (CMCC)

- R3-206795 TP to 38.890 for NR QoE management (CMCC)

- R3-206834 Discussion on signaling-based NR QoE management (LG Electronics)

- R3-206175 Interworking with LTE QoE (Qualcomm Incorporated)

- R3-206715 Consideration on slice QoE measurement (ZTE)