3GPP TSG-RAN WG3 #111-e	draft-R3-210961

Online, 25th January -5th February 2021

Agenda Item:	9.3.3

Source:	Ericsson (moderator)

Title:	Summary of Offline Discussion on CB: # 8_ASrekey_EmergencyFallback

Document for:	Approval

# Introduction

CB: # 8_ASrekey_EmergencyFallback

Nok

specify abnormal condition for NG-RAN node to fail the procedure with UE Context Modification Failure

HW

leave the NG-RAN node handling for such rare and abnormal case to implementation

E///,ZTE

NG-RAN node shall only handle the Emergency Fallback in case it is requested together with AS Re-keying in the same UE context modification message; liaise back SA2

CATT

In case of collision between emergency fallback and AS re-keying, NG-RAN should succeed the UE Context Modification procedure, and indicate AMF AS re-keying is failed due to the collision of the procedures.

An indication should be introduced in the UE CONTEXT MODIFICATION RESPONSE message to indicate the failure of AS re-keying procedure due to the collision.

- consensus for “light” approach / leaving to implementation

- check details

(E/// - moderator)

Summary of offline disc R3-210961

# For the Chairman’s Notes

RAN3 has agreed the blow:

- the new AMF implements the SA2 CR shall not send two requests in the same message.

- the old AMF may send two requests in one message (even it is not often)

- the old NG-RAN node, according to current NGAP specification, would proceed both requests.  NG-RAN node will send back in successful case UE CONTEXT MODIFICATION RESPONSE, in the failure case UE CONTEXT MODIFICATION FAILURE.

A compromised solution is to add “A Note” in the abnormal case in Chapter 8.3.4.4.

Note: When the AS re-keying request and the Emergency Fallback request are included in the same message, the NG-RAN node may only handle the Emergency Fallback request.

Agree CR R3-211171 (revised from R3-210328) and R3-211172 (revised from R3-210329)

# Discussion

Among the interested companies’ contributions, the sets of solutions proposed to handle when AS rekeying and Emergency Fallback are requested in the same UE context Modification message are:

- to specify abnormal condition for NG-RAN node to fail the procedure with UE Context Modification Failure (Nokia in [1], [2], [3])

- to leave the NG-RAN node handling for such rare and abnormal case to implementation (Huawei in [4], [5], [6])

- to specify that NG-RAN should ignore the AS Rekeying and proceed with Emergency fallback (Ericsson, ZTE in [7], [8], [9]

- NG-RAN should succeed the UE Context Modification procedure and indicate AMF AS re-keying is failed due to the collision of the procedures. (CATT in [11], [12], [13])

A LS is also proposed to send back to SA2 to make a closure of the discuss related to the AS rekeying and Emergency fall back in [10].

As a first step, we discuss the four cases when old and new AMF and NG-RAN nodes are involved.

| Case | Configuration |
| --- | --- |
| Case 1 | New AMF, new NG-RAN node |
| Case 2 | New AMF, old NG-RAN node |
| Case 3 | Old AMF, old NG-RAN node |
| Case 4 | Old AMF, new NG-RAN node |

In below, two requests are “AS Rekeying request” and “Emergency Fallback” request.

New AMF:  means the AMF implemented the SA2 solution.

New NG-RAN node: means the NG-RAN node having the solution that we are discussing in this CB.

Question 1: for Case 1 and Case 2, the new AMF should make sure the two requests are not sent in the same UE context modification message. There should be no issue, do you agree?

| Company | Comment |
| --- | --- |
| Ericsson | Yes |
| Nokia | Yes |
| ZTE | Yes |
| Huawei | Yes |
| CATT | Yes |

Question 2: Case 3 is legacy. It belongs to a normal operation that the two requests are sent in the same UE context modification message. Do you agree?

| Company | Comment |
| --- | --- |
| Ericsson | Yes The NGAP protocol allows the two requests to be sent in the same message and specify the handling that NG-RAN node should handle the two requests. |
| Nokia | The standards does not prevent it but it could be seen as a bad implementation, considering the clarification from SA2. |
| ZTE | Yes, Agree with E/// |
| Huawei | Very rare, considering the clarification from SA2. |
| CATT | For the old AMF, it may be possible, although it’s a kind of bad implementation. |

Question 2b: What would an old NR-RAN node do if the two requests are sent in the same message, according to the current NGAP specification?

| Company | Comment |
| --- | --- |
| Ericsson | The NG-RAN node would proceed both requests and in successful case, sends back UE CONTEXT MODIFICATION RESPONSE. |
| Huawei | The NG-RAN node would proceed both requests and in successful case, sends back UE Context Modification Response, or the NG-RAN node would fail both requests by sending back failure message, both may happen, up to the RAN node implementation. |
| CATT | Share the view with HW. |

Question 3: Case 4 is when the old AMF sends two requests in the same UE context modification message, what should a new NG-RAN node do? We need to find solution for this case, do you agree?

| Company | Comments |
| --- | --- |
| Ericsson | Yes. We think this is the case that we need a solution. And in our view, the solution should not differ so much with the legacy Case 3. We propose that the new NG-RAN node is allowed to ignore the AS rekeying request and only perform Emergency Fallback. Note that with this solution: - the new NG-RAN node is still be able to perform as the old NG-RAN node to handle the two requests. - The new NG-RAN node can ignore the AS Rekeying and only perform emergency fallback |
| Nokia | Disagree. For case 2, 3 the NG-RAN node was not specified. This should remain the same. No CR at all might be the best way forward. |
| ZTE | Yes, Agree with E/// |
| Huawei | As we mentioned in our paper, all three options are workable and feasible. i.e. fail the procedure, only perform AS-rekey, perform both AS-rekey and emergency fallback. In order to avoid IoT issue in the future, we propose to capture in the specification that it is up to RAN implementation. |
| CATT | For case 4, it’s better to have a definite solution to handle such kind of abnormal case. As SA2 has provided the guidance on handling of such kind of collision, it seems good for a new NG-RAN node to follow the principle of SA2, i.e. the new NG-RAN node could ignore the AS rekeying request and only perform Emergency Fallback in case of the collision. But in this case, we assume an indication is beneficial to inform AMF the AS rekeying is failed due to the collision, as proposed in [11]~[13]. |

# Conclusion, Recommendations

RAN3 has agreed the blow:

- the new AMF implements the SA2 CR shall not send two requests in the same message.

- the old AMF may send two requests in one message (even it is not often)

- the old NG-RAN node, according to current NGAP specification, would proceed both requests.  NG-RAN node will send back in successful case UE CONTEXT MODIFICATION RESPONSE, in the failure case UE CONTEXT MODIFICATION FAILURE.

The discussed solutions for the new NG-RAN node are:

- Three Companies: Specify that NG-RAN node may ignore the AS rekeying and proceed with Emergency Fallback only; One company further propose an indication to AMF when AS Rekeying is not performed

- One Company: Nothing is needed.

- One Company: specify that “it is up to implementation”.

SA2 Agreements: - In S2-2005407, it is specified that “When the AS re-keying procedure and the Emergency Fallback procedure collides, the AMF gives up the AS re-keying procedure and only initiates the Emergency Fallback procedure.” - In S2-2005909 LS reply to RAN3, SA2 states that: If AMF includes both the Security Key IE and the Emergency Fallback Indicator IE within one NGAP UE CONTEXT MODIFICATION REQUEST message, the NG-RAN takes it as the abnormal case.

Three companies believe that RAN3 should follow the SA2 principle to specify the handling in the abnormal case.

Proposed compromise solution:

Included only a Note in the Abnormal case in Chapter 8.3.4.4 aligning to the SA2 agreement:

Note: When the AS re-keying request and the Emergency Fallback request are included in the same message, the NG-RAN node may only handle the Emergency Fallback request.

R3-210328/ R3-210329 are revised in R3-21171/R3-21172

# References

- R3-210046	Correction of AS Rekeying and Emergency Fallback (Nokia, Nokia Shanghai Bell)	discussion

- R3-210047	Correction of AS Rekeying and Emergency Fallback (Nokia, Nokia Shanghai Bell)	CR0531r, TS 38.413 v15.10.0, Rel-15, Cat. F

- R3-210048	Correction of AS Rekeying and Emergency Fallback (Nokia, Nokia Shanghai Bell)	CR0532r, TS 38.413 v16.4.0, Rel-16, Cat. A

- R3-210128	Consideration on AS-rekey and Emergency fallback (Huawei)	discussion

- R3-210129	Correction on AS-rekey and Emergency fallback (Huawei)	CR0539r, TS 38.413 v15.10.0, Rel-15, Cat. F

- R3-210130	Correction on AS-rekey and Emergency fallback (Huawei)	CR0540r, TS 38.413 v16.4.0, Rel-16, Cat. A

- R3-210327	AS re-keying and Emergency Fallback in the UE Context Modification procedure (Ericsson)	discussion

- R3-210328	Clarification of AS re-keying in the UE Context Modification procedure (Ericsson, ZTE)	CR0354r4, TS 38.413 v15.10.0, Rel-15, Cat. F

- R3-210329	Clarification of AS re-keying in the UE Context Modification procedure (Ericsson, ZTE)	CR0355r4, TS 38.413 v16.4.0, Rel-16, Cat. A

- R3-210330	Draft] Reply LS on AS rekeying handling (Ericsson)	LS out

- R3-210466	Discussion on AS rekey and Emergency fallback (CATT)	discussion

- R3-210467	Correction on collision of AS-rekey and Emergency fallback_NGAP Rel-15 (CATT)	CR0551r, TS 38.413 v15.10.0, Rel-15, Cat. F

- R3-210468	Correction on collision of AS-rekey and Emergency fallback_NGAP Rel-16 (CATT)	CR0552r, TS 38.413 v16.4.0, Rel-16, Cat. A