3GPP TSG-RAN WG3 Meeting #112-e	R3-211530

E-meeting, 17-28 May 2021

Title:	[DRAFT] Reply LS on signalling SN initiated release of SCG

Response to:	LS R2-2102493 on signalling SN initiated release of SCG from RAN2

Release:	Release-15

Work Item:	NR_newRAT-Core

Source:	Huawei [will be 3GPP RAN3]

To:	3GPP RAN2

Cc:

Contact person:	Yan Wang

wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 thanks RAN2 on the LS on signalling SN initiated release of SCG.

RAN3 discussed the case in which SN wants to initiate release of the SCG resources while DRBs may still remain SN terminated (i.e., an SCG release request initiated by the SN), and would like to provide the following feedback:

RAN2’s Question: Whether, in current X2/Xn signaling, it is supported for the SN to indicate such SCG release request to the MN.

RAN3’s Answer:

The SN can inform the MN about SCG Release via the following information:

- X2AP: SgNB to MeNB Container IE (e.g. absence of pSCellFrequency in CG-Config), the EN-DC Resource Configuration IE and maybe also the SgNB Resource Coordination Information IE in SGNB MODIFICATION REQUIRED message.

- XnAP: S-NG-RAN node to M-NG-RAN node Container IE (e.g. absence of pSCellFrequency and pSCellFrequencyEUTRA in CG-Config), and maybe also the MR-DC Resource Coordination Information IE in S-NODE MODIFICATION REQUIRED message.

2	Actions

To 3GPP RAN2

ACTION: 	RAN3 respectfully asks RAN2 to take above feedback into account in their discussion.

3	Dates of next RAN3 meetings

RAN3#113	2021-08-23 - 2021-08-27 (may be updated)

RAN3#113-bis	2021-10-11 -  2021-10-15 (may be updated)