3GPP TSG-RAN WG3 Meeting #112-e	R3-211594

# Online, 17 May – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 0741 | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE information delivery in F1AP PAGING |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Qualcomm Incorporated, Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The DRX cycle (T) used for the dertermination of Paging subframe may be different from that used for the dertermination of Paging radio frame for UE in RRC_INACTIVE. E.g. The DRX cycle (T) used for the dertermination of Paging subframe is debermined by the shortest of the UE specific DRX, if allocated by upper layers and the default paging cycle. The DRX cycle (T) used for the dertermination of Paging radio frame is debermined by the shortest of RAN Paging DRX, UE Specific DRX, if allocated by upper layers and the default paging cycle. - Thus, - gNB-DU should know the UE Specific DRX. 2. For compatibility, gNB-DU should also know the UE Radio Capability for Paging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Include UE Specific DRX IE in the PAGING message. - Include UE Radio Capability for Paging of NR IE in the PAGING message. Impact Analysis Impacted functionality: This CR has an isolated impact towards the previous version of the specification (same release). This CR only has an impact on the Paging function for UE in RRC_INACTIVE. . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Possible paging loss for UE in RRC_INACTIVE if the RAN paging cycle is smaller than the UE specific DRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7, 9.2.6.1, 9.3.1.xy(new), ASN.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the first change

## 8.7	Paging procedures

### 8.7.1	Paging

#### 8.7.1.1	General

The purpose of the Paging procedure is used to provide the paging information to enable the gNB-DU to page a UE. The procedure uses non-UE associated signalling.

#### 8.7.1.2	Successful Operation

Figure 8.7.1.2-1: Paging procedure. Successful operation.

The gNB-CU initiates the procedure by sending a PAGING message.

The Paging DRX IE may be included in the PAGING message, and if present the gNB-DU may use it to determine the final paging cycle for the UE.

The Paging Priority IE may be included in the PAGING message, and if present the gNB-DU may use it according to TS 23.501 [21].

At the reception of the PAGING message, the gNB-DU shall perform paging of the UE in cells which belong to cells as indicated in the Paging Cell List IE.

The Paging Origin IE may be included in the PAGING message, and if present the gNB-DU shall transfer it to the UE.

The UE Specific DRX IE may be included in the PAGING message, and if present the gNB-DU shall use it according to TS 38.304 [24].

The UE Radio Capability for Paging of NR IE may be included in the PAGING message, and if present the gNB-DU shall use it according to TS 38.304 [24].

#### 8.7.1.3	Abnormal Conditions

Not applicable.

Next change

#### 9.2.6.1	PAGING

This message is sent by the gNB-CU and is used to request the gNB-DU to page UEs.

Direction: gNB-CU  gNB-DU

| IE/Group Name |  | Presence |  | Range |  | IE type and reference |  | Semantics description |  | Criticality |  | Assigned Criticality |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Type |  | M |  |  |  | 9.3.1.1 |  |  |  | YES |  | ignore |  |
| UE Identity Index value |  | M |  |  |  | 9.3.1.39 |  |  |  | YES |  | reject |  |
| CHOICE Paging Identity |  | M |  |  |  |  |  |  |  | YES |  | reject |  |
| >RAN UE Paging identity |  | M |  |  |  | 9.3.1.43 |  |  |  | - |  |  |  |
| >CN UE paging identity |  | M |  |  |  | 9.3.1.44 |  |  |  | - |  |  |  |
| Paging DRX |  | O |  |  |  | 9.3.1.40 |  | It is defined as the minimum between the RAN UE Paging DRX and CN UE Paging DRX |  | YES |  | ignore |  |
| Paging Priority |  | O |  |  |  | 9.3.1.41 |  |  |  | YES |  | ignore |  |
| Paging Cell List |  |  |  | 1 |  |  |  |  |  | YES |  | ignore |  |
| >Paging Cell Item IEs |  |  |  | 1 .. <maxnoofPagingCells> |  |  |  |  |  | EACH |  | ignore |  |
| >>NR CGI |  | M |  |  |  | 9.3.1.12 |  |  |  | - |  |  |  |
| Paging Origin |  | O |  |  |  | 9.3.1.79 |  |  |  | YES |  | ignore |  |  |
| UE Specific DRX |  | O |  |  |  | 9.3.1.40 |  | Includes the UE specific DRX as received in the Core Network Assistance Information IE in TS 38.413 [3]. |  | YES |  | ignore |  |  |
| UE Radio Capability for Paging of NR | O |  | 9.3.1.xy |  | YES | ignore |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPagingCells | Maximum no. of paging cells, the maximum value is 512. |

Next change

9.3.1.xy	UE Radio Capability for Paging of NR

This IE contains paging specific UE Radio Capability information.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| UE Radio Capability for Paging of NR | O |  | OCTET STRING | Includes the RRC UERadioPagingInformation message as defined in TS 38.331 [18]. |

Next change

9.4.4	PDU Definitions

-- ASN1START

-- **************************************************************

--

-- PDU definitions for XnAP.

--

-- **************************************************************

//SKIP THE UNRELATED PART//

NotificationInformation,

UESpecificDRX,

UERadioCapabilityForPaging,

PagingPriority,

//SKIP THE UNRELATED PART//

id-NotificationInformation,

id-UESpecificDRX,

id-UERadioCapabilityForPagingOfNR,

maxCellingNBDU,

maxnoofCandidateSpCells,

//SKIP THE UNRELATED PART//

Next change

-- **************************************************************

--

##### -- Paging

--

-- **************************************************************

Paging ::= SEQUENCE {

protocolIEs			ProtocolIE-Container       {{ PagingIEs}},

...

}

PagingIEs F1AP-PROTOCOL-IES ::= {

{ ID id-UEIdentityIndexValue	CRITICALITY reject	TYPE UEIdentityIndexValue		PRESENCE mandatory	}|

{ ID id-PagingIdentity			CRITICALITY reject	TYPE PagingIdentity				PRESENCE mandatory	}|

{ ID id-PagingDRX				CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-PagingPriority			CRITICALITY ignore	TYPE PagingPriority				PRESENCE optional	}|

{ ID id-PagingCell-List			CRITICALITY ignore	TYPE PagingCell-list			PRESENCE mandatory	}|

{ ID id-PagingOrigin			CRITICALITY ignore	TYPE PagingOrigin				PRESENCE optional	}|

{ ID id-UESpecificDRX			CRITICALITY ignore	TYPE PagingDRX					PRESENCE optional	}|

{ ID id-UERadioCapabilityForPagingOfNR			CRITICALITY ignore	TYPE UERadioCapabilityForPagingOfNR				PRESENCE optional	},

...

}

PagingCell-list::= SEQUENCE (SIZE(1.. maxnoofPagingCells)) OF ProtocolIE-SingleContainer { { PagingCell-ItemIEs } }

PagingCell-ItemIEs F1AP-PROTOCOL-IES ::= {

{ ID id-PagingCell-Item		CRITICALITY ignore	TYPE PagingCell-Item			PRESENCE mandatory}	,

...

}

//SKIP THE UNRELATED PART//

Next change

### 9.4.5	Information Element definitions

//SKIP THE UNRELATED PART//

UEIdentityIndexValueChoice-ExtIEs F1AP-PROTOCOL-IES ::= {

...

}

UERadioCapabilityForPagingOfNR ::= OCTET STRING

ULConfiguration ::= SEQUENCE	{

uLUEConfiguration		ULUEConfiguration,

iE-Extensions	ProtocolExtensionContainer { { ULConfigurationExtIEs } }	OPTIONAL,

...

}

//SKIP THE UNRELATED PART//

Next change

9.4.7	Constant Definitions

//SKIP THE UNRELATED PART//

id-ConfiguredTACIndication							ProtocolIE-ID ::= 425

id-UESpecificDRX									ProtocolIE-ID ::= 43x

id-UERadioCapabilityForPagingOfNR					ProtocolIE-ID ::= 43y

END

-- ASN1STOP

End of the change