3GPP TSG-RAN WG3 Meeting #112-e	R3-211924

# E-meeting, 17-28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.413 | CR | 0376 | rev | 4 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NAS Non-Delivery |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In case of the PDU session release which includes a NAS PDU for a UE in RRC_INACTIVE state but not reachable, it is not clear whether the NG-RAN should send the NAS NON DELIVERY INDICATION to the AMF. The LS reply from SA2 has provided (see R3-204126) . For the 2nd question, SA2 think that for the non-PDU session related NAS-PDU in the "Initial Context Setup Request" & "PDU Session Resource Setup", the NAS non-delivery may be needed. This is due to the fact there is no response message to indicate the cause for these UE level NAS-PDU. Also an example for selective activation of UP connection is provided in the PDU Session Resource Setup but failed to be delivered to the UE. In this case there is no clear indication (e.g. cause value) to respond the transmission status. Hence, there is a need to Report the NAS NON DELIVERY INDICATION for the UE level NAS-PDU in the "Initial Context Setup Request" & "PDU Session Resource Setup" in case of inactive UE unreachable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the NAS NON DELIEVERY INDICATION to be also used for the UE level NAS-PDU in the "Initial Context Setup Request" & "PDU Session Resource Setup". Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release) because it corrects the NGAP on the NAS non-delivery . The impact can be considered isolated because the change only affects the NAS non-delivery function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The unsuccessful UE level NAS-PDU delivery can not be notified to the CN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1, 8.3.1.3, 9.2.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: R3-203642 Rev2: R3-204990 Update based on the SA2 LS reply R3-204239 Rev3: R3-206441 Submitted to RAN3-110-e meeting Rev4: R3-211924 Update based on the SA2 LS reply |  |  |  |  |  |  |  |  |

Change Begins

8.2.1	PDU Session Resource Setup

8.2.1.1	General

The purpose of the PDU Session Resource Setup procedure is to assign resources on Uu and NG-U for one or several PDU sessions and the corresponding QoS flows, and to setup corresponding DRBs for a given UE. The procedure uses UE-associated signalling.

<Unchanged Text Omitted>

8.2.1.3	Unsuccessful Operation

The unsuccessful operation is specified in the successful operation section.

Interactions with NAS Non Delivery Indication procedure:

If the NAS-PDU IE is contained in the PDU SESSION RESOURCE SETUP REQUEST message, the NG-RAN node shall, if supported, send the NAS NON DELIVERY INDICATION message to report the non-delivery of the received NAS PDU.

8.3.1.3	Unsuccessful Operation

Figure 8.3.1.3-1: Initial context setup: unsuccessful operation

If the NG-RAN node is not able to establish an NG UE context, it shall consider the procedure as failed and reply with the INITIAL CONTEXT SETUP FAILURE message.

If the PDU Session Resource Setup Request List IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall report to the AMF, in the INITIAL CONTEXT SETUP FAILURE message, the unsuccessful establishment result for each PDU session resource requested to be setup as defined in the PDU Session Resource Setup procedure.

Upon reception of the INITIAL CONTEXT SETUP FAILURE message the AMF shall, for each PDU session indicated in the PDU Session ID IE, transfer transparently the PDU Session Resource Setup Unsuccessful Transfer IE to the SMF associated with the concerned PDU session.

Interactions with NAS Non Delivery Indication procedure:

If the NAS-PDU IE is contained in the INITIAL CONTEXT SETUP REQUEST message, the NG-RAN node shall, if supported, send the NAS NON DELIVERY INDICATION message to report the non-delivery of the received NAS PDU.

<Unchanged Text Omitted>

9.2.5.4	NAS NON DELIVERY INDICATION

This message is sent by the NG-RAN node and is used for reporting the non-delivery of a NAS PDU previously received within a DOWNLINK NAS TRANSPORT message and other messages specified in this version of the specification over the NG interface.

Direction: NG-RAN node  AMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | ignore |
| AMF UE NGAP ID | M |  | 9.3.3.1 |  | YES | reject |
| RAN UE NGAP ID | M |  | 9.3.3.2 |  | YES | reject |
| NAS-PDU | M |  | 9.3.3.4 |  | YES | ignore |
| Cause | M |  | 9.3.1.2 |  | YES | ignore |

Change Ends