3GPP TSG-RAN WG3 Meeting #112-e	R3-212025

E-meeting, 17-28 May 2021

Title: 	Further discussions on signalling transport for the new interface

Source: 	Huawei

Agenda item:	23.2.2

Document Type:	Discussion

# 1. Introduction

The signalling transport for the eNB CP-UP separation was discussed during RAN3#111-e. In R3-210977 [1], the summary of the offline discussion was presented. This paper tried to have further discussions on this topic.

# 2. Background

The following was excerpted from the chairnotes of RAN3#111-e, from which we could see that the signalling transport related issues are almost stable, the rest of the paper tries to summarize and give text proposals.

- use SCTP as transport bearer for AP signaling

- reuse the E1 signaling bearer protocol stack for the new interface

- the same functions as specified for E1 signaling bearer should also be supported over the new interface

- DTLS over SCTP should be supported

Common understanding is that IPsec is feasible

- Multiple SCTP endpoints per CP/UP pair should be supported

Common understanding is that the same principles as for all current RAN3 interfaces are kept

- The first SCTP association could be triggered either by CP or by UP, but the additional SCTP associations should be initiated by UP

- SCTP multi-homing should also be supported

-	Continue to discuss if anything is missing; To be continued...

# 3. Discussion

The functions provided by E1 signalling bearer are as follows:

-	Provision of reliable transfer of E1AP message over E1 interface.

-	Provision of networking and routeing function

-	Provision of redundancy in the signalling network

-	Support for flow control and congestion control

We think the same functions should also be supported over the new interface. They were also agreed in the last meeting.

Proposal 1:  The functions as specified for E1 signalling bearer should also be supported over the new interface.

In the stage 2 specification, only few occurrences of gNB-CU-CP and gNB-CU-UP appear. Therefore, it is easy to add the new eNB logical entities to each occurrence of gNB-CU-CP and gNB-CU-UP.

Proposal 2:  Capture the new logical entities to each occurrence of gNB-CU-CP and gNB-CU-UP for ng-eNB-CU and eNB CP-UP separation in TS 38.462.

The corresponding text proposals are shown in R3-212026 [2].

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	The functions as specified for E1 signalling bearer should also be supported over the new interface.

Proposal 2:	Capture the new logical entities to each occurrence of gNB-CU-CP and gNB-CU-UP for ng-eNB-CU and eNB CP-UP separation in TS 38.462.

# 5. Reference

- R3-210977, “Summary of offline discussion on signalling transport for enhanced eNB architecture evolution”, Huawei.

- R3-212026, “CR to 38.462 on the introduction of new interface”, Huawei.