3GPP TSG-RAN WG3 Meeting #112-e	R3-212164

Online, 17 – 28 May, 2021

Title: 	Remaining issues for IAB inter-donor migration

Source: 	Lenovo, Motorola Mobility

Agenda Item:	13.2.1.1

Document for:	Discussion and decision

- Introduction

In R16, IAB intra-donor migration has been specified, and in the R17 the scenario has been extended to inter-donor migration which is the key feature of the IAB R17. After discussion by three e-meetings, the procedure of the IAB inter-donor migration becomes more and more clear, and some phased agreements have been achieved in the last RAN3 111-e meeting [1].

RAN3 111-e: For a single-connected IAB-MT: The procedure for inter-donor migration of a (top-level) migrating IAB-MT supports: - reuse Xn handover procedure of the (top-level) migrating IAB-MT between two parent nodes connected to different IAB-donors, and - the migration of F1 transport path for the collocated and all descendent IAB-DUs (i.e. the anchor nodes for the logical F1 connection do not change) Inter-donor migration may terminate after top-level IAB-MT migration WA: migration of collocated IAB-DU after the migration of the (top-level) migrating IAB-MT, is not precluded If collocated IAB-DU is migrated, the Inter-donor migration procedure involves, among others: - the establishment of an F1-C association to the target donor, and - the context migration of the IAB-DU’s UEs and child IAB-MTs to the target CU.

In this contribution, some aspects for IAB inter-donor migration are further discussed, including resource collision, UL routing and bearer mapping for the migrating IAB node.

- Discussion

Resource collision between migration IAB-MT and IAB-DU

Based on the agreements above, inter-donor migration may terminate after top-level IAB-MT migration, as shown in the right part of Figure 1 [2]. At this stage, IAB-MT3 has migrated to the topology of CU2 and establishes the RRC connection with CU2. While the F1 connections of IAB-DU3 and IAB-DU of descendent IAB node(s) still terminate at source CU1 even though the F1 messages need to be transported via target path.

However, since IAB-MT3 and IAB-DU3 are under the control of different IAB-donor-CUs, the resource configuration of the IAB-MT3 is generated by the target parent IAB node, while the resource configuration of the IAB-DU3 is generated by the source IAB donor.

Observation 1: For the IAB node which MT and DU subject to different IAB-donor-CUs, the resource configuration of the MT is generated by the target parent IAB node, while the resource configuration of the DU is generated by the source IAB donor.

In addition, the resource configuration of IAB-MT3 and IAB-DU3 need to meet the multiplexing capability of the IAB node. Therefore, once either the IAB-MT or the IAB-DU updates its resource configuration, the peer side needs to update the resource configuration accordingly.

Observation 2: To meet the multiplexing capability of the IAB node, once either the IAB-MT or the IAB-DU updates its resource configuration, the peer side needs to update the resource configuration accordingly.

During the IAB-MT3 migration, the resource configuration of IAB-MT3 may be updated via the handover command. Or after IAB-MT3 has migrated to the CU2, the resource configuration of IAB-MT3 may be also reconfigured occasionally. CU1 needs to be aware of the updated resource configuration of IAB-MT3 so as to update the resource configuration of IAB-DU3.

Proposal 1: For the IAB node which MT and DU subject to different CUs, the CU which terminates the F1 connection needs to be aware of the updated resource configuration of MT so as to update the resource configuration of DU.

In the same way when the resource configuration of IAB-DU3 is reconfigured, target parent IAB node needs to be aware of the updated resource configuration of IAB-DU3 so as to update the resource configuration of IAB-MT3. In addition, target parent IAB node shall be notified with the multiplexing capability of the IAB node.

Proposal 2: For the IAB node which MT and DU subject to different CUs, the target parent node needs to be aware of the updated resource configuration of DU as well as the multiplexing capability of the IAB node so as to update the resource configuration of MT.

Figure 1: An example for top-level IAB-MT inter-donor migration

UL routing and bearer mapping configuration for migration IAB-MT

In R16, BAP routing and bearer mapping for BAP entity are configured by the IAB-donor-CU via F1AP signaling both for the UL(MT) and DL(DU), except for the UL default configuration which configured by RRC. And IAB node delivers the UL BAP routing and bearer mapping from DU to the collocated MT via the internal interface.

However, for the migration IAB node which MT and DU subject to different CUs, the legacy way for UL BAP routing and bearer mapping configuration is no longer suitable. Firstly, when it comes to the configuration entity, target CU2 is preferred than source CU1 since MT has migrated from source CU1 to target CU2.

Proposal 3: For the IAB node which MT and DU subject to different CUs, the UL routing and bearer mapping configurations are determined by the CU which MT connected.

But the target CU2 does have the F1 connection with migration IAB node and the configuration cannot be transported to IAB node via the F1AP signaling. Thus, how to transport the configuration between IAB node and target CU2 need to be studied. One straightforward method is to use the RRC message directly between CU2 and IAB-MT3. Alternatively, the configuration which generated by the CU2 can be firstly transported to CU1 and then forward to the IAB-node via F1AP.

Proposal 4: The generated UL routing and bearer mapping configurations can be delivered to the IAB-MT via the following two candidate options.

- Opt1: the target CU configure the configuration to the IAB-MT directly via RRC

- Opt2: the configurations are sent to source CU and then forward to the IAB-node via F1AP

As agreed in the RAN3 111-e meeting for IAB topology redundancy:

To support the bearer mapping across two topologies at the boundary IAB node, the non-F1-termination donor CU needs to provide the ingress BH RLC CH ID(s) for DL traffic and egress BH RLC CH ID(s) for UL traffic to the F1-termination donor CU.

For the DL bearer mapping configurations for the migration IAB-DU, it is similar to the scenario of IAB topology redundancy, the CU2 (non-F1-termination donor CU) needs to provide the ingress BH RLC CH ID(s) for DL traffic to the CU1 (F1-termination donor CU).

While for the UL bearer mapping configurations for the migration IAB-MT, it is just the opposite of solution for DL. The CU1 (F1-termination donor CU) needs to provide the egress BH RLC CH ID(s) for UL traffic to the CU2 (F1-termination donor CU).

Proposal 5: To support the bearer mapping across two topologies at the migration IAB node:

- the non-F1-termination donor CU needs to provide the ingress BH RLC CH ID(s) for DL traffic to the F1-termination donor CU

- the F1-termination donor CU needs to provide the egress BH RLC CH ID(s) for UL traffic to the F1-termination donor CU

# Conclusion

This contribution aims to analyze the IAB inter-donor migration, including resource collision, UL routing and bearer mapping for the migrating IAB node. And following observations and proposals are concluded.

Observation 1: For the IAB node which MT and DU subject to different IAB-donor-CUs, the resource configuration of the MT is generated by the target parent IAB node, while the resource configuration of the DU is generated by the source IAB donor.

Observation 2: To meet the multiplexing capability of the IAB node, once either the IAB-MT or the IAB-DU updates its resource configuration, the peer side needs to update the resource configuration accordingly.

Proposal 1: For the IAB node which MT and DU subject to different CUs, the CU which terminates the F1 connection needs to be aware of the updated resource configuration of MT so as to update the resource configuration of DU.

Proposal 2: For the IAB node which MT and DU subject to different CUs, the target parent node needs to be aware of the updated resource configuration of DU as well as the multiplexing capability of the IAB node so as to update the resource configuration of MT.

Proposal 3: For the IAB node which MT and DU subject to different CUs, the UL routing and bearer mapping configurations are determined by the CU which MT connected.

Proposal 4: The generated UL routing and bearer mapping configurations can be delivered to the IAB-MT via the following two candidate options.

- Opt1: the target CU configure the configuration to the IAB-MT directly via RRC

- Opt2: the configurations are sent to source CU and then forward to the IAB-node via F1AP

Proposal 5: To support the bearer mapping across two topologies at the migration IAB node:

- the non-F1-termination donor CU needs to provide the ingress BH RLC CH ID(s) for DL traffic to the F1-termination donor CU

- the F1-termination donor CU needs to provide the egress BH RLC CH ID(s) for UL traffic to the F1-termination donor CU

# Reference

- Chairman notes of RAN3 111 e-meeting.

- R3-211199. CB#34 IAB_MigrationProcedureDetails. Qualcomm (moderator).