3GPP TSG-RAN WG3 #110-e	R3-202190

E-meeting, 2nd – 12th November 2020

Source:	CATT

Title:	Discussion on use cases for AI in RAN

Agenda Item:	18.3

Document for:	Discussion and decision

- Introduction

During RAN3#110-e meeting, there was a discussion on use cases for AI in RAN, and finally 3 use cases were agreed to be focused, while others remained FFS (not precluded). Generally speaking, some of the TDocs raised in that meeting focused on predictions, and others focused on what could be optimised (e.g. MLB, similar to the use cases in conventional SON).

In this TDoc we want to emphasise that the predictions and “what could be optimised” are two different layers of the issue, and to ask RAN3 on how to reflect this in the TR, considering the centre for AI function in RAN seems to be predictions at least in the coming couple of years.

- Discussion

In conventional SON for RAN, the optimisation methods for different use cases are largely decoupled, as well as the impacts on specs. Nevertheless, there are still a few scenarios where use cases are more or less relative, e.g. the Mobility Setting Change procedure is used for both MRO (Mobility Robustness Optimisation) and MLB (Mobility-based Load Balancing), and the load reporting mechanism is used for both MLB and Energy Saving. However, the entangled parts are not complex, e.g. for load reporting one node just reports its current load status toward another.

But for AI-enabled RAN, the relation among use cases is expected to be much higher. For the 3 use cases that agreed to be focused during the RAN3#110-e meeting, both the “load balancing” and the “energy saving” deeply rely on cell load prediction, whereas the “traffic steering/mobility optimization” deeply rely on UE location prediction and/or service data prediction, along with some other use cases proposed in that meeting but not agreed to be focused. The AI-enabled prediction module more or less tends to be a separate module other than the AI-enabled decision module (i.e. the module deciding what parameter to adjust, and how), which may be deployed in different positions (for the training part and/or the inference part).

Considering the inputs for both the prediction part and the decision part are core topics for AI-enabled RAN, mixing the discussion of prediction and decision in RAN3’s discussion may not be a good approach.

Therefore, we propose to split the use cases into two types: “AI-assisted decision” where the output is the decision itself (i.e. to turn on/off a cell, to hand over a UE from this cell toward that cell), and the “AI-assisted prediction” where the output is merely the prediction result used as an intermediate input for decision making.

Proposal 1: RAN3 is proposed to consider splitting the use cases into two parts: “AI-assisted decision” where the output is the decision itself (i.e. to turn on/off a cell, to hand over a UE from this cell toward that cell), and the “AI-assisted prediction” where the output is merely the prediction result used as an intermediate input for decision making.

For the 3 use cases agreed to be focused:

- The “load balancing” use case can be translated into “load prediction” and “AI-assisted handover / data offloading decision”;

- The “energy saving” use case can be translated into “load prediction” and “AI-assisted cell/carrier/symbol/etc turning on/off decision”;

- The “traffic steering/mobility optimization” use case can be translated into “UE location prediction”, “service data prediction”, “AI-assisted handover / data offloading decision” and “AI-assisted radio resource configuration / scheduling”.

However, the “AI-assisted cell/carrier/symbol/etc turning on/off decision” part and the “AI-assisted radio resource configuration / scheduling” part, according to our observation, have not been focused by any TDoc: the benefit to including any “AI-assistance” into the decision making process is unclear yet. Therefore, we prefer not to include them at this point. Instead, we think they—without the “AI-assistance” feature—should be confirmed as two applications of the predictions listed above.

Proposal 2: If proposal 1 is agreed, RAN3 is proposed to focus firstly on “Load prediction” “UE location prediction”, “Service data prediction” and “AI-assisted handover / data offloading decision”.

Proposal 3: If proposal 1 is agreed, RAN3 is proposed to confirm that “cell/carrier/symbol/etc turning on/off decision” and “radio resource configuration / scheduling” can be affected by the aforementioned predictions.

Nevertheless, there may be some use cases with small benefit not worthy to introduce a huge prediction method dedicated for them, but they may still be worthy to use if the prediction method is already included in the network and additional work dedicated for these small use case is negligible.

- Conclusion

Proposal 1: RAN3 is proposed to consider splitting the use cases into two parts: “AI-assisted decision” where the output is the decision itself (i.e. to turn on/off a cell, to hand over a UE from this cell toward that cell), and the “AI-assisted prediction” where the output is merely the prediction result used as an intermediate input for decision making.

Proposal 2: If proposal 1 is agreed, RAN3 is proposed to focus firstly on “Load prediction” “UE location prediction”, “Service data prediction” and “AI-assisted handover / data offloading decision”.

Proposal 3: If proposal 1 is agreed, RAN3 is proposed to confirm that “cell/carrier/symbol/etc turning on/off decision” and “radio resource configuration / scheduling” can be affected by the aforementioned predictions.