3GPP TSG-RAN WG3 Meeting #112-e	R3-212209

E-meeting, 17-28 May 2021

Title: 	(TP for SON BLCR for 38.473) Coverage and Capacity Optimization

Source: 	Huawei

Agenda item:	10.2.2

Document Type:	other

# 1. Introduction

The Coverage and Capacity Optimization was discussed during RAN3#110-e and RAN3#111-e.

# 2. Background

Revise current agreements as follows:

In NG-RAN scenario, a NG-RAN node may send to a neighbor NG-RAN node a coverage modification list which includes deployment related information concerning the serving cells.

Add following agreements:

- The CCO signaling over Xn supports SSB beam coverage optimizations.

Add the following FFSs:

- FFS whether CCO over Xn is signaled as separate per cell state information and SSB state information or whether each cell state reflect a specific SSB configuration

- FFS who decides that a coverage modification is needed: gNB-DU or gNB-CU

- FFS who decides how to modify the coverage: gNB-DU or gNB-CU

# 3. Discussion

## 3.1 Signalling coverage state

Coverage modifications can be achieved in different ways, e.g. by modifying transmission power and/or the angular direction of a cell/beam. In LTE, one of the most important conclusions when discussing coverage modifications, was that the freedom for the RAN node should be limited in a sense that OAM pre-configures the possible configurations (e.g. power/angle) but RAN is free to choose among these configurations.

Observation 1: Similar principles as in LTE applies – OAM sends a set of configurations and RAN nodes are free to choose among these

Even if nodes would exchange detailed coverage configurations (e.g. power/angle), it is difficult for the receiving RAN node to understand the impact on his own coverage area. But if at least the cell state, related to the specific OAM configuration, is exchanged the receiving node can eventually learn the impact. Further – OAM could also configure each node with the best match between configurations in this cell and neighbour cells. This behaviour is however left to implementation in the past. It is sufficient to note that with the set of allowed configurations from OAM it is possible to let the receiving node adjust its own coverage based on coverage adjustments in neighbour cells.

Observation 2: If we rely on the set of OAM configurations, it is possible to perform mutual coverage adjustments.

In LTE, we studied and specified functionality for reducing the impact on MRO and mobility failures caused by these coverage modifications.

The cell configuration exchange and how to use it in LTE is described in 36.300 as follows:

Each eNB may be configured with alternative coverage configurations and an eNB may autonomously select and switch between these configurations, e.g. using the Active Antenna Systems functions.

An eNB may notify its neighbour eNBs about the coverage reconfiguration using the ENB CONFIGURATION UPDATE message with the list of cells with modified coverage included. The list contains the ECGI of each modified cell and its coverage state indicator. The indicator may be used at the receiving eNB to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the indicator to retrieve a previously stored Mobility Robustness Optimisation state. If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving eNB may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending eNB adds cells in inactive state, the receiving eNB may use this information to avoid connection or re-establishment failures.

The receiving node may also use the notification to reduce the impact on mobility. For example, the receiving eNB should avoid triggering handovers towards cell(s) that are indicated to be inactive.

For MRO, we relied on the exchanged cell state. Since MRO is a statistical procedure, it was specified that the cell coverage state could be used as follows when receiving an updated cell coverage state:

- save current MRO state for the previous cell coverage state

- retrieve a previously store MRO state for the new cell coverage state

Since the MRO state could contain not only the optimised parameters but also statistics received so far (but not yet adjusted for) this would help to reduce the time for MRO to reach a steady state for each cell coverage state in neighbour cells. There was a careful discussion on the range for the cell state configuration – a large value would allow for a more granular signalling of coverage states. But the drawback is that this reduce the chance that a specific MRO state has been previously visited. It should be noted that for MRO, the actual state to take into account is the combination of source and target cell. For the source cell (same node as hosting the MRO) it is a bit simpler since MRO can possible understand the impact of the different coverage configurations (e.g. that a small power increase will lead to a small coverage range increase) but this information is not known for cell coverage states of cells in other nodes.

Hence, controlling the value range for the signalled cell coverage state is important. Therefore, even if OAM would configure a huge set of allowed coverage states, it would still make sense to group these and only signal a limited set of coverage states to neighbour cells

Observation 3: The flexibility of the configured sets are not directly related to the granularity of the signalled coverage state

So when we discuss the solution for NR, we think that there is still a benefit to manage the number of states we introduce. It may be reasonable to extend the range slightly but it should be important to manage the states that we exchange.

Adding SSB signalling would mean that we extend the number of states. One can argue that we can still make an effort to limit the number of states, by e.g. having some rules that only a few SSB combinations are allowed to be signalled. But there is no real benefit of this. If we stick with indications per cell, we could for example have two cell configurations, where the only difference is the modification of one SSB. So we can still use cell configurations to express the differences in SSB. In short – signalling SSB does not bring any additional benefit.

Observation 4: There is no real benefit of exchanging the SSB configuration.

The only remaining part is to decide on the range for the coverage state. In the past we used 0..15. We think this should be increased to 0..31 to enable the signalling of more different states.

Define the range of cell coverage states as 0..31.

## 3.2 Who decides what over F1

The questions to answer seems to be who decides when to correct and who selects the appropriate action. The problem when assigning this functionality is that while the gNB-CU has a more global view, it has less information about radio related parameters. In the past, there is a principle that since gNB-DU owns the radio resources, he makes all radio related decisions. This principle is a little bit relaxed due to agreements on PCI selection, where gNB-CU is now in charge of selecting PCI, i.e. there is a shift to give more decisions to the gNB-CU and also allow cross-OAM configuration of gNB-DU parameters to gNB-CU.

Hence, if we would let the gNB-CU make all decisions, it would be up to the gNB-CU to decide on exact radio related parameters for which he may not have the full understanding. While if we let the gNB-DU make the decisions the different gNB-DUs may make non-optimal decisions. It seems that if we select one of these options we will end up with a solution with drawbacks.

Therefore, we would like to suggest a more flexible approach to allow for some freedom for implementation specific solutions.

Building on the discussions before, if we rely on OAM to configure a set of coverage configurations, and allowing that OAM can configure suitable relations between configurations in different cells, the decision could be split by gNB-DU and gNB-CU. One such example is if gNB-CU sends a set of recommended configurations to the DU possibly also indicating how beneficial the different options are or the priorities of different options. Similar solutions have been achieved before, e.g. in eICIC.

In the figure below we outline a possible solution. Depending on the content of the recommended cells, the solution can be:

- Centralised, i.e. the gNB-CU only provides a single recommended state

- Distributed, i.e. the gNB-CU provides a set of recommended states and the gNB-DU selects

As discussed in the previous section, the centralised solution is not optimal enough due to lack of full understanding on the radio resources for gNB-CU. Therefore, we prefer the distributed solution which gives the chance for gNB-DU to further select the finally optimal configuration.

The solution would be beneficial since:

- it allows to use the information available in the gNB-CU, e.g. on coordinated actions in different cells;

- it would give some freedom for gNB-DU to select a configuration that is suitable considering limitations in radio resources;

- If a gNB-CU is not configured with enough information to select a suitable configuration, he can leave this decision to the gNB-DU

- If a gNB-DU is not able to use the suggested configuration, he can reject all proposed configurations.

gNB-CU provides recommended confirmation change list to gNB-DU with benefit and gNB-DU makes the decision on which configuration to use.

To assist the gNB-DU to decide the SSB resources more reasonable, it is desirable that gNB-CU can provide more issue information to gNB-DU along with the cell coverage modification. One potential solution is that the gNB-CU can provide more precise details of the detected CCO issue. This will be beneficial for the gNB-DU to perform fine-tune the related configuration.

The detailed information of the detected CCO issue can be the issue type and issue description. The issue type can be weak coverage, coverage hole, overshoot coverage, or the DL and UL coverage mismatch. The issue description can include more precise analysis of the issue, e.g., the direction or the location, the time period or the severity level.

gNB-CU provides the issue information to gNB-DU including the issue type and issue description

- Note that the TP for this is available in Annex 1. Taking the current situation into account where it is difficult to reach a consensus, we also provide a proposal for compromise in the following section.

## 3.3 Potential compromise

As discussed in last meeting there are different solutions proposed by different companies. On one hand it is seen beneficial to let gNB-U decide on the adjustment, since the gNB-CU has a global view. On the other hand, it is the gNB-DU that has the detailed information on the available resources required for the different configurations (e.g. antenna tilt, power, etc). But another important factor is how the nodes are configured by OAM. In the discussion so far we have clarified that OAM defines the different configurations, but this information could be distributed in different ways.

Therefore one possible way forward is to define a signalling solution that can cover multiple scenarios:

- Scenario 1: CU decides by sending a single set of configuration to the gNB-DU (one per cell)
In this scenario, the gNB-CU is configured with all possible configurations available in the DU. gNB-CU is also configured with the relationship between the configurations and can select a combination involving multiple cells and multiple DUs. In the end it is however up to the gNB-DU whether the configurations is feasible to switch to.

- Scenario 2:  CU decides by sending multiple sets of configuration to the gNB-DU
The gNB-CU is configured in a similar way as above, but in order to give some flexibility for the DU to select feasible configurations, the gNB-CU provides an ordered list (or using a benefit value) of configuration sets to indicate which set is most beneficial for the gNB-CU. The gNB-DU can select one set of configuration accounting for the radio resource information or reject all the configurations.

- Scenario 3: CU triggers the change but gives no recommendation on the configurations
In this scenario, the gNB-CU is not configured with the different coverage configurations in the gNB-DU and therefore does not provide any recommended configurations, but instead it is completely left to the gNB-DU to decide which one to use. This can be taken as the implicit way for gNB-CU to indicate the gNB-DU to autonomously decide the configurations for the coverage change.

The signalling solution supporting these three scenarios could rely on sending either no suggested configuration, one set of suggested configurations or multiple sets of suggested configurations.

The idea would be that gNB-CU will decide which option to use, for example based on the available configurations from OAM.

- Discuss a possible compromise where gNB-CU sends either no suggested configuration, one set of configurations or multiple sets of suggested configurations.

- Note that the TP for this is available in Annex 2.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1:	Define the range of cell coverage states as 0..31.

Proposal 2:	gNB-CU provides recommended confirmation change list to gNB-DU with benefit and gNB-DU makes the decision on which configuration to use.

Proposal 3:	Proposal 3: gNB-CU provides the issue information to gNB-DU including the issue type and issue description

Proposal 4:	Discuss a possible compromise where gNB-CU sends either no suggested configuration, one set of configurations or multiple sets of suggested configurations.

In the annex, the relevant TP to the different BL CRs can be found.

# Annex 1 – TP on TS 38.473

Start of the change

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the gNB-CU TNL Association To Add List IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use it to establish the TNL association(s) with the gNB-CU. The gNB-DU shall report to the gNB-CU, in the gNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU as follows:

-	A list of TNL address(es) with which the gNB-DU successfully established the TNL association shall be included in the gNB-CU TNL Association Setup List IE;

-	A list of TNL address(es) with which the gNB-DU failed to establish the TNL association shall be included in the gNB-CU TNL Association Failed To Setup List IE.

If the GNB-CU CONFIGURATION UPDATE message includes gNB-CU TNL Association To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by both received TNL endpoints towards the gNB-CU. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by the received endpoint IP address(es).

If the gNB-CU TNL Association To Update List IE is contained in the gNB-CU CONFIGURATION UPDATE message the gNB-DU shall, if supported, overwrite the previously stored information for the related TNL Association(s).

If in the gNB-CU CONFIGURATION UPDATE message the TNL Association usage IE is included in the gNB-CU TNL Association To Add List IE or the gNB-CU TNL Association To Update List IE, the gNB-DU node shall, if supported, use it as described in TS 38.472 [22].

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Protected E-UTRA Resources List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall protect the corresponding resource of the cells indicated by E-UTRA Cells List IE for spectrum sharing between E-UTRA and NR.

If the GNB-CU CONFIGURATION UPDATE message contains the Protected E-UTRA Resource Indication IE, the receiving gNB-DU should forward it to lower layers and use it for cell-level resource coordination. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE when expressing its desired resource allocation during gNB-DU Resource Coordination procedure. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same gNB-DU.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE is absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows IAB-node access or not.

If the GNB-CU CONFIGURATION UPDATE message contains the Recommended Coverage Modification Item IE the gNB-DU shall, if supported, take the information into account for cell coverage configuration decisions. If the Coverage Issue Information IE is included in the Recommended Coverage Modification List IE, the gNB-DU shall use this to determine the nature of coverage issues. If the gNB-DU selects one of the recommended coverage configurations in the Cell Coverage Modification List IE in the Recommended Coverage Modification Item IE it shall include the corresponding coverage modification index in the Agreed Coverage Modificaton Index IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU Configuration Update: Unsuccessful Operation

If the gNB-DU cannot accept the update, it shall respond with a GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-CU CONFIGURATION UPDATE message towards the same gNB-DU.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

Start of next change

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >> E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>> E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Global Identifier as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| Recommended Coverage Modification List | O |  |  |  | Yes | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

Start of next change

#### 9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-DU to a gNB-CU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instance, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| >Cells Failed to be Activated Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| gNB-CU TNL Association Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| gNB-CU TNL Association Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Failed To Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE List |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - | - |
| >>NR CGI | M |  | 9.3.1.12 |  | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Agreed Coverage Modificaton Index | O |  | INTEGER (0..31, …) |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |

Start of next change

#### 9.3.1.x	Recommended Coverage Modification List

This IE indicates the list of available coverage modification information of a cell in the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Coverage Modification Item |  | 0.. <maxConfigItems> |  |  |
| >Coverage Modification Index | M |  | INTEGER (0..31, …) |  |
| >Coverage Issue Information | O |  | Coverage Issue Information 9.3.1.y |  |
| >Coverage Modification Benefit | M |  | INTEGER (0..100, ...) | Value 100 indicates the maximum benefit. Value 0 indicates unknown benefit. Values from 1 to 100 should be calculated on a linear scale |
| >Cell Modification list |  | 1 |  |  |
| >>Cell modification Item |  | 0 .. <maxCellingNBDU> |  |  |
| >>Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NR CGI of the cell to be modified. |
| >>Cell Coverage State | M |  | INTEGER (0..31, …) | Value '0' indicates that the cell will be inactivated. Other values Indicates that the cell will be actived and also indicates the coverage configuration of the concerned cell. |

| Range bound | Explanation |
| --- | --- |
| maxConfigItems | Maximum no. of coverage configuration items. Value is 512. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

#### 9.3.1.y	Coverage Issue Information

This IE indicates the coverage issue information between a cell in the gNB-DU and neighbour cell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Coverage Issue Information |  |  |  |  |
| >Issue Type | M |  | ENUMERATED(weak coverage, coverage hole, overshoot coverage, DL and UL imbalance, ...) |  |
| >Issue Description |  | 0..1 |  |  |
| >>Severity Level | M |  | Integer (0..10) | The value indicates the severity of the problem, where value "10" indicates the highest severity |
| >>Involved Cells | M |  | Enumerated(cell1only, cell1tocell2, bothcells) |  |
| >>Cell1 | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NR CGI of the cell to be modified. |
| >>SSB Index of Cell1 | O |  | INTEGER (0..63) |  |
| >>Cell2 | C-ifCell2 |  | Global NG-RAN Cell Identity 9.2.2.27 | NR CGI of the cell to be modified. |
| >>SSB Index of Cell2 | O |  | INTEGER (0..63) |  |
| >>Location | FFS |  | FFS | Geographic location, if available from RLF report |
| >>Time Stamp | FFS |  | FFS | Time when faliure occurred |

| Condition | Explanation |
| --- | --- |
| ifCell2 | This IE shall be present if the Involved Cells IE is set to value "cell1tocell2" or "bothCells". |

End of the change

# Annex 2 – TP on TS 38.473 – Compromise solution

Start of the change

### 8.2.5	gNB-CU Configuration Update

#### 8.2.5.1	General

The purpose of the gNB-CU Configuration Update procedure is to update application level configuration data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface. This procedure does not affect existing UE-related contexts, if any. The procedure uses non-UE associated signalling.

#### 8.2.5.2	Successful Operation

Figure 8.2.5.2-1: gNB-CU Configuration Update procedure: Successful Operation

The gNB-CU initiates the procedure by sending a GNB-CU CONFIGURATION UPDATE message including the appropriate updated configuration data to the gNB-DU. The gNB-DU responds with a GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message to acknowledge that it successfully updated the configuration data. If an information element is not included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall interpret that the corresponding configuration data is not changed and shall continue to operate the F1-C interface with the existing related configuration data.

The updated configuration data shall be stored in the respective node and used as long as there is an operational TNL association or until any further update is performed.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall activate the cell indicated by NR CGI IE and reconfigure the physical cell identity for which the NR PCI IE is included.

If Cells to be Deactivated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall deactivate the cell indicated by NR CGI IE.

If Cells to be Activated List Item IE is contained in the GNB-CU CONFIGURATION UPDATE message and the indicated cells are already activated, the gNB-DU shall update the cell information received in Cells to be Activated List Item IE.

If Cells to be Activated List Item IE is included in the GNB-CU CONFIGURATION UPDATE message, and the information for the cell indicated by the NR CGI IE includes the IAB Info IAB-donor-CU IE, the gNB-DU shall, if supported, apply the IAB STC Info IE therein to the indicated cell.

If the gNB-CU System Information IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall include the Dedicated SI Delivery Needed UE List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message for UEs that are unable to receive system information from broadcast.

If Dedicated SI Delivery Needed UE List IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU should take it into account when informing the UE of the updated system information via the dedicated RRC message.

If the gNB-CU TNL Association To Add List IE is contained in the gNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, use it to establish the TNL association(s) with the gNB-CU. The gNB-DU shall report to the gNB-CU, in the gNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the successful establishment of the TNL association(s) with the gNB-CU as follows:

-	A list of TNL address(es) with which the gNB-DU successfully established the TNL association shall be included in the gNB-CU TNL Association Setup List IE;

-	A list of TNL address(es) with which the gNB-DU failed to establish the TNL association shall be included in the gNB-CU TNL Association Failed To Setup List IE.

If the GNB-CU CONFIGURATION UPDATE message includes gNB-CU TNL Association To Remove List IE, and the Endpoint IP address IE and the Port Number IE for both TNL endpoints of the TNL association(s) are included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by both received TNL endpoints towards the gNB-CU. If the Endpoint IP address IE, or the Endpoint IP address IE and the Port Number IE for one or both of the TNL endpoints is included in the gNB-CU TNL Association To Remove List IE, the gNB-DU shall, if supported, initiate removal of the TNL association(s) indicated by the received endpoint IP address(es).

If the gNB-CU TNL Association To Update List IE is contained in the gNB-CU CONFIGURATION UPDATE message the gNB-DU shall, if supported, overwrite the previously stored information for the related TNL Association(s).

If in the gNB-CU CONFIGURATION UPDATE message the TNL Association usage IE is included in the gNB-CU TNL Association To Add List IE or the gNB-CU TNL Association To Update List IE, the gNB-DU node shall, if supported, use it as described in TS 38.472 [22].

For NG-RAN, the gNB-CU shall include the gNB-CU System Information IE in the GNB-CU CONFIGURATION UPDATE message. The SIB type to Be Updated List IE shall contain the full list of SIBs to be broadcast.

If Protected E-UTRA Resources List IE is contained in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall protect the corresponding resource of the cells indicated by E-UTRA Cells List IE for spectrum sharing between E-UTRA and NR.

If the GNB-CU CONFIGURATION UPDATE message contains the Protected E-UTRA Resource Indication IE, the receiving gNB-DU should forward it to lower layers and use it for cell-level resource coordination. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE when expressing its desired resource allocation during gNB-DU Resource Coordination procedure. The gNB-DU shall consider the received Protected E-UTRA Resource Indication IE content valid until reception of a new update of the IE for the same gNB-DU.

If Available PLMN List IE, and optionally also Extended Available PLMN List IE, is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available PLMN list and update the corresponding system information.

If Available SNPN ID List IE is contained in GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall overwrite the whole available SNPN ID list and update the corresponding system information.

If Cells Failed to be Activated Item IE is contained in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message, the gNB-CU shall consider that the indicated cells are out-of-service as defined in TS 38.401 [4].

If the Neighbour Cell Information List IE is present in the GNB-CU CONFIGURATION UPDATE message, the receiving gNB-DU shall use the received information for Cross Link Interference management and/or NR-DC power coordination. The gNB-DU shall consider the received Neighbour Cell Information List IE content valid until reception of an update of the IE for the same cell(s). If the Intended TDD DL-UL Configuration NR IE is absent from the Neighbour Cell Information List IE, whereas the corresponding NR CGI IE is present, the receiving gNB-DU shall remove the previously stored Neighbour Cell Information IE corresponding to the NR CGI.

If the GNB-CU CONFIGURATION UPDATE message includes Transport Layer Address Info IE, the gNB-DU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message includes Transport Layer Address Info IE, the gNB-CU shall, if supported, take into account for IPSec tunnel establishment.

If the GNB-CU CONFIGURATION UPDATE message contains the Uplink BH Non-UP Traffic Mapping IE, the gNB-DU shall, if supported, consider the information therein for mapping of non-UP uplink traffic.

If the IAB Barred IE is included in the GNB-CU CONFIGURATION UPDATE message, the gNB-DU shall, if supported, consider it as an indication of whether the cell allows IAB-node access or not.

If the GNB-CU CONFIGURATION UPDATE message contains the Recommended Coverage Modification Item IE the gNB-DU shall, if supported, take the included information into account for cell coverage configuration decisions. If the Coverage Issue Information IE is included in the Recommended Coverage Modification List IE, the gNB-DU shall use this to determine the nature of coverage issues. If at least one Cell Modification list IE is included in the Recommended Coverage Modification List IE the gNB-DU shall select one of the proposed coverage configurations in the Cell Coverage Modification List IE in the Recommended Coverage Modification Item IE and include this in the Agreed Coverage Modificaton List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message. If the Cell Modification list IE is not included in the Recommended Coverage Modification List IE, the gNB-DU may select a new coverage configuration and include this in the Agreed Coverage Modificaton List IE in the GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE message.

#### 8.2.5.3	Unsuccessful Operation

Figure 8.2.5.3-1: gNB-CU Configuration Update: Unsuccessful Operation

If the gNB-DU cannot accept the update, it shall respond with a GNB-CU CONFIGURATION UPDATE FAILURE message and appropriate cause value.

If the GNB-CU CONFIGURATION UPDATE FAILURE message includes the Time To Wait IE, the gNB-CU shall wait at least for the indicated time before reinitiating the GNB-CU CONFIGURATION UPDATE message towards the same gNB-DU.

#### 8.2.5.4	Abnormal Conditions

Not applicable.

Start of next change

#### 9.2.1.10	GNB-CU CONFIGURATION UPDATE

This message is sent by the gNB-CU to transfer updated information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instances, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-CU  gNB-DU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells to be Activated List |  | 0..1 |  | List of cells to be activated or modified | YES | reject |
| >Cells to be Activated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >> NR PCI | O |  | INTEGER (0..1007) | Physical Cell ID | - |  |
| >> gNB-CU System Information | O |  | 9.3.1.42 | RRC container with system information owned by gNB-CU | YES | reject |
| >>Available PLMN List | O |  | 9.3.1.65 |  | YES | ignore |
| >>Extended Available PLMN List | O |  | 9.3.1.76 | This is included if Available PLMN List IE is included and if more than 6 Available PLMNs is to be signalled. | YES | ignore |
| >>IAB Info IAB-donor-CU | O |  | 9.3.1.105 | IAB-related configuration sent by the IAB-donor-CU. | YES | ignore |
| >>Available SNPN ID List | O |  | 9.3.1.163 | Indicates the available SNPN ID list. If this IE is included, the content of the Available PLMN List IE and Extended Available PLMN List IE if present in the Cells to be Activated List Item IE is ignored. | YES | ignore |
| Cells to be Deactivated List |  | 0..1 |  | List of cells to be deactivated | YES | reject |
| >Cells to be Deactivated List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| gNB-CU TNL Association To Add List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Add Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Information | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | M |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| gNB-CU TNL Association To Remove List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Remove Item IEs |  | 1..<maxnoofTNLAssociation> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Transport Layer Address gNB-DU | O |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-DU. | YES | reject |
| gNB-CU TNL Association To Update List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association To Update Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU. | - |  |
| >>TNL Association Usage | O |  | ENUMERATED (ue, non-ue, both, ...) | Indicates whether the TNL association is only used for UE-associated signalling, or non-UE-associated signalling, or both. For usage of this IE, refer to TS 38.472 [22]. | - |  |
| Cells to be barred List |  | 0..1 |  | List of cells to be barred. | YES | ignore |
| >Cells to be barred List Item |  | 1.. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cell Barred | M |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| >>IAB Barred | O |  | ENUMERATED (barred, not-barred, ...) |  | - |  |
| Protected E-UTRA Resources List |  | 0..1 |  | List of Protected E-UTRA Resources. | YES | reject |
| >Protected E-UTRA Resources List Item |  | 1.. <maxCellineNB> |  |  | EACH | reject |
| >>Spectrum Sharing Group ID | M |  | INTEGER (1.. maxCellineNB) | Indicates the E-UTRA cells involved in resource coordination with the NR cells affiliated with the same Spectrum Sharing Group ID. | - |  |
| >> E-UTRA Cells List |  | 1 |  | List of applicable E-UTRA cells. | - |  |
| >>> E-UTRA Cells List Item |  | 1 .. <maxCellineNB> |  |  | - |  |
| >>>>EUTRA Cell ID | M |  | BIT STRING (SIZE(28)) | Indicates the E-UTRAN Cell Global Identifier as defined in subclause 9.2.14 in TS 36.423 [9]. | - |  |
| >>>>Served E-UTRA Cell Information | M |  | 9.3.1.64 |  | - |  |
| Neighbour Cell Information List |  | 0..1 |  |  | YES | ignore |
| >Neighbour Cell Information List Item |  | 1 .. <maxCellingNBDU> |  |  | EACH | ignore |
| >>NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Intended TDD DL-UL Configuration | O |  | 9.3.1.89 |  | - |  |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Uplink BH Non-UP Traffic Mapping | O |  | 9.3.1.103 |  | YES | reject |
| Recommended Coverage Modification List | O |  |  |  | Yes | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum numbers of cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum numbers of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxCellineNB | Maximum no. cells that can be served by an eNB. Value is 256. |

Start of next change

#### 9.2.1.11	GNB-CU CONFIGURATION UPDATE ACKNOWLEDGE

This message is sent by a gNB-DU to a gNB-CU to acknowledge update of information associated to an F1-C interface instance.

NOTE:	If F1-C signalling transport is shared among several F1-C interface instance, this message may transfer updated information associated to several F1-C interface instances.

Direction: gNB-DU  gNB-CU

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.3.1.1 |  | YES | reject |
| Transaction ID | M |  | 9.3.1.23 |  | YES | reject |
| Cells Failed to be Activated List |  | 0..1 |  | List of cells which are failed to be activated | YES | reject |
| >Cells Failed to be Activated Item |  | 1.. <maxCellingNBDU> |  |  | EACH | reject |
| >> NR CGI | M |  | 9.3.1.12 |  | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Criticality Diagnostics | O |  | 9.3.1.3 |  | YES | ignore |
| gNB-CU TNL Association Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| gNB-CU TNL Association Failed to Setup List |  | 0..1 |  |  | YES | ignore |
| >gNB-CU TNL Association Failed To Setup Item IEs |  | 1..<maxnoofTNLAssociations> |  |  | EACH | ignore |
| >>TNL Association Transport Layer Address | M |  | CP Transport Layer Address 9.3.2.4 | Transport Layer Address of the gNB-CU | - |  |
| >>Cause | M |  | 9.3.1.2 |  | - |  |
| Dedicated SI Delivery Needed UE List |  | 0..1 |  | List of UEs unable to receive system information from broadcast | YES | ignore |
| >Dedicated SI Delivery Needed UE List |  | 1 .. <maxnoofUEIDs> |  |  | EACH | ignore |
| >>gNB-CU UE F1AP ID | M |  | 9.3.1.4 |  | - | - |
| >>NR CGI | M |  | 9.3.1.12 |  | - | - |
| Transport Layer Address Info | O |  | 9.3.2.5 |  | YES | ignore |
| Agreed Coverage Modificaton List | O |  | 9.3.1.z |  | YES | reject |

| Range bound | Explanation |
| --- | --- |
| maxCellingNBDU | Maximum no. cells that can be served by a gNB-DU. Value is 512. |
| maxnoofTNLAssociations | Maximum no. of TNL Associations between the gNB-CU and the gNB-DU. Value is 32. |
| maxnoofUEIDs | Maximum no. of UEs that can be served by a gNB-DU. Value is 65536. |

Start of next change

#### 9.3.1.x	Recommended Coverage Modification List

This IE includes information on detected coverage issues and optional recommended coverage modifications.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Recommended Coverage Modification Item |  | 0.. <maxConfigItems> |  |  |
| >Coverage Issue Information | O |  | Coverage Issue Information 9.3.1.y |  |
| >Coverage Modification Benefit | M |  | INTEGER (0..100, ...) | Value 100 indicates the maximum benefit. Value 0 indicates unknown benefit. Values from 1 to 100 should be calculated on a linear scale |
| >Cell Modification list |  | 0..1 |  |  |
| >>Cell modification Item |  | 1 .. <maxCellingNBDU> |  |  |
| >>Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NR CGI of the cell to be modified. |
| >>Cell Coverage State | M |  | INTEGER (0..31, …) | Value '0' indicates that the cell will be inactivated. Other values Indicates that the cell will be actived and also indicates the coverage configuration of the concerned cell. |

| Range bound | Explanation |
| --- | --- |
| maxConfigItems | Maximum no. of coverage configuration items. Value is 512. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

#### 9.3.1.y	Coverage Issue Information

This IE indicates the coverage issue information between a cell in the gNB-DU and neighbour cell(s).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Coverage Issue Information |  |  |  |  |
| >Issue Type | M |  | ENUMERATED(weak coverage, coverage hole, overshoot coverage, DL and UL imbalance, ...) |  |
| >Issue Description |  | 0..1 |  |  |
| >>Severity Level | M |  | Integer (0..10) | The value indicates the severity of the problem, where value "10" indicates the highest severity |
| >>Involved Cells | M |  | Enumerated(cell1only, cell1tocell2, bothcells) |  |
| >>Cell1 | M |  | Global NG-RAN Cell Identity 9.2.2.27 |  |
| >>SSB Index of Cell1 | O |  | INTEGER (0..63) |  |
| >>Cell2 | C-ifCell2 |  | Global NG-RAN Cell Identity 9.2.2.27 |  |
| >>SSB Index of Cell2 | O |  | INTEGER (0..63) |  |
| >>Location | FFS |  | FFS | Geographic location, if available from RLF report |
| >>Time Stamp | FFS |  | FFS | Time when faliure occurred |

| Condition | Explanation |
| --- | --- |
| ifCell2 | This IE shall be present if the Involved Cells IE is set to value "cell1tocell2" or "bothCells". |

#### 9.3.1.z	Agreed Coverage Modification List

This IE indicates the list of agreed coverage modification information from the gNB-DU.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Cell Modification list |  | 1 |  |  |
| >Cell modification Item |  | 1 .. <maxCellingNBDU> |  |  |
| >Cell Identity | M |  | Global NG-RAN Cell Identity 9.2.2.27 | NR CGI of the cell to be modified. |
| >Cell Coverage State | M |  | INTEGER (0..31, …) | Value '0' indicates that the cell will be inactivated. Other values Indicates that the cell will be actived and also indicates the coverage configuration of the concerned cell. |

| Range bound | Explanation |
| --- | --- |
| maxConfigItems | Maximum no. of coverage configuration items. Value is 512. |

| Condition | Explanation |
| --- | --- |
| ifCellDeploymentStatusIndicatorPresent | This IE shall be present if the Cell Deployment Status Indicator IE is present. |

End of the change

# Annex 3 – TP for 38.401

# 7	NG-RAN functions description

Start of the change

## 7.X	Coverage and Capacity Optimisation Function

The Coverage and Capacity Optimisation Function in non-split gNB case is specified in TS 38.300 [2].

In case of split gNB architecture, Coverage and Capacity Optimisation Function detection function is located at the gNB-CU. To perform Coverage and Capacity optimisation at gNB-DU, gNB-CU may send a set of alternative recommended configurations to gNB-DU via F1AP signalling. The gNB-DU selects a suitable configuration and signals the selected configuration to gNB-CU.

End of the change

# Annex 4 – TP for 38.470

# 5	Functions of the F1 interface

Start of the change

## 5.1	General

The following clauses describe the functions supported over F1-C and F1-U.

## 5.2	F1-C functions

### 5.2.1	F1 interface management function

The error indication function is used by the gNB-DU or gNB-CU to indicate to the gNB-CU or gNB-DU that an error has occurred.

The reset function is used to initialize the peer entity after node setup and after a failure event occurred. This procedure can be used by both the gNB-DU and the gNB-CU.

The F1 setup function allows to exchange application level data needed for the gNB-DU and gNB-CU to interoperate correctly on the F1 interface, and exchange the intended TDD DL-UL configuration originating from the gNB-DU or destined to the gNB-DU. The F1 setup is initiated by the gNB-DU.

The gNB-CU Configuration Update and gNB-DU Configuration Update functions allow to update application level configuration data needed between gNB-CU and gNB-DU to interoperate correctly over the F1 interface, and may activate or deactivate cells. For cross-link interference mitigation, the gNB-CU may coordinate the exchange of  intended TDD DL-UL configuration by merging, forwarding and selective forwarding of intended TDD DL-UL configuration(s) between its gNB-DUs, or between its gNB-DUs and other gNBs, gNB-CUs. With the gNB-CU Configuration Update function, energy saving with cell activation/deactivation can be supported as defined in TS 38.300 [8]. For coverage and capacity optimisation, the gNB-CU may send a set of recommended configurations to the gNB-DU and the gNB-DU responds with the selected configuration.

The F1 setup and gNB-DU Configuration Update functions allow to inform the S-NSSAI(s), CAG ID(s) and NID(s) supported by the gNB-DU.

The F1 setup and gNB-CU Configuration Update functions allow to inform the NID(s) available at the gNB-CU.

The F1 resource coordination function is used to transfer information about frequency resource sharing between gNB-CU and gNB-DU. In case of split gNB architecture, the gNB-CU may consolidate the outgoing messages from multiple gNB-DUs and distribute the incoming messages to the involved gNB-DUs, to perform resource coordination.

The gNB-DU status indication function allows the gNB-DU to indicate overload status to gNB-CU.

The network access rate reduction function is used to indicate to the gNB-DU that the rate at which UEs are accessing the network need to be reduced.

The F1 removal function is used to remove the interface instance and all related resources between the gNB-DU and the gNB-CU in a controlled manner.

End of the change

# Annex 5 – TP for 38.300

Start of the change

### 15.5.2.X	Dynamic coverage configuration changes

Each NG-RAN node may be configured with alternative coverage configurations and a NG-RAN node may autonomously select and switch between these configurations, e.g. using the Active Antenna Systems functions.

A NG-RAN node may notify its neighbour NG-RAN nodes about the coverage reconfiguration using the NG-RAN NODE CONFIGURATION UPDATE message with the list of cells with modified coverage included. The list contains the CGI of each modified cell and its coverage state indicator. The indicator may be used at the receiving NG-RAN node to adjust the functions of the Mobility Robustness Optimisation, e.g. by using the indicator to retrieve a previously stored Mobility Robustness Optimisation state. If the list includes indication about planned reconfiguration and possibly a list of replacing cells, the receiving NG-RAN node may use this to avoid connection or re-establishment failures during the reconfiguration. Also, if the sending NG-RAN node adds cells in inactive state, the receiving NG-RAN node may use this information to avoid connection or re-establishment failures.

The receiving NG-RAN node may also use the notification to reduce the impact on mobility. For example, the receiving NG-RAN node should avoid triggering handovers towards cell(s) that are indicated to be inactive.

End of the change