3GPP TSG-RAN WG3 Meeting #112e	R3-212329

# Online Meeting, 17th – 27th May 2021

Agenda Item:	9.2.3

Source:	Ericsson

Title: 	Discussion on IMS Voice EPS Fallback

Document for:	Discussion and Approval

# 1	Background

At the last RAN3 meeting, we further discussed the IMS Voice EPS Fallback in EPC.

This paper continues on this topic.

# 2	Discussion

RAN3 has agreed in CR1694 to introduce new IE “IMS voice EPS fallback from 5G” in the “Source eNB to Target eNB Transparent Container”. So that the eNB could after IMS call is terminated, perform action e.g. sending the UE back to 5G.

However this is no wording either in semantic or procedural text to describe this optional IE. In our view, it would be clearer to have such a wording. We propose to have a short sentence in the procedural text. It would also work if include in the tabular semantic description.

Proposal 1: RAN3 to discuss and agree to describe in S1AP the early agreed IE IMS voice EPS fallback from 5G.

The correction Cs are submitted in [1] and [2]. The proposal is to include the description in the procedural text, but it would be fine to include it in the semantic, if so preferred.

At the last meeting, we discussed if the IMS voice EPS fallback from 5G Information needs to be included in X2 Handover.

In the case that there is a radio link failure and the UE performs the reestablishment in a new eNB, to align the handling of “IMS voice EPS fallback from 5G”, it would be beneficial to provide the IMS voice EPS fallback from 5G to the new eNB in the Context Retrieve procedure.

Proposal 2: RAN3 to discuss to introduce the IMS voice EPS fallback from 5G in the UE Context Retrieve procedure

The CRs are submitted in [3] and [4].

# 3	Proposals

Proposal 1: RAN3 to discuss and agree to describe in S1AP the early agreed IE IMS voice EPS fallback from 5G.

The correction Cs are submitted in [1] and [2]. The proposal is to include the description in the procedural text, but it would be fine to include it in the semantic, if so preferred.

Proposal 2: RAN3 to discuss to introduce the IMS voice EPS fallback from 5G in the UE Context Retrieve procedure

The CRs are submitted in [3] and [4].

# 4	Reference

[1]	R3-212330	Correction of IMS voice EPS fallback from 5G, Ericsson

[2]	R3-212331	Correction of IMS voice EPS fallback from 5G, Ericsson

[3]	R3-212332	Introduce the IMS voice EPS fallback from 5G in the Retrieve UE Context procedure, Ericsson

[4]	R3-212333	Introduce the IMS voice EPS fallback from 5G in the Retrieve UE Context procedure, Ericsson