3GPP TSG-RAN WG3 Meeting #112-e	R3-212634

# Online, 17th – 27th May 2021

Agenda Item:	8.1

Source:	Ericsson

Title:	Response to R3-212061, R3-212062, R3-212063 and R3-212064

Document for:	Agreement

# Introduction

The SA2 LS on making PSCell ID available at the SGW of EPC [1] states the following for Rel-17:

SA2 thanks SA5 and ETSI TC LI for their guidance on making PSCell ID available at the SGW in EPC so that it can be included in the charging information. SA2 apologies for delay in introducing this Rel-17 change into 3GPP specifications. Due to high number of Rel-16 corrections, SA2 was not able to trigger the generation of Rel-17 TS versions earlier.

In earlier TS versions, the awareness of PSCell ID was used for MME internal processing, but now SA2 has agreed the attached Rel-17 CR on TS 23.401 that adds the MME procedure for passing on the PSCell ID information to SGW.

In SA2 understanding, corresponding 5GS CR is not necessary.

Although in paper R3-212061, it says ULI should be added to more NGAP/S1AP messages since PScell ID needs to be reported. This statement misinterprets the intention of the above LS.

# Discussion

Starting from RAN3#103 meeting, the requirements of lawful interception in LS [2] triggered RAN3 work on the introduction of PSCell ID to ULI. Massive discussion was held in the meetings by firstly analysing the scenarios from operator and then the dedicated changes. A summary of offline concludes the followings [3]:

- In NGAP, PSCell is missing in ULI.

- In S1AP, equivalent functionality exists but is not on a par with NGAP.

Finally the CRs were agreed for S1AP and NGAP with careful thought to avoid heavy signaling, e.g., filtering out Resume message and etc. Go further, the location reporting procedure is in place for the CN to request for UE location from RAN node once required, not to say in which scenario.

Proposal 1: Current spec supports transferring PSCell ID within ULI to the CN, and this is already aligned with SA2/SA3-LI requirements.

This is why the LS [1] correctly has no action for RAN3 (we are only in cc). Furthermore, it is worth noting that SA2 is asking for the availability of PSCell ID info in the SGW for R17. The CRs proposed are for R15 and R16 though.

Proposal 2: Keep RAN3’s agreements unchanged for R17 and earlier releases.

# References

[1] R3-211444, Reply LS on making PSCell ID available at the SGW of EPC, SA2

[2] R3-190119, Reporting All Cell IDs in 5G

[3] R3-191110, Summary of offline discussion, Location Reporting