3GPP TSG-RAN WG3 #112-e	R3-212607

Online, 17 – 28 May 2021

Agenda Item:	8.1

Source:	Nokia (moderator), Nokia Shanghai Bell

Title:	Summary of Offline Discussion on CHO with SCG config (CB #10)

Document for:	Approval

# Introduction

CB: # 10_CHO_SCGconfig

- (Nok) enabling the scenarios listed in the LS requires changes in RAN3 signaling, hence with the current signaling CHO with MR-DC will not work; add the necessary signaling

- (HW) Scenarios 1-3 has RAN3 st3 impact; Scenario 4 has st2 impact

- (E///) st3 impact is limited (flag in SN Add Req)

- (Gg) There could be st3 impacts (e.g. timer issue at tgt SN for Rel-16)

- consensus that no other scenarios have been identified?

- agree reply LS

(Nok - moderator)

[NWM] Summary of offline disc R3-212607

# For the Chairman’s Notes

Propose the following:

The scenarios listed in the LS from RAN2 (R3-211433) require changes in RAN3 specifications. Identified possible impacts:

- an indicator of CHO,

- signalling for the early data forwarding to the target SN,

- enhancements to avoid over-booking of resources at the SN.

RAN3 has not identified any other scenario, not listed in the LS from RAN2.

RAN3 will ask RAN2 for guidance if the changes are to be introduced in Rel-16.

The addition of the missing arrival probability in F1AP is a Rel-16 correction:

R3-211520 rev in R3-212847 – agreed

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## Indication of CHO

All companies that contributed in the discussion acknowledge that at least some of the scenarios listed in the LS from RAN2 [1] require changes in RAN3’s specifications. Some companies consider that the problem is the delay [2,11], other directly suggest that an indicator is needed to let the SN know the Addition is related to a CHO [2, 7, 9].

Question 1-1: May it be assumed RAN3 acknowledges the need to add an indicator or a flag to the Addition procedure (in Rel.16) to let the SN know the Addition is related to a prepared CHO?

| Company | Comment |
| --- | --- |
| Nokia | Yes, we propose to enable CHO indication in Rel.16. |
| Huawei | Yes, the CHO indicator is needed for SN additional procedure. But we also think that this indicator is also needed for SN modification procedure in intra node handover with SN configuration case in scenario 1 as analysed in [7]. And we should also study how to support early data forwarding for split bearers in this case. No sure if we should introduce it in rel-16 or later. Because the original RAN2 agreement is to support this function only in cases without RAN3 impact. And in the LS, RAN2 just asks RAN3 to check the RAN3 impact. |
| Google | No. From RAN2’s meeting minutes "RAN2#109-bis agreed that we will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration and limit to cases without RAN3 impact. RAN2 would like to check with RAN3 on scenario 1/2/3/4 in R2-2103332. The intention is to see whether there are new RAN3 impacts for these scenarios or not. From RAN2 point of view, if there are new RAN3 impacts for a specific scenario, the scenario will not be supported in Rel-16." it can be seen that they want to avoid RAN3 impact in Rel-16. We should just answer that there is indeed RAN3 impact in the listed scenarios. |
| ZTE | No, in the LS, it states that “We will not preclude SCG configuration in RRC Reconfiguration with conditional reconfiguration. Limit to cases without RAN3 impact”, it means that support of this case shall not impact RAN3 specs. Meanwhile, considering Rel-16 is frozen, we prefer to consider the enhancement in R17 DCCA or R17 TEI. |
| Ericsson | Yes indicator is needed if we want to support the cases mentioned in the LS. But to be fair, RAN2 is asking for impact, not corrections. |
| Intel | Agree with Google, ZTE, and Ericsson. For Rel-16, we already agreed and specified in TS 37.340 Section 10.1 as follows: In case MR-DC is configured, CHO is only supported in Master Node to eNB/gNB Change procedure in this release. Based on that, the scenarios considered in RAN2 LS seems out of scope of Rel-16.. |
| Lenovo, Motorola Mobility | We also think an indication or flag is needed. However, it is out of scope of Rel-16. |
| CATT | Agree with Gg, ZTE. |
| Verizon | Yes. |
| Qualcomm | Yes, R16 is fine |

Furthermore, in [2], it is proposed that the indicator is made so that it can be reused for CPA purposes in Rel.17.

Question 1-2: Shall RAN3 make the CHO indicator/flag so that it can be reused to indicate CPA in Rel.17?

| Company | Comment |
| --- | --- |
| Nokia | This is not a mandatory approach, but RAN3 hall consider if such combination would not help us to avoid duplication of flags with nearly identical meaning. |
| Huawei | We prefer to reuse a single indicator for this two cases. |
| Google | Only introduce it in Rel-17. |
| ZTE | See the same comment as above. The indication can be considered in R17 DCCA WI. |
| Ericsson | Should be possible. Actions at receiving node (i.e. target SN) look similar. But may need further study |
| Intel | Agree with Google and ZTE. This RAN2 LS is out of scope of Rel-16. |
| Lenovo, Motorola Mobility | Not sure, the indication for CPA has not been decided yet. |
| CATT | Only support in R17 |
| Verizon | Yes, re-use this single indicator for the two cases is good. |
| Qualcomm | Yes, we can use the indicator for both use cases. |

## Other issues related to the CHO with SCG config

In [2], it is explained that CHO may be very “expensive” in terms of resource allocation. A list of other enhancements based on the solutions already specified for the target MN is proposed to support CHO with SCG config without overloading the SN. These enhancements are:

- Relying the arrival probability from the CHO request to the Addition request (X2/Xn) and the UE Context Setup/Modification (F1);

- Enabling the SN to identify the Addition requests related to the same UE (multiple preparation in the CHO).

Question 2-1: Do companies acknowledge the above enhancements, or one of them, is beneficial for CHO with SCG config?

| Company | Comment |
| --- | --- |
| Nokia | We have proposed the two solutions complementary to what was enabled in CHO. We do not see why the SN could not benefit from resource optimisations that were seen necessary for the target MN (especially that CHO is expected to be “expensive” if not optimisation is enabled). So, yes, both are needed. |
| Huawei | First of all, the SN should be able to identify the multiple addition requests is for a single UE. OK to reuse the arrival probability |
| Google | FFS in Rel-17 |
| ZTE | See the same comment as above. Such enhancements can be considered in R17. |
| Ericsson | Arrival probability over Xn can be considered. But not in the scope of this LS Multiple SN addition for the same target SN are is not compatible with rel-17 CPAC WA |
| Intel | Agree with Google, ZTE, and Ericsson. However, we think (1) for F1 is relevant for Rel-16 as correction. We think it is beneficial for gNB-DU to consider the UE’s arrival probability during admission process of CHO. |
| Lenovo, Motorola Mobility | FFS in R17. |
| CATT | FFS in R17. |
| Verizon | Yes, the enhancements could be beneficial. |
| Qualcomm | The optimizations seem to be useful. We can discuss in R17. |

## Other scenarios

In the LS, RAN2 asks if RAN3 is aware of any other, not listed, scenario of a CHO with an SCG config. All contributing companies do not see to have identified such new scenario,

Question 3-1: Is there any other scenario, except of those listed in the LS?

| Company | Comment |
| --- | --- |
| Nokia | No other scenario identified. |
| Huawei | No. Do we need a rely LS at this meeting? |
| Google | No other scenario identified. |
| ZTE | Not sure of other scenarios, can consider later. |
| Ericsson | No. To Huawei: I expect this reply LS to be quite simple (yes there is an impact, no other scenario identified) so we should answer at this meeting |
| Intel | No. I think we can simply reply in a way that the scenarios in the LS are all out of scope of Rel-16 (by referring to Rel-16 TS 37.340 Section 10.1). |
| Lenovo, Motorola Mobility | No. we agree to send reply LS to RAN2 to state that all scenarios have RAN3 impact and they are out of scope of R16. |
| CATT | NO |
| Qualcomm | No |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-211433, LS on Conditional Handover with SCG configuration scenarios (TSG RAN WG2)

- R3-211517, Discussion on the possible impacts of a CHO with MR-DC operation (Nokia, Nokia Shanghai Bell)

- R3-211518, Enabling CHO with SCG configuration (Nokia, Nokia Shanghai Bell)

- R3-211519, Enabling CHO with SCG configuration (Nokia, Nokia Shanghai Bell)

- R3-211520, Enabling CHO with SCG configuration (Nokia, Nokia Shanghai Bell)

- R3-211521, Response LS on Conditional Handover with SCG configuration scenarios (Nokia, Nokia Shanghai Bell)

- R3-212171, Discussion on conditional handover with SCG configuration (Huawei)

- R3-212172, [Draft] Reply LS on on Conditional Handover with SCG configuration scenarios (Huawei)

- R3-212365, Conditional Handover with SCG configuration scenarios (Ericsson)

- R3-212366, [DRAFT] Reply LS on Conditional Handover with SCG configuration scenarios (Ericsson)

- R3-212405, Discussion on Conditional Handover with SCG configuration scenarios (Google Inc.)

- R3-212409, [Draft] Reply LS on Conditional Handover with SCG configuration scenarios (Google Inc.)