3GPP TSG-RAN WG3 Meeting #112-e	R3-212061

E-meeting, 17-28 May 2021

Title: 	Consideration on making PSCell ID available at the SGW of EPC

Source: 	Huawei

Agenda item:	8.1

Document Type:	Discussion

# Introduction

A Reply LS on making PSCell ID available at the SGW of EPC [1] was sent by SA2. In this reply LS, SA2 agreed to make PSCell ID available at the SGW in EPC so that it can be included in the charging information. To support this requirement from the perspective of RAN3, the PSCell ID information shall be provided to MME when necessary.

In S1 interface, the PSCell ID could be included in User Location Information IE and reported to core network via S1AP messages. However, we found that the User Location Information (ULI) is missing in some of the S1AP procedures, as well as the NGAP procedures.

In this document we will discuss the S1AP and NGAP procedures that may require the User Location Information IE to be reported to the core network.

# Discussion

In order to provide the PSCell ID to the core network, the User Location Information IE shall be transmitted to the MME. Generally, the User Location Information IE is provided to the core network from the RAN side when the location of the user is required, data transmission session (i.e. the PDU session or E-RAB) is released or modified, etc. However, we found that the User Location Information IE is missing in some necessary procedures; and furthermore, the User Location Information IE is misaligned in some S1AP and NGAP messages.

## Missing of User Location Information IE

In order to acknowledge the network about the PSCell ID contained in the ULI, the User Location Information IE should be included in some NGAP/S1AP messages.

The availability of the User Location Information IE or PSCell ID information in the NGAP and S1AP messages are shown in Table 1. For example, the User Location Information IE is included in the NGAP PDU SESSION RESOURCE MODIFY RESPONSE message; however, it is missing in S1AP E-RAB MODIFY RESPONSE message which has similar function with the NGAP PDU SESSION RESOURCE MODIFY RESPONSE message. To avoid that the absence of the PSCell ID in the core network, the content of the S1AP and NGAP messages on the User Location Information IE should be aligned.

Therefore, the User Location Information IE should be introduced to the following S1AP messages:

- E-RAB MODIFY RESPONSE,

- UE CONTEXT MODIFICATION RESPONSE,

- UE CONTEXT MODIFICATION INDICATION, and

- INITIAL UE MESSAGE.

And the following NGAP message:

- UE CONTEXT SUSPEND REQUEST message.

Table 1 Misalignment of the messages on the User Location Information IE

|  | LTE S1AP messages | ULI/ PSCell ID | NR NGAP messages | ULI/ PSCell ID |
| --- | --- | --- | --- | --- |
| E-RAB/ PDU Session management messages | E-RAB RELEASE RESPONSE | YES | PDU SESSION RESOURCE RELEASE RESPONSE | YES |
|  | E-RAB RELEASE INDICATION | YES |  | YES |
|  | E-RAB MODIFY RESPONSE | NO | PDU SESSION RESOURCE MODIFY RESPONSE | YES |
|  |  |  | PDU SESSION RESOURCE NOTIFY | YES |
|  | E-RAB MODIFICATION INDICATION | YES | PDU SESSION RESOURCE MODIFY INDICATION | YES |
| UE Context management messages | UE CONTEXT RELEASE COMPLETE | YES | UE CONTEXT RELEASE COMPLETE | YES |
|  | UE CONTEXT MODIFICATION RESPONSE | NO | UE CONTEXT MODIFICATION RESPONSE | YES |
|  | UE CONTEXT MODIFICATION INDICATION | NO |  |  |
|  | UE CONTEXT SUSPEND REQUEST | YES | UE CONTEXT SUSPEND REQUEST | NO |
| Mobility management messages | HANDOVER NOTIFY | YES | HANDOVER NOTIFY | YES |
|  | PATH SWITCH REQUEST | YES | PATH SWITCH REQUEST | YES |
| NAS Transport messages | INITIAL UE MESSAGE | NO | INITIAL UE MESSAGE | YES |
|  | UPLINK NAS TRANSPORT | YES | UPLINK NAS TRANSPORT | YES |
| Location Reporting messages | LOCATION REPORT | YES | LOCATION REPORT | YES |
| Data Usage Reporting | SECONDARY RAT DATA USAGE REPORT | YES | SECONDARY RAT DATA USAGE REPORT | YES |

- The User Location Information IE should be added to following S1AP messages:

  - E-RAB MODIFY RESPONSE,

  - UE CONTEXT MODIFICATION RESPONSE,

  - UE CONTEXT MODIFICATION INDICATION, and

  - INITIAL UE MESSAGE.

- The User Location Information IE should be added to following NGAP message(s):

  - UE CONTEXT SUSPEND REQUEST message.

## IoT related scenarios

At the scenarios of CIoT, a UE may triggered the MO-EDT procedure to send uplink data as shown in Figure 1. If the UE indicates to 5GC that there is no further UL/DL higher layer PDU after the 1st transmission, ng-eNB may request for immediate transition to RRC IDLE with Suspend after Step 3. Therefore, PDU sessions will not be resumed. In detail, to implement the immediate suspend, the AMF send the NG-AP Context Resume Response message with a Suspend indication in Step 5 to keep the UE in RRC_IDLE state.

During this procedure, it would be beneficial to inform User Location Information IE to the AMF; and therefore, the core network is able to obtain the latest location of the UE for further use. However, currently, there is no chance for the RAN side to provide the UE location information to the AMF in this case. Therefore, we suggest to include the User Location Information IE in the NGAP UE CONTEXT RESUME REQUEST message.

Figure 1 MO-EDT for User Plane CIoT 5GS Optimisation in TS 36.300

- Consider to include the User Location Information IE in to the following NGAP message to support the immediately suspend for CIoT scenarios:

  - NGAP UE CONTEXT RESUME REQUEST message

# Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: The User Location Information IE should be added to following S1AP messages:

  - E-RAB MODIFY RESPONSE,

  - UE CONTEXT MODIFICATION RESPONSE,

  - UE CONTEXT MODIFICATION INDICATION, and

  - INITIAL UE MESSAGE.

Proposal 2: The User Location Information IE should be added to following NGAP message(s):

  - UE CONTEXT SUSPEND REQUEST message.

Proposal 3: Consider to include the User Location Information IE in to the following NGAP message to support the immediately suspend for CIoT scenarios:

  - NGAP UE CONTEXT RESUME REQUEST message

# Reference

- S2-2100248, Reply LS on making PSCell ID available at the SGW of EPC, SA2 143e.