3GPP TSG-RAN WG3 Meeting #112-e	R3-212207

E-meeting, 17 – 28 May 2021

Title: 	(TP for SON BLCR for 38.423) MRO for SN Change Failure

Source: 	Huawei

Agenda item:	10.2.1.6

Document Type:	other

# 1. Introduction

The MRO for SN Change Failure was discussed during RAN3#111-e.

# 2. Background

In RAN3#111 e-meeting, there were some agreements and open issues on SN change failure.

MRO issues for PSCell change failure are defined as below:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

MN performs initial analysis to identify the node that caused the failure. The node that caused the failure performs root cause analysis.

Define new message from MN to the initiating SN to forward SCGfailureinformation.

Additional information related to SCG failure reported from UE may be beneficial; details FFS.

Open issue to be further discussed:

1.	Which information should be reported from the UE?

2.	Information other than SCGfailureinformation in new XnAP message.

3.	Mobility Information in S-NODE ADDITION REQUEST ACKNOWLEDGE message and the new XnAP message.

In this paper, we’ll give our considerations based on the agreements and open issues.

# 3. Discussion

RAN3 has agreed to define a new message for MN to forward SCGfailureinformation message to the initiating SN.

Define new message from MN to the initiating SN to forward SCGfailureinformation.

Proposal 1: The SCG CHANGE REPORT message is introduced from MN to the initiating SN to forward SCGfailureinformation.

Currently, when the UE detects SCG failure, it can send the SCGfailureinformation message to the MN. There are the latest measurement results in the SCGfailureinformation message.

In last meeting, it was agreed that:

MN performs initial analysis to identify the node that caused the failure. The node that caused the failure performs root cause analysis.

In our understanding, upon receiving SCGFailureInformation message, MN can perform the similar initial analysis as the failure node in the MRO scheme. In MRO, the RLF report will first be sent to the failure node where the UE detected the RLF. The failure node can perform the initial analysis to decide the failure type and the right node to optimize the corresponding mobility parameters. Likewise, for the PSCell change failure, the MN can also decide the PSCell failure type, e.g., Too late PSCell change, Too early PSCell change or Triggering PSCell change to wrong PSCell, based on the time elapsed since the last PSCell change initialization until SCG failure and the possibly suitable cell(s). Based on the above analysis, the MN can identify the right S-NG-RAN node and send the SCGFailureInformation message to the initiating SN.

Proposal 2: The SN CHANGE REPORT message includes the PSCell failure type.

Besides, upon the PSCell change failure, if the MN decides to continue the DC for the UE, it is highly anticipated that the MN selects a new target PSCell based on the measurement results in the SCGFailureInformation message. This can quickly recover the SCG configuration and ensure the UE experience compared to the SN decision solution.

If we wait for the source SN to re-decide the new target PSCell, it will bring additional latency due to the delivery of the SCGFailureInformation and the SN Modification Required messages. Besides, it is a bit questionable how the source SN can know whether the new PSCell belonging to a new SN is a suitable candidate for SCG connection, e.g. whether there is an Xn connection between the MN and the new SN. In this way, the MN can also send the new suitable target PSCell decision to the initiating SN node.

In other hand, if the MN decides not to continue the DC configuration for the UE. It depends on MN implementation whether to include the new suitable target PSCell in the SN CHANGE REPORT message.

Proposal 3: The SN CHANGE REPORT message includes the new suitable target PSCell.

Finally, to retrieve more relevant information collected at the initiating SN side as part of the UE context, the MN provides the UE ID for the initiating SN to identify the UE. But in MR-DC, the MN does not know the C-RNTI of UE in the source SN. Therefore it is preferred to use the S-NG-RAN node UE X2AP ID as a reference to the UE context in the source SN.

Proposal 4: The SN CHANGE REPORT message includes the S-NG-RAN node UE X2AP ID for the initiating SN to identify the UE.

Here we show a possible flow chart in case of too early PSCell change.

# 4. Conclusion

Based on the discussion in this paper, we propose the following:

Proposal 1: The SCG CHANGE REPORT message is introduced from MN to the initiating SN to forward SCGfailureinformation.

Proposal 2: The SCG CHANGE REPORT message includes the PSCell failure type.

Proposal 3: The SCG CHANGE REPORT message includes the new suitable target PSCell.

Proposal 4: The SCG CHANGE REPORT message includes the S-NG-RAN node UE X2AP ID for the initiating SN to identify the UE.

# 5. Reference

- R3-206191, (TP for SON BLCR for 38.423) MRO for SN Change Failure, Huawei

# Annex – TP on TS 38.423

Start of the first change

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SN change report | SN CHANGE REPORT |

Start of the next change

### 8.4.x	SN change Report

#### 8.4.x.1	General

The purpose of the SN Change Report procedure is to transfer mobility related information from M-NG-RAN node to S-NG-RAN node.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.x.2-1: SN Change Report, successful operation

An M-NG-RAN node initiates the procedure by sending an SN CHANGE REPORT message to S-NG-RAN node. By sending the message M-NG-RAN node indicates to S-NG-RAN node that a SN mobility-related problem was detected.

If the SN change Report Type IE is set to "too early PSCell change" or "triggering PSCell change to wrong PSCell", then the M-NG-RAN node indicates to S-NG-RAN node that, following a PSCell change from a source PSCell to a target PSCell, an SCG failure occurred and the UE established connection either at the original PSCell (too early PSCell change), or at another PSCell (triggering PSCell change to wrong PSCell). If the SN change Report Type IE is set to "too late PSCell change", then the M-NG-RAN node indicates to S-NG-RAN node that an SCG failure occurred and there was no recent PSCell change before the SCG failure.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

Start of the next change

#### 9.1.3.x	SN CHANGE REPORT

This message is sent by the M-NG-RAN node to report a PSCell change failure event to S-NG-RAN node.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| CHOICE PSCell change Report Type | M |  |  |  |  |  |
| > too early PSCell change |  |  |  |  |  |  |
| >>Source PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PSCell for PSCell change procedure | YES | ignore |
| >>Failure PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of PSCell where the failure occurred | YES | ignore |
| > too late PSCell change |  |  |  |  |  |  |
| >>Failure PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of PSCell where the failure occured | YES | ignore |
| >>Suitable PSCell CGI | O |  | NR CGI 9.2.2.7 | CGI of the new suitable PSCell | YES | ignore |
| > triggering PSCell change to wrong PSCell |  |  |  |  |  |  |
| >>Source PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of source PSCell for PSCell change procedure | YES | ignore |
| >>Failure PSCell CGI | M |  | NR CGI 9.2.2.7 | CGI of PSCell where the failure occurred | YES | ignore |
| >>Suitable PSCell CGI | O |  | NR CGI 9.2.2.7 | CGI of the new suitable PSCell | YES | ignore |
| UE SCGfailureinformation Report Container | M |  | OCTET STRING | The received SCGFailureInformation or SCGFailureInformationEUTRA message defined in TS 38.331 [10] from UE. | YES | ignore |

End of the last change