3GPP TSG-RAN3 #112-e	R3-212259

17th May – 27th May 2021

Online

Agenda Item:	10.2.1.6

Source:	Ericsson

Title:	(TP for SON BL CR for TS 38.423) MRO for SN Change Failure

Document for:	Discussion, TP

# Introduction

In the previous meeting RAN3 continued discussing MRO for SN Change Failure and further agreements were made, among them an agreement on the signalling needed in order to forward SCGfailureinformation from MN to the initiating SN. In the following we will further analyse this issue and bring forward our proposals.

# Discussion

In the previous meeting a detailed discussion on MRO for SN Change Failure took place and the following agreements were made.

MRO issues for PSCell change failure are defined as below:

-	Too late PSCell change: an SCG failure occurs after the UE has stayed for a long period of time in the PSCell; a suitable different PSCell is found based on the measurements reported from the UE.

-	Too early PSCell change: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; source PSCell is still the suitable PSCell based on the measurements reported from the UE.

-	Triggering PSCell change to wrong PSCell: an SCG failure occurs shortly after a successful PSCell change from a source PSCell to a target PSCell or a PSCell change failure occurs during the PSCell change procedure; a suitable PSCell different with source PSCell or target PSCell is found based on the measurements reported from the UE.

MN performs initial analysis to identify the node that caused the failure. The node that caused the failure performs root cause analysis.

Define new message from MN to the initiating SN to forward SCGfailureinformation.

Additional information related to SCG failure reported from UE may be beneficial; details FFS.

As we read from the agreements, the MN will perform the initial analysis in order to identify the mode that caused the failure and if the node in question is SN then it is SN that will perform root cause analysis. This creates therefore the need to forward the SCGfailureinformation to the source SN in case of SN initiated SN change. Again, from the above agreements we see that it has been decided to use a new message for this purpose.

Based on the above we propose to define a new message, named SCG Failure Information Report, that will be used to forward the SCGfailureinformation from the MN to the initiating SN.

Proposal 1: we propose to define a new message, named SCG Failure Information Report, that will be used to forward the SCGfailureinformation from the MN to the initiating SN.

Since this issue is also discussed in RAN2, we would defer the information included in this new message till RAN2 decides on it.

Proposal 2: It is proposed to leave further details on the new SCG Failure Information Report message as FFS, pending decisions in RAN2.

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1: we propose to define a new message SCG failure information report that will forward the SCGfailureinformation from the MN to the initiating SN.

Proposal 2: It is proposed to leave further details on the new SCG Failure Information Report message as FFS, pending decisions in RAN2.

- TP reflecting the proposals above are provided in the Annex.

# References

RP-201281, “New WID on enhancement of data collection for SON/MDT in NR”

TR 37.816, 3GPP WG RAN3, Ljubljana, SI, August 2019.

# Annex

TP for SON BL CR for TS 38.423

============ Start of next change ==============

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |
| SCG Failure Information Report | SCG FAILURE INFORMATION REPORT |

============ Start of next change ==============

### 8.4.x SCG Failure Information Report

#### 8.4.x.1	General

The purpose of the SCG Failure Information Report procedure is to transfer SCGfailureinformation between NG-RAN nodes.

The procedure uses non UE-associated signalling.

#### 8.4.x.2	Successful Operation

Figure 8.4.8.2-1: Handover Report, successful operation

NG-RAN node1 initiates the procedure by sending the SCG FAILURE INFORMATION REPORT message to NG-RAN node2 following the detection of an SCG failure.

#### 8.4.x.3	Unsuccessful Operation

Not applicable.

#### 8.4.x.4	Abnormal Conditions

Void.

============ Start of next change ==============

#### 9.1.3.x	SCG Failure Information Report

This message is sent by NG-RAN node2 to indicate reception of an SCG RLF Report.

Direction: NG-RAN node1  NG-RAN node2.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | ignore |
| UE S-RLF Report Container | M |  | OCTET STRING | FFS, pending RAN2 agreements | YES | reject |

============ End of changes ==============