3GPP TSG-RAN WG3 Meeting #112-e	R3-212352

# Online, 17 – 28 May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.455 | CR | 0037 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of NRPPa NR Positioning enhancements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introducing support of on-demand PRS transmission in NRPPa |  |  |  |  |  |  |  |  |
| Summary of change: |  | We propose three examples for enabling provision of PRS beam utilization in NRPPa to help reducing the PRS overhead in gNB. - Option 1: enhancing the Positioning Information exchange procedure by introducing a Requested DL PRS Transmission Characteristics IE in the POSITIONING INFORMATION REQUEST message and DL-PRS Information IE in the response and update messages. - Option 2: defining new class1 PRS Activity Configuration Exchange procedure initiated by LMF. - Option 3: defining new class1 PRS Activity Report Exchange procedure initiated by gNB. Providing an initial set of relevant metrics for the PRS beam utilization report from LMF including: - The list of PRS resources set that can be configured by the TRPs hosted in the NG-RAN node. - The resource ID for each resource set - For each PRS resource ID: - The number of UEs that have detected RSRP values with good quality - Their average RSRP/RSRQ - FFS The gNB can consider the received PRS beam activity report to optimize its TRPs beam utilization. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support of network initiated on-demand PRS transmission in NRPPa. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | TBD |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.473 CR 0769 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## ===================START OF CHANGES FOR OPTION 1=====================

### 8.2.6	Positioning Information Exchange

#### 8.2.6.1	General

The Positioning Information Exchange procedure is initiated by the LMF to request to the NG-RAN node positioning information for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.6.2	Successful Operation

Figure 8.2.6.2-1: Positioning Information Exchange procedure, successful operation

The LMF initiates the procedure by sending a POSITIONING INFORMATION REQUEST message to the NG-RAN node.

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring SRS transmissions for the UE, and it shall include the SRS Configuration IE and the SFN Initialisation Time IE in the POSITIONING INFORMATION RESPONSE message.

If the Requested PRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message, the NG-RAN node may take this information into account when configuring PRS transmissions for the UE, and it shall include the DL-PRS Information IE in the POSITIONING INFORMATION RESPONSE message.

#### 8.2.6.3	Unsuccessful Operation

Figure 8.2.6.3-1: Positioning Information Exchange procedure, unsuccessful operation

If the Requested SRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message and the NG-RAN node is unable to configure any SRS transmissions for the UE, it shall respond with a POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

If the Requested PRS Transmission Characteristics IE is included in the POSITIONING INFORMATION REQUEST message and the NG-RAN node is unable to configure any PRS transmissions for the UE according to the suggested information from LMF, it shall respond with a POSITIONING INFORMATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a POSITIONING INFORMATION FAILURE message with an appropriate cause value.

#### 8.2.6.4	Abnormal Conditions

Void.

### 8.2.7	Positioning Information Update

#### 8.2.7.1	General

The Positioning Information Update procedure is initiated by the NG-RAN node to indicate to the LMF that a change has occurred in the SRS or PRS configuration. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.7.2	Successful Operation

Figure 8.2.7.2-1: Positioning Information Update procedure, successful operation

The NG-RAN node initiates the procedure by sending a POSITIONING INFORMATION UPDATE message to the LMF. If the SRS Configuration IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated SRS Configuration for the UE. If the SFN Initialisation Time IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the SFN Initialisation Time associated to the SRS Configuration.

If the DL-PRS Information IE is included in the POSITIONING INFORMATION UPDATE message, the LMF shall consider this information as the updated PRS Configuration for the UE

#### 8.2.7.3	Unsuccessful Operation

Not Applicable.

#### 8.2.7.4	Abnormal Conditions

Void.

NEXT CHANGE

#### 9.1.1.10	POSITIONING INFORMATION REQUEST

This message is sent by LMF to request positioning information.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested SRS Transmission Characteristics | O |  | 9.2.27 |  | YES | ignore |
| Requested DL PRS Transmission Characteristics | O |  | 9.2.X |  | YES | ignore |

#### 9.1.1.11	POSITIONING INFORMATION RESPONSE

This message is sent by NG-RAN node to provide positioning information.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | 9.2.36 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |
| DL-PRS Information | O |  | 9.2.Y |  | YES | ignore |

#### 9.1.1.12	POSITIONING INFORMATION FAILURE

This message is sent by NG-RAN node to indicate that the positioning information cannot be provided.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

#### 9.1.1.13	POSITIONING INFORMATION UPDATE

This message is sent by NG-RAN node to indicate that a change in the SRS configuration has occurred.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | ignore |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| SRS Configuration | O |  | 9.2.28 |  | YES | ignore |
| SFN Initialisation Time | O |  | 9.2.36 |  | YES | ignore |
| DL-PRS Information | O |  | 9.2.Y |  | YES | ignore |

NEXT CHANGE

### 9.2.x Requested DL PRS Transmission Characteristics (FFS)

This IE contains the PRS beam utilization know by the LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PRS Resource Set List |  |  | 1.. <maxnoPRSResourceSet> |  |
| > PRS Resource Set ID | M |  | INTEGER(0...7) |  |
| > PRS Resource List |  |  | 1.. <maxnoPRSResource> |  |
| >> PRS resource ID | M |  | INTEGER(0..63) |  |
| >> UE statistics | M |  |  |  |
| >>> Number of UEs | M |  | INTEGER(0..N) |  |
| >>> Number of UEs above RSRP Threshold | M |  | INTEGER(0..N) | The number of UEs that have reported RSRP of good quality |
| >>> average RSRP | O |  | INTEGER (0..127) |  |
| >>> average RSRQ | O |  | INTEGER (0..127) |  |
| Other information FFS |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

### 9.2.Y DL-PRS Information (FFS)

This IE contains the updated PRS configuration for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL PRS Cells |  | 1.. <maxCellinRANnode> |  | Served cells/TRPs that broadcast PRS. May be used to signal multiple PRS configurations per cell/TRPs |
| >TRP ID | O |  | 9.2.24 | The TRP ID of the TRP included in the cell identified by the Cell ID |
| >Cell ID | M |  | NR CGI 9.2.9 |  |
| >DL PRS Cell Configuration | M |  | 9.2.44 |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinRANnode | Maximum no. cells that can be served by a RAN Node. Value is 16384. |

END OF CHANGES

## ===================START OF CHANGES FOR OPTION 2=====================

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation e) PRS Activity Configuration Exchange |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Activity Configuration Exchange | PRS CONFIGURATION REQUEST | PRS CONFIGURATION RESPONSE | PRS CONFIGURATION FAILURE |

NEXT CHANGE

### 8.2.X	PRS Activity Configuration Exchange

#### 8.2.X.1	General

The PRS Activity Configuration Exchange procedure is initiated by the LMF to request the NG-RAN node to update its PRS configuration for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: PRS Configuration Exchange procedure, successful operation

The LMF initiates the procedure by sending a PRS CONFIGURATION REQUEST message to the NG-RAN node.

If the Requested PRS Transmission Characteristics IE is included in the PRS CONFIGURATION REQUEST message, the NG-RAN node may take this information into account when configuring PRS transmissions for the UE, and it shall include the DL-PRS Information IE in the PRS CONFIGURATION ACKNWLEDGE message.

#### 8.2.X.3	Unsuccessful Operation

Figure 8.2.X.3-1: PRS Configuration Exchange procedure, unsuccessful operation

If the Requested PRS Transmission Characteristics IE is included in the PRS CONFIGURATION REQUEST message and the NG-RAN node is unable to configure any PRS transmissions for the UE, it shall respond with a PRS CONFIGURATION FAILURE message. If a handover of the target UE has been triggered, the NG-RAN node shall send a PRS CONFIGURATION FAILURE message with an appropriate cause value.

#### 8.2.X.4	Abnormal Conditions

Void.

NEXT CHANGE

#### 9.1.1.a1	PRS CONFIGURATION REQUEST

This message is sent by LMF to request updating the PRS configuration for the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Requested DL PRS Transmission Characteristics | O |  | 9.2.X |  | YES | ignore |

#### 9.1.1.a2	PRS CONFIGURATION ACKNOWLEDGE

This message is sent by NG-RAN node to acknowledge updating the PRS configuration.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| DL-PRS Information | O |  | 9.2.Y |  | YES | ignore |

#### 9.1.1.a3	PRS CONFIGURATION FAILURE

This message is sent by NG-RAN node to indicate that it cannot update any PRS configuration as requested.

Direction: NG-RAN node  LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

NEXT CHANGE

### 9.2.x Requested DL PRS Transmission Characteristics (FFS)

This IE contains the PRS beam utilization know by the LMF.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| PRS Resource Set List |  |  | 1.. <maxnoPRSResourceSet> |  |
| > PRS Resource Set ID | M |  | INTEGER(0...7) |  |
| > PRS Resource List |  |  | 1.. <maxnoPRSResource> |  |
| >> PRS resource ID | M |  | INTEGER(0..63) |  |
| >> UE statistics | M |  |  |  |
| >>> Number of UEs | M |  | INTEGER(0..N) |  |
| >>> Number of UEs above RSRP Threshold | M |  | INTEGER(0..N) | The number of UEs that have reported RSRP of good quality |
| >>> average RSRP | O |  | INTEGER (0..127) |  |
| >>> average RSRQ | O |  | INTEGER (0..127) |  |
| Other information FFS |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

### 9.2.Y DL-PRS Information (FFS)

This IE contains the updated PRS configuration for the UE.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| DL PRS Cells |  | 0.. <maxCellinRANnode> |  | Served cells/TRPs that broadcast PRS. May be used to signal multiple PRS configurations per cell/TRPs |
| >TRP ID | O |  | 9.2.24 | The TRP ID of the TRP included in the cell identified by the Cell ID |
| >Cell ID | M |  | NR CGI 9.2.9 |  |
| >DL PRS Cell Configuration | M |  | 9.2.44 |  |

| Range bound | Explanation |
| --- | --- |
| maxCellinRANnode | Maximum no. cells that can be served by a RAN Node. Value is 16384. |

END OF CHANGES

## ===================START OF CHANGES FOR OPTION 3=====================

# 7	Functions of NRPPa

The NRPPa protocol provides the following functions:

-	E-CID Location Information Transfer. This function allows the NG-RAN node to exchange location information with LMF for the purpose of E-CID positioning and NR E-CID positioning.

-	OTDOA Information Transfer. This function allows the NG-RAN node to exchange information with the LMF for the purpose of OTDOA positioning.

-	Reporting of General Error Situations. This function allows reporting of general error situations, for which function specific error messages have not been defined.

-	Assistance Information Transfer. This function allows the LMF to exchange information with the NG-RAN node for the purpose of assistance information broadcasting.

-	Positioning Information Transfer. This function allows the NG-RAN node to exchange positioning information with the LMF for the purpose of positioning.

-	Measurement Information Transfer. This function allows the LMF to exchange measurement information with the NG-RAN node for the purpose of positioning.

-	TRP Information Transfer. This function allows an LMF to obtain TRP related information from an NG-RAN node.

The mapping between the above functions and NRPPa EPs is shown in the table below.

Table 7-1: Mapping between NRPPa functions and NRPPa EPs

| Function | Elementary Procedure(s) |
| --- | --- |
| E-CID Location Information Transfer | a) E-CID Measurement Initiation b) E-CID Measurement Failure Indication c) E-CID Measurement Report d) E-CID Measurement Termination |
| OTDOA Information Transfer | OTDOA Information Exchange |
| Assistance Information Transfer | a) Assistance Information Control b) Assistance Information Feedback |
| Reporting of General Error Situations | Error Indication |
| Positioning Information Transfer | a) Positioning Information Exchange b) Positioning Information Update c) Positioning Activation d) Positioning Deactivation e) PRS Activity Report Exchange |
| TRP Information Transfer | TRP Information Exchange |
| Measurement Information Transfer | a) Measurement b) Measurement Update c) Measurement Report d) Measurement Abort e) Measurement Failure Indication |

# 8	NRPPa procedures

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| E-CID Measurement Initiation | E-CID MEASUREMENT INITIATION REQUEST | E-CID MEASUREMENT INITIATION RESPONSE | E-CID MEASUREMENT INITIATION FAILURE |
| OTDOA Information Exchange | OTDOA INFORMATION REQUEST | OTDOA INFORMATION RESPONSE | OTDOA INFORMATION FAILURE |
| Positioning Information Exchange | POSITIONING INFORMATION REQUEST | POSITIONING INFORMATION RESPONSE | POSITIONING INFORMATION FAILURE |
| TRP Information Exchange | TRP INFORMATION REQUEST | TRP INFORMATION RESPONSE | TRP INFORMATION FAILURE |
| Measurement | MEASUREMENT REQUEST | MEASUREMENT RESPONSE | MEASUREMENT FAILURE |
| Positioning Activation | POSITIONING ACTIVATION REQUEST | POSITIONING ACTIVATION RESPONSE | POSITIONING ACTIVATION FAILURE |
| PRS Activity Report Exchange | PRS ACTIVITY REPORT REQUEST | PRS ACTIVITY REPORT RESPONSE | PRS ACTIVITY REPORT FAILURE |

NEXT CHANGE

### 8.2.X	PRS Activity Report Exchange

#### 8.2.X.1	General

The PRS Activity Report Exchange procedure is initiated by the NG-RAN node to request the LMF of any PRS activity report know for the UE. This procedure applies only if the NG-RAN node is a gNB.

#### 8.2.X.2	Successful Operation

Figure 8.2.X.2-1: PRS Activity Report Exchange procedure, successful operation

The NG-RAN node initiates the procedure by sending a PRS ACTIVITY REPORT REQUEST message to the LMF.

#### 8.2.X.3	Unsuccessful Operation

Figure 8.2.X.3-1: PRS Activity Report Exchange procedure, unsuccessful operation

If the LMF is unable to provide any PRS Activity report to the gNB, it shall respond with a PRS ACTIVITY REPORT FAILURE message.

#### 8.2.X.4	Abnormal Conditions

Void.

NEXT CHANGE

#### 9.1.1.a1	PRS ACTIVITY REPORT REQUEST

This message is sent from the NG-RAN node to request PRS Activity Report from the LMF.

Direction: NG-RAN node  LMF

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  |
| NRPPa Transaction ID | M |  | 9.2.4 |  |

#### 9.1.1.a2	PRS ACTIVITY REPORT RESPONSE

This message is sent from the LMF to inform the NG-RAN node of the PRS Activity report it currently knows.

Direction: LMF NG-RAN node

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  |
| NRPPa Transaction ID | M |  | 9.2.4 |  |
| PRS Resource Set List |  |  | 1.. <maxnoPRSResourceSet> |  |
| > PRS Resource Set ID | M |  | INTEGER(0...7) |  |
| > PRS Resource List |  |  | 1.. <maxnoPRSResource> |  |
| >> PRS resource ID | M |  | INTEGER(0..63) |  |
| >> UE statistics | M |  |  |  |
| >>> Number of UEs | M |  | INTEGER(0..N) |  |
| >>> Number of UEs above RSRP Threshold | M |  | INTEGER(0..N) | The number of UEs that have reported RSRP of good quality |
| >>> average RSRP | O |  | INTEGER (0..127) |  |
| >>> average RSRQ | O |  | INTEGER (0..127) |  |
| Other information FFS |  |  |  |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofPRSresourceSet | Maximum no of PRS resources set. Value is 8. |
| maxnoofPRSresource | Maximum no of PRS resources per PRS resource set. Value is 64. |

#### 9.1.1.a3	PRS ACTIVITY REPORT FAILURE

This message is sent by LMF to indicate that it cannot provide any PRS configuration for the UE.

Direction: LMF  NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3 |  | YES | reject |
| NRPPa Transaction ID | M |  | 9.2.4 |  | - |  |
| Cause | M |  | 9.2.1 |  | YES | ignore |
| Criticality Diagnostics | O |  | 9.2.2 |  | YES | ignore |

END OF CHANGES