3GPP TSG-RAN3 Meeting #112-e	R3-212403

# E-meeting, 17th May - 28th May, 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.423 | CR | 0625 | Rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.423 on including survival time in TSC Assistance Information enhancements. |
| --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IIOT_URLLC_enh-Core |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To include the survival time in the TSC Assistance Information. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To introduce the Survival Time IE into the TSC Assistance Information IE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The survival time can’t be supported in the NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.115, 9.2.3.1xx, 9.3.5, 9.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS 38.413 CR 0613 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the 1st change

9.2.3.115	TSC Assistance Information

This IE provides the TSC assistance information for a TSC QoS flow in the uplink or downlink (see TS 23.501 [9]).

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | 9.3.1.116 | Periodicity as specified in TS 23.501 [7]. |
| Burst Arrival Time | O |  | 9.3.1.117 | Burst Arrival Time as specified in TS 23.501 [7]. |
| Survival Time | O |  | 9.3.1.1xx | Survival Time as specified in TS 23.501 |

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

9.2.3.1xx	Survival Time

This IE indicates the Survial Time as defined in TS 23.501 [21].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0..1920000, …) | Survival time expressed in units of 1 us. |

End of the 1st change

Start of the 2nd change

9.3.5	Information Element definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

id-SFN-Offset,

id-SurvivalTime,

maxEARFCN,

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

SupportedSULBandItem ::= SEQUENCE {

sulBandItem					SUL-FrequencyBand,

iE-Extensions				ProtocolExtensionContainer { {SupportedSULBandItem-ExtIEs} } OPTIONAL,

...

}

SupportedSULBandItem-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

}

SurvivalTime ::= INTEGER (0..1920000, ...)

SymbolAllocation-in-Slot ::= CHOICE {

allDL				SymbolAllocation-in-Slot-AllDL,

allUL				SymbolAllocation-in-Slot-AllUL,

bothDLandUL			SymbolAllocation-in-Slot-BothDLandUL,

choice-extension	ProtocolIE-Single-Container { {SymbolAllocation-in-Slot-ExtIEs} }

}

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

TSCTrafficCharacteristics-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

...

}

TSCAssistanceInformation ::= SEQUENCE {

periodicity			INTEGER (0.. 640000, ...),

burstArrivalTime	OCTET STRING				OPTIONAL,

ie-Extension 			ProtocolExtensionContainer { { TSCAssistanceInformation-ExtIEs} } OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs XNAP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime			CRITICALITY ignore	EXTENSION SurvivalTIme		PRESENCE optional },

...

}

TypeOfError ::= ENUMERATED {

not-understood,

missing,

...

}

End of the 2nd change

Start of the 3rd change

9.3.7	Constant definitions

***************************************************************************************************************************

Skip unchanged part

***************************************************************************************************************************

id-QoSMonitoringDisabled																				ProtocolIE-ID ::= 243

id-ExtendedUEIdentityIndexValue																		ProtocolIE-ID ::= 244

id-SurvivalTIme																				ProtocolIE-ID ::= xxx

END

-- ASN1STOP

End of the 3rd change