3GPP TSG-RAN WG3 Meeting #112-e                                                                            R3-212946

17-27 May 2021, E-meeting

Title:	LS to RAN2 on reduction of service interruption during intra-donor IAB-node migration

Response to:

Release:	Rel-17

Work Item:	NR_IAB_enh-Core

Source:	RAN3

To:	RAN2

Cc:

Contact person:

- Name:			Milap Majmundar

- E-mail Address:	milap.majmundar@att.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN3 is currently evaluating the following two solutions for reduction of service interruption during INTRA-donor IAB-node migration. In both solutions, the transfer of RRCReconfiguration for TNL migration of a descendent IAB node occurs over the source path. The goal of these solutions is to initiate execution of RRCReconfiguration by the descendent nodes as soon as the target path has become available.

Solution 1:

The RRCReconfiguration message for TNL migration of a descendent node IAB-MT is withheld by this descendant node’s parent IAB-DU, and it is delivered only when a condition is satisfied. The indication of buffering and conditional delivery may be provided by the IAB-donor-CU to the parent IAB-DU via an F1AP message including the RRCReconfiguration message.  The condition is set so that a sequential delivery and execution of RRCReconfigurations is created downstream.

While exact details of Solution 1 are still FFS, an example procedure is provided in Figure 1.

Figure 1. Example procedure for Solution 1 (R3-211740)

Solution 2:

The RRCReconfiguration message for TNL migration of the descendant-node IAB-MT is buffered by the descendent-node’s IAB-MT itself, and it is executed only when an indication is received from the parent IAB-DU. The indication of buffering and conditional execution may be included in the RRCReconfiguration. The condition for initiation and propagation of this indication  is set so that it causes  a sequential execution of RRCReconfigurations downstream.

While exact details of Solution 2 are still FFS, an example procedure is provided in Figure 2.

Figure 2. Example procedure for Solution 2 (R3-211740)

RAN3 would like to ask RAN2 to provide feedback regarding issues and feasibility of Solutions 1 and 2.

# 2	Actions

To: RAN2

ACTION: 	RAN3 kindly asks RAN2 to provide feedback regarding issues and feasibility on Solutions 1 and 2.

# 3	Dates of next TSG RAN3 meetings

RAN3#113e 	Aug. 16~27, 2021	online

RAN3#114e	Nov. 1~12, 2021	online