3GPP TSG-RAN WG3#113-e	R3-21

- E-meeting, 16 th  – 26th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.463 | CR | 0612 | rev | 1 | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of PDU Session level Data Forwarding Information and QoS Flow list |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Samsung, ZTE, Huawei, Nokia, Nokia Shanghai Bell, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CPUP_Split-Core |  |  |  |  |  | Date: |  |  | 2021-08-16 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the intra-system handover , it has been generally assumed that for the data forwarding aspect, a list of QoS Flows within a PDU Session may be subjected to be forwarded. For example in the XNAP: HANDOVER REQUEST ACKNOWLEDGE message the QoS Flows Accepted For Data Forwarding List IE is always included. This apply to PDU Session level data forwarding. For the gNB-CU-CP/UP aspect in the E1AP, such QoS Flow list information is not present from the source gNB-CU-CP to source gNB-CU-UP. Without the QoS Flow list information, the source gNB-CU-UP will not be able to know which QoS Flow is accepted for data forwarding by the target. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Rev 0: Add the QoS Flow List in the PDU Session Resource To Modify List IE. Rev 1:after the discussion, the correction is to add QoS flow ID list under the 9.3.2.6 Data Forwarding Information, which is referred by the PDU Session Data Forwarding Information IE. Add procedure text for optional IE when it is included. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has an impact under protocol and functional point of view. This CR has isolated impact because it only impacts on the intra-system handover the CP-UP aspect the signalling from source CP to source UP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without the QoS Flow list information, the source gNB-CU-UP will not be able to know which QoS Flow is accepted for data forwarding by the target. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.3.2.2, 9.3.2.6, 9.4.5, 9.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev 0: Add the QoS Flow List in the PDU Session Resource To Modify List IE. Rev1: after the discussion, the correction is to add QoS flow ID list under the 9.3.2.6 Data Forwarding Information, which is referred by the PDU Session Data Forwarding Information IE |  |  |  |  |  |  |  |  |

### 8.3.2	Bearer Context Modification (gNB-CU-CP initiated)

#### 8.3.2.1	General

The purpose of the Bearer Context Modification procedure is to allow the gNB-CU-CP to modify a bearer context in the gNB-CU-UP. The procedure uses UE-associated signalling.

#### 8.3.2.2	Successful Operation

Figure 8.3.2.2-1: Bearer Context Modification procedure: Successful Operation.

<Unchanged Text Omited>

If the Data Forwarding Information Request IE, PDU Session Data Forwarding Information Request IE or the DRB Data Forwarding Information Request IE are included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall include the requested forwarding information in the Data Forwarding Information Response IE, PDU Session Data Forwarding Information Response IE or the DRB Data Forwarding Information Response IE in the BEARER CONTEXT MODIFICATION RESPONSE message.

If the PDU Session Data Forwarding Information IE is included in the BEARER CONTEXT MODIFICATION REQUEST message, the gNB-CU-UP shall, if supported, consider that data forwarding is applicable for the indicated QoS flows for the concerned PDU session.

<Unchanged Text Omited>

#### 9.3.2.6	Data Forwarding Information

This IE provides the data forwarding information when performing handover or data offloading.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| UL Data Forwarding | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| DL Data Forwarding | O |  | UP Transport Layer Information 9.3.2.1 |  | - | - |
| Data Forwarding to NG-RAN QoS Flow Information List |  | 0..1 |  | Providing QoS flows accepted for data forwarding to the source gNB-CU-UP. | Yes | Ignore |
| >Data Forwarding to NG-RAN QoS Flow Information List Item |  | 1..<maxnoofQoSflows> |  |  | - | - |
| >>QoS Flow Identifier | M |  | QoS Flow Identifier 9.3.1.24 |  | - | - |

Skip unchanged part

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

- -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

id-DRB-QoS,

id-endpoint-IP-Address-and-Port,

id-NetworkInstance,

id-QoSFlowMappingIndication,

id-TNLAssociationTransportLayerAddressgNBCUUP,

id-Cause,

id-QoSMonitoringRequest,

id-QosMonitoringReportingFrequency,

id-QoSMonitoringDisabled,

id-PDCP-StatusReportIndication,

id-RedundantCommonNetworkInstance,

id-redundant-nG-UL-UP-TNL-Information,

id-redundant-nG-DL-UP-TNL-Information,

id-RedundantQosFlowIndicator,

id-TSCTrafficCharacteristics,

id-ExtendedPacketDelayBudget,

id-CNPacketDelayBudgetDownlink,

id-CNPacketDelayBudgetUplink,

id-AdditionalPDCPduplicationInformation,

id-RedundantPDUSessionInformation,

id-RedundantPDUSessionInformation-used,

id-QoS-Mapping-Information,

id-MDTConfiguration,

id-TraceCollectionEntityURI,

id-EHC-Parameters,

id-DAPSRequestInfo,

id-EarlyForwardingCOUNTReq,

id-EarlyForwardingCOUNTInfo,

id-AlternativeQoSParaSetList,

id-MCG-OfferedGBRQoSFlowInfo,

id-Number-of-tunnels,

id-DataForwardingtoE-UTRANInformationList,

id-DataForwardingtoNG-RANQoSFlowInformationList,

maxnoofQoSParaSets,

maxnoofErrors,

maxnoofSliceItems,

maxnoofEUTRANQOSParameters,

maxnoofNGRANQOSParameters,

maxnoofDRBs,

maxnoofPDUSessionResource,

maxnoofQoSFlows,

maxnoofUPParameters,

maxnoofCellGroups,

maxnooftimeperiods,

maxnoofNRCGI,

maxnoofTLAs,

maxnoofGTPTLAs,

maxnoofSPLMNs,

maxnoofMDTPLMNs,

maxnoofExtSliceItems,

maxnoofDataForwardingTunneltoE-UTRAN,

maxnoofExtNRCGI

FROM E1AP-Constants

Criticality,

ProcedureCode,

ProtocolIE-ID,

TriggeringMessage

FROM E1AP-CommonDataTypes

ProtocolExtensionContainer{},

ProtocolIE-SingleContainer{},

E1AP-PROTOCOL-EXTENSION,

E1AP-PROTOCOL-IES

FROM E1AP-Containers;

Skip unchanged part

- -- D

DAPSRequestInfo ::= SEQUENCE {

dapsIndicator				ENUMERATED {daps-HO-required, ...},

iE-Extensions				ProtocolExtensionContainer { {DAPSRequestInfo-ExtIEs} } OPTIONAL,

...

}

DAPSRequestInfo-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information-Request	::=	SEQUENCE {

data-Forwarding-Request					Data-Forwarding-Request,

qoS-Flows-Forwarded-On-Fwd-Tunnels	QoS-Flow-Mapping-List			OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-Request-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-Request-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Forwarding-Information	::=	SEQUENCE {

uL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

dL-Data-Forwarding						UP-TNL-Information		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { Data-Forwarding-Information-ExtIEs } }	OPTIONAL,

...

}

Data-Forwarding-Information-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-DataForwardingtoNG-RANQoSFlowInformationList	CRITICALITY ignore	EXTENSION DataForwardingtoNG-RANQoSFlowInformationList	PRESENCE optional}	...

}

Data-Forwarding-Request ::= ENUMERATED	{

uL,

dL,

both,

...

}

DataForwardingtoE-UTRANInformationList ::= SEQUENCE (SIZE(1.. maxnoofDataForwardingTunneltoE-UTRAN)) OF DataForwardingtoE-UTRANInformationListItem

DataForwardingtoE-UTRANInformationListItem ::= SEQUENCE {

data-forwarding-tunnel-information							UP-TNL-Information,

qoS-Flows-to-be-forwarded-List						QoS-Flows-to-be-forwarded-List,

iE-Extensions		ProtocolExtensionContainer { { DataForwardingtoE-UTRANInformationListItem-ExtIEs} }	OPTIONAL,

...

}

DataForwardingtoE-UTRANInformationListItem-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Data-Usage-per-PDU-Session-Report ::= SEQUENCE {

secondaryRATType			ENUMERATED {nR, e-UTRA, ...},

pDU-session-Timed-Report-List			SEQUENCE (SIZE(1..maxnooftimeperiods)) OF MRDC-Data-Usage-Report-Item,

iE-Extensions				ProtocolExtensionContainer { { Data-Usage-per-PDU-Session-Report-ExtIEs} } OPTIONAL,

...

}

Data-Usage-per-PDU-Session-Report-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

Skip unchanged part

QoS-Flow-Identifier	::=	INTEGER (0..63)

QoS-Flow-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Item

QoS-Flow-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSFlowMappingIndication		CRITICALITY ignore	EXTENSION QoS-Flow-Mapping-Indication		PRESENCE optional},

...

}

QoS-Flow-Failed-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Failed-Item

QoS-Flow-Failed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

cause									Cause,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Failed-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Failed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-Mapping-Item

QoS-Flow-Mapping-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowMappingIndication						QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-Mapping-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-Mapping-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flow-Mapping-Indication ::= ENUMERATED {ul, dl, ...}

QoS-Parameters-Support-List	::= SEQUENCE {

eUTRAN-QoS-Support-List			EUTRAN-QoS-Support-List			OPTIONAL,

nG-RAN-QoS-Support-List			NG-RAN-QoS-Support-List			OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { QoS-Parameters-Support-List-ItemExtIEs} } OPTIONAL,

...

}

QoS-Parameters-Support-List-ItemExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoSPriorityLevel ::= INTEGER (0..127, ...)

QoS-Flow-QoS-Parameter-List	::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flow-QoS-Parameter-Item

QoS-Flow-QoS-Parameter-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoSFlowLevelQoSParameters				QoSFlowLevelQoSParameters,

qoSFlowMappingIndication				QoS-Flow-Mapping-Indication		OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ { QoS-Flow-QoS-Parameter-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flow-QoS-Parameter-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

{ID id-RedundantQosFlowIndicator		CRITICALITY ignore 	EXTENSION RedundantQoSFlowIndicator			PRESENCE optional}|

{ID id-TSCTrafficCharacteristics 		CRITICALITY ignore 	EXTENSION TSCTrafficCharacteristics 			PRESENCE optional},

...

}

QoSFlowLevelQoSParameters	::= SEQUENCE {

qoS-Characteristics						QoS-Characteristics,

nGRANallocationRetentionPriority		NGRANAllocationAndRetentionPriority,

gBR-QoS-Flow-Information				GBR-QoSFlowInformation						OPTIONAL,

reflective-QoS-Attribute				ENUMERATED {subject-to, ...}				OPTIONAL,

additional-QoS-Information				ENUMERATED {more-likely, ...}				OPTIONAL,

paging-Policy-Indicator					INTEGER	(1..8,	...)						OPTIONAL,

reflective-QoS-Indicator				ENUMERATED {enabled, ...}					OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoSFlowLevelQoSParameters-ExtIEs } }	OPTIONAL

}

QoSFlowLevelQoSParameters-ExtIEs 	E1AP-PROTOCOL-EXTENSION ::= {

{ID id-QoSMonitoringRequest				CRITICALITY ignore	EXTENSION QosMonitoringRequest					PRESENCE optional}|

{ID id-MCG-OfferedGBRQoSFlowInfo		CRITICALITY ignore	EXTENSION GBR-QoSFlowInformation				PRESENCE optional}|

{ID id-QosMonitoringReportingFrequency	CRITICALITY ignore	EXTENSION QosMonitoringReportingFrequency	PRESENCE optional}|

{ID id-QoSMonitoringDisabled			CRITICALITY ignore	EXTENSION QosMonitoringDisabled					PRESENCE optional},

...

}

QosMonitoringRequest ::= ENUMERATED {ul, dl, both}

QosMonitoringReportingFrequency ::= INTEGER (1..1800, ...)

QosMonitoringDisabled ::= ENUMERATED {true, ...}

QoS-Flow-Removed-Item	::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

qoS-Flow-Released-In-Session			ENUMERATED {released-in-session, not-released-in-session, ...}			OPTIONAL,

qoS-Flow-Accumulated-Session-Time		OCTET STRING (SIZE(5))														OPTIONAL,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flow-Removed-Item-ExtIEs } }		OPTIONAL,

...

}

QoS-Flow-Removed-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Flows-to-be-forwarded-List ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF QoS-Flows-to-be-forwarded-Item

QoS-Flows-to-be-forwarded-Item ::=	SEQUENCE {

qoS-Flow-Identifier						QoS-Flow-Identifier,

iE-Extensions							ProtocolExtensionContainer { { QoS-Flows-to-be-forwarded-Item-ExtIEs } }	OPTIONAL,

...

}

QoS-Flows-to-be-forwarded-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

QoS-Mapping-Information ::= SEQUENCE {

dscp							BIT STRING (SIZE(6))		OPTIONAL,

flow-label						BIT STRING (SIZE(20))	OPTIONAL,

...

}

DataForwardingtoNG-RANQoSFlowInformationList		::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF DataForwardingtoNG-RANQoSFlowInformationList-Item

DataForwardingtoNG-RANQoSFlowInformationList-Item	::=	SEQUENCE {

qoS-Flow-Identifier					QoS-Flow-Identifier,

iE-Extensions						ProtocolExtensionContainer	{ { DataForwardingtoNG-RANQoSFlowInformationList-Item-ExtIEs} }	OPTIONAL,

...

}

DataForwardingtoNG-RANQoSFlowInformationList-Item-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

- -- R

Skip unchanged part

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

- -- Constant definitions

--

-- **************************************************************

E1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM E1AP-CommonDataTypes;

-- **************************************************************

--

- -- Elementary Procedures

--

-- **************************************************************

id-reset														ProcedureCode ::= 0

id-errorIndication												ProcedureCode ::= 1

id-privateMessage												ProcedureCode ::= 2

id-gNB-CU-UP-E1Setup											ProcedureCode ::= 3

id-gNB-CU-CP-E1Setup											ProcedureCode ::= 4

id-gNB-CU-UP-ConfigurationUpdate								ProcedureCode ::= 5

id-gNB-CU-CP-ConfigurationUpdate								ProcedureCode ::= 6

id-e1Release													ProcedureCode ::= 7

id-bearerContextSetup											ProcedureCode ::= 8

id-bearerContextModification									ProcedureCode ::= 9

id-bearerContextModificationRequired							ProcedureCode ::= 10

id-bearerContextRelease											ProcedureCode ::= 11

id-bearerContextReleaseRequest									ProcedureCode ::= 12

id-bearerContextInactivityNotification							ProcedureCode ::= 13

id-dLDataNotification											ProcedureCode ::= 14

id-dataUsageReport												ProcedureCode ::= 15

id-gNB-CU-UP-CounterCheck										ProcedureCode ::= 16

id-gNB-CU-UP-StatusIndication									ProcedureCode ::= 17

id-uLDataNotification											ProcedureCode ::= 18

id-mRDC-DataUsageReport											ProcedureCode ::= 19

id-TraceStart													ProcedureCode ::= 20

id-DeactivateTrace												ProcedureCode ::= 21

id-resourceStatusReportingInitiation							ProcedureCode ::= 22

id-resourceStatusReporting										ProcedureCode ::= 23

id-iAB-UPTNLAddressUpdate										ProcedureCode ::= 24

id-CellTrafficTrace												ProcedureCode ::= 25

id-earlyForwardingSNTransfer									ProcedureCode ::= 26

id-gNB-CU-CPMeasurementResultsInformation						ProcedureCode ::= 27

-- **************************************************************

--

- -- Lists

--

-- **************************************************************

maxnoofErrors								INTEGER ::= 256

maxnoofSPLMNs								INTEGER ::= 12

maxnoofSliceItems							INTEGER ::= 1024

maxnoofIndividualE1ConnectionsToReset		INTEGER ::= 65536

maxnoofEUTRANQOSParameters					INTEGER ::= 256

maxnoofNGRANQOSParameters					INTEGER ::= 256

maxnoofDRBs									INTEGER	::= 32

maxnoofNRCGI								INTEGER	::= 512

maxnoofPDUSessionResource					INTEGER	::= 256

maxnoofQoSFlows								INTEGER	::= 64

maxnoofUPParameters							INTEGER	::= 8

maxnoofCellGroups							INTEGER	::= 4

maxnooftimeperiods							INTEGER	::= 2

maxnoofTNLAssociations						INTEGER ::= 32

maxnoofTLAs									INTEGER ::= 16

maxnoofGTPTLAs								INTEGER ::= 16

maxnoofTNLAddresses							INTEGER ::= 8

maxnoofMDTPLMNs								INTEGER ::= 16

maxnoofQoSParaSets							INTEGER ::= 8

maxnoofExtSliceItems						INTEGER ::= 65535

maxnoofDataForwardingTunneltoE-UTRAN		INTEGER ::= 256

maxnoofExtNRCGI 							INTEGER	::= 16384

-- **************************************************************

--

- -- IEs

--

-- **************************************************************

id-Cause														ProtocolIE-ID ::= 0

id-CriticalityDiagnostics										ProtocolIE-ID ::= 1

id-gNB-CU-CP-UE-E1AP-ID 										ProtocolIE-ID ::= 2

id-gNB-CU-UP-UE-E1AP-ID											ProtocolIE-ID ::= 3

id-ResetType													ProtocolIE-ID ::= 4

id-UE-associatedLogicalE1-ConnectionItem						ProtocolIE-ID ::= 5

id-UE-associatedLogicalE1-ConnectionListResAck					ProtocolIE-ID ::= 6

id-gNB-CU-UP-ID													ProtocolIE-ID ::= 7

id-gNB-CU-UP-Name												ProtocolIE-ID ::= 8

id-gNB-CU-CP-Name												ProtocolIE-ID ::= 9

id-CNSupport													ProtocolIE-ID ::= 10

id-SupportedPLMNs												ProtocolIE-ID ::= 11

id-TimeToWait													ProtocolIE-ID ::= 12

id-SecurityInformation											ProtocolIE-ID ::= 13

id-UEDLAggregateMaximumBitRate									ProtocolIE-ID ::= 14

id-System-BearerContextSetupRequest								ProtocolIE-ID ::= 15

id-System-BearerContextSetupResponse							ProtocolIE-ID ::= 16

id-BearerContextStatusChange									ProtocolIE-ID ::= 17

id-System-BearerContextModificationRequest						ProtocolIE-ID ::= 18

id-System-BearerContextModificationResponse						ProtocolIE-ID ::= 19

id-System-BearerContextModificationConfirm						ProtocolIE-ID ::= 20

id-System-BearerContextModificationRequired						ProtocolIE-ID ::= 21

id-DRB-Status-List												ProtocolIE-ID ::= 22

id-ActivityNotificationLevel									ProtocolIE-ID ::= 23

id-ActivityInformation											ProtocolIE-ID ::= 24

id-Data-Usage-Report-List										ProtocolIE-ID ::= 25

id-New-UL-TNL-Information-Required								ProtocolIE-ID ::= 26

id-GNB-CU-CP-TNLA-To-Add-List									ProtocolIE-ID ::= 27

id-GNB-CU-CP-TNLA-To-Remove-List								ProtocolIE-ID ::= 28

id-GNB-CU-CP-TNLA-To-Update-List								ProtocolIE-ID ::= 29

id-GNB-CU-CP-TNLA-Setup-List									ProtocolIE-ID ::= 30

id-GNB-CU-CP-TNLA-Failed-To-Setup-List							ProtocolIE-ID ::= 31

id-DRB-To-Setup-List-EUTRAN										ProtocolIE-ID ::= 32

id-DRB-To-Modify-List-EUTRAN									ProtocolIE-ID ::= 33

id-DRB-To-Remove-List-EUTRAN									ProtocolIE-ID ::= 34

id-DRB-Required-To-Modify-List-EUTRAN							ProtocolIE-ID ::= 35

id-DRB-Required-To-Remove-List-EUTRAN							ProtocolIE-ID ::= 36

id-DRB-Setup-List-EUTRAN										ProtocolIE-ID ::= 37

id-DRB-Failed-List-EUTRAN										ProtocolIE-ID ::= 38

id-DRB-Modified-List-EUTRAN										ProtocolIE-ID ::= 39

id-DRB-Failed-To-Modify-List-EUTRAN								ProtocolIE-ID ::= 40

id-DRB-Confirm-Modified-List-EUTRAN								ProtocolIE-ID ::= 41

id-PDU-Session-Resource-To-Setup-List							ProtocolIE-ID ::= 42

id-PDU-Session-Resource-To-Modify-List							ProtocolIE-ID ::= 43

id-PDU-Session-Resource-To-Remove-List							ProtocolIE-ID ::= 44

id-PDU-Session-Resource-Required-To-Modify-List					ProtocolIE-ID ::= 45

id-PDU-Session-Resource-Setup-List								ProtocolIE-ID ::= 46

id-PDU-Session-Resource-Failed-List								ProtocolIE-ID ::= 47

id-PDU-Session-Resource-Modified-List							ProtocolIE-ID ::= 48

id-PDU-Session-Resource-Failed-To-Modify-List					ProtocolIE-ID ::= 49

id-PDU-Session-Resource-Confirm-Modified-List					ProtocolIE-ID ::= 50

id-DRB-To-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 51

id-DRB-Setup-Mod-List-EUTRAN									ProtocolIE-ID ::= 52

id-DRB-Failed-Mod-List-EUTRAN									ProtocolIE-ID ::= 53

id-PDU-Session-Resource-Setup-Mod-List							ProtocolIE-ID ::= 54

id-PDU-Session-Resource-Failed-Mod-List							ProtocolIE-ID ::= 55

id-PDU-Session-Resource-To-Setup-Mod-List						ProtocolIE-ID ::= 56

id-TransactionID												ProtocolIE-ID ::= 57

id-Serving-PLMN													ProtocolIE-ID ::= 58

id-UE-Inactivity-Timer											ProtocolIE-ID ::= 59

id-System-GNB-CU-UP-CounterCheckRequest							ProtocolIE-ID ::= 60

id-DRBs-Subject-To-Counter-Check-List-EUTRAN					ProtocolIE-ID ::= 61

id-DRBs-Subject-To-Counter-Check-List-NG-RAN					ProtocolIE-ID ::= 62

id-PPI															ProtocolIE-ID ::= 63

id-gNB-CU-UP-Capacity											ProtocolIE-ID ::= 64

id-GNB-CU-UP-OverloadInformation								ProtocolIE-ID ::= 65

id-UEDLMaximumIntegrityProtectedDataRate						ProtocolIE-ID ::= 66

id-PDU-Session-To-Notify-List									ProtocolIE-ID ::= 67

id-PDU-Session-Resource-Data-Usage-List							ProtocolIE-ID ::= 68

id-SNSSAI														ProtocolIE-ID ::= 69

id-DataDiscardRequired											ProtocolIE-ID ::= 70

id-OldQoSFlowMap-ULendmarkerexpected							ProtocolIE-ID ::= 71

id-DRB-QoS														ProtocolIE-ID ::= 72

id-GNB-CU-UP-TNLA-To-Remove-List								ProtocolIE-ID ::= 73

id-endpoint-IP-Address-and-Port									ProtocolIE-ID ::= 74

id-TNLAssociationTransportLayerAddressgNBCUUP					ProtocolIE-ID ::= 75

id-RANUEID														ProtocolIE-ID ::= 76

id-GNB-DU-ID													ProtocolIE-ID ::= 77

id-CommonNetworkInstance										ProtocolIE-ID ::= 78

id-NetworkInstance												ProtocolIE-ID ::= 79

id-QoSFlowMappingIndication										ProtocolIE-ID ::= 80

id-TraceActivation												ProtocolIE-ID ::= 81

id-TraceID														ProtocolIE-ID ::= 82

id-SubscriberProfileIDforRFP									ProtocolIE-ID ::= 83

id-AdditionalRRMPriorityIndex									ProtocolIE-ID ::= 84

id-RetainabilityMeasurementsInfo								ProtocolIE-ID ::= 85

id-Transport-Layer-Address-Info									ProtocolIE-ID ::= 86

id-QoSMonitoringRequest											ProtocolIE-ID ::= 87

id-PDCP-StatusReportIndication									ProtocolIE-ID ::= 88

id-gNB-CU-CP-Measurement-ID										ProtocolIE-ID ::= 89

id-gNB-CU-UP-Measurement-ID										ProtocolIE-ID ::= 90

id-RegistrationRequest											ProtocolIE-ID ::= 91

id-ReportCharacteristics										ProtocolIE-ID ::= 92

id-ReportingPeriodicity											ProtocolIE-ID ::= 93

id-TNL-AvailableCapacityIndicator								ProtocolIE-ID ::= 94

id-HW-CapacityIndicator											ProtocolIE-ID ::= 95

id-RedundantCommonNetworkInstance								ProtocolIE-ID ::= 96

id-redundant-nG-UL-UP-TNL-Information							ProtocolIE-ID ::= 97

id-redundant-nG-DL-UP-TNL-Information							ProtocolIE-ID ::= 98

id-RedundantQosFlowIndicator									ProtocolIE-ID ::= 99

id-TSCTrafficCharacteristics									ProtocolIE-ID ::= 100

id-CNPacketDelayBudgetDownlink									ProtocolIE-ID ::= 101

id-CNPacketDelayBudgetUplink									ProtocolIE-ID ::= 102

id-ExtendedPacketDelayBudget									ProtocolIE-ID ::= 103

id-AdditionalPDCPduplicationInformation							ProtocolIE-ID ::= 104

id-RedundantPDUSessionInformation								ProtocolIE-ID ::= 105

id-RedundantPDUSessionInformation-used							ProtocolIE-ID ::= 106

id-QoS-Mapping-Information										ProtocolIE-ID ::= 107

id-DLUPTNLAddressToUpdateList									ProtocolIE-ID ::= 108

id-ULUPTNLAddressToUpdateList									ProtocolIE-ID ::= 109

id-NPNSupportInfo												ProtocolIE-ID ::= 110

id-NPNContextInfo												ProtocolIE-ID ::= 111

id-MDTConfiguration												ProtocolIE-ID ::= 112

id-ManagementBasedMDTPLMNList									ProtocolIE-ID ::= 113

id-TraceCollectionEntityIPAddress								ProtocolIE-ID ::= 114

id-PrivacyIndicator												ProtocolIE-ID ::= 115

id-TraceCollectionEntityURI										ProtocolIE-ID ::= 116

id-URIaddress													ProtocolIE-ID ::= 117

id-EHC-Parameters												ProtocolIE-ID ::= 118

id-DRBs-Subject-To-Early-Forwarding-List						ProtocolIE-ID ::= 119

id-DAPSRequestInfo												ProtocolIE-ID ::= 120

id-CHOInitiation												ProtocolIE-ID ::= 121

id-EarlyForwardingCOUNTReq										ProtocolIE-ID ::= 122

id-EarlyForwardingCOUNTInfo										ProtocolIE-ID ::= 123

id-AlternativeQoSParaSetList									ProtocolIE-ID ::= 124

id-ExtendedSliceSupportList									ProtocolIE-ID ::= 125

id-MCG-OfferedGBRQoSFlowInfo									ProtocolIE-ID ::= 126

id-Number-of-tunnels											ProtocolIE-ID ::= 127

id-DRB-Measurement-Results-Information-List						ProtocolIE-ID ::= 128

id-Extended-GNB-CU-CP-Name										ProtocolIE-ID ::= 129

id-Extended-GNB-CU-UP-Name										ProtocolIE-ID ::= 130

id-DataForwardingtoE-UTRANInformationList						ProtocolIE-ID ::= 131

id-QosMonitoringReportingFrequency								ProtocolIE-ID ::= 132

id-QoSMonitoringDisabled										ProtocolIE-ID ::= 133

id-AdditionalHandoverInfo										ProtocolIE-ID ::= 134

id-Extended-NR-CGI-Support-List									ProtocolIE-ID ::= 135

id-DataForwardingtoNG-RANQoSFlowInformationList					ProtocolIE-ID ::= x1

END

-- ASN1STOP