3GPP TSG-RAN WG3 #113-e	R3-214149

Online, 16 – 27 Aug 2021

Agenda Item:	9.3.1

Source:	Nokia (moderator)

Title:	Summary of Offline Discussion on SCG release/addition (CB #15)

Document for:	Approval

# Introduction

CB: # 15_SCGConfigRelease

- For X2 interface: Either the use of the existing X2 signalling (the EN-DC Resource Configuration IE) is clarified, or A new flag is added for backward-compatibility (and for alignment with the F1AP/XnAP solution)?

- Indicate to SCG addition over the F1 and Xn?

- Any spec impact on SRB3 by SCG release?

- try to close this topic in this meeting, provide CRs if agreeable

- Reply LS to RAN2, if agreeable

(Nok- moderator)

Summary of offline disc in R3-214149

# For the Chairman’s Notes

Propose the following:

R3-20xxxa, R3-20xxxc merged

R3-20xxxc rev [in xxxg] – agreed

R3-20xxxd rev [in xxxh] – agreed

R3-20xxxe rev [in xxxi] – agreed

R3-20xxxf rev [in xxxj] – endorsed

Propose to capture the following:

Agreement text…

Agreement text…

WA: carefully crafted text…

Issue 1: no consensus

Issue 2: issue is acknowledged; need to further check the impact on xxx. May be possible to address with a pure st2 change. To be continued…

# Discussion

## EN-DC

In X2AP, there is the EN-DC Resource Configuration IE introduced since Rel.15. The same IE does not exist in XnAP (where there is no CHOICE structure for resource configuration). At the start of the meeting, companies make following declarations:

- In [1] and [10] it is declared that no changes are needed in X2AP at all – the EN-DC Resource Configuration IE can be used as is.

- In [14] and [5] (as one of possible options), it is proposed to reuse the EN-DC Resource Configuration IE, but to clarify how it is used to provide the information on SCG presence.

- In [11] and again [5] (as the other possible option), it is proposed to add an explicit flag to indicate presence of the SCG. However, while [11] proposes to signal SCG release only, [5] considers it necessary to indicate both, release and addition of the SCG resources.

In order to reach the conclusion, we must have a look at the original purpose of the IE.

Question 1-1: Companies are requested to comment on their understanding what was the original purpose of the EN-DC Resource Configuration IE, why it was defined in X2AP only and thus if using it to indicate resource configuration is functionally backward-compatible?

| Company | Comment |
| --- | --- |
| Nokia | When the EN-DC Resource Configuration IE was added, it was meant to help decode the CHOICE structure present only in X2AP (hence, it was not defined in XnAP). At the time, the SN was not meant to change anything in the resource configuration on its own. However, the structure of the IE maps onto the resource configuration and therefore, even if not meant, it could always have been interpreted as the indication of resource configuration. Thus, such interpretation of the IE is backward-compatible. |
| ZTE | Agree with Nokia’s analysis. |

In [5], it is observed that, currently, usage of the EN-DC Resource Configuration IE is not defined within RAN3 specifications. Thus, if it is assumed that the IE may be used to indicate also resource configuration changes, the sender could also modify other fields of the IE, e.g. PDCP location.

Question 1-2: Companies are requested to comment if, according to their understanding, any part of the EN-DC Resource Configuration IE can be changed at the sender? If not, how is it defined in the 3GPP what the sender may change and what not?

| Company | Comment |
| --- | --- |
| Nokia | As we discuss in [5], currently there is no clear guidance regarding how the EN-DC Resource Configuration IE is to be used. Some indirect hints may be derived from RRC and stage-2, but they apply to the allowed configuration changes, not the usage of the IE in question. Therefore, if it is assumed that the SN may signal different value than received from the MN, there is a risk that some implementation may assume in future that other changes of the configuration are allowed in the SN, too. So, we consider that if the SN is allowed to use the IE to indicate resource configuration changes, the it can be used should be clarified. |
| ZTE | No strong view, we are fine if it is needed. |

Considering the controversy and the fact that F1 and Xn already have an explicit SCG deactivation indication (and SCG addition indication is being discussed), [11] and [5] consider adding such indicator to X2AP as the easiest way to conclude the discussion.

Question 1-3: Companies are requested to comment if having an explicit indicator in X2AP could help avoid controversy about the use of EN-DC Resource Configuration IE and to sync with the Xn/F1 solution?

| Company | Comment |
| --- | --- |
| Nokia | It could help, but considering that the EN-DC Resource Configuration IE allows to signal both, the SCG release and the SCG addition, the new IE would make sense only if both changes are explicitly indicated, too. |
| ZTE | If the existing IE is reused, then the new IE is not needed. |

## NR-DC (Xn/F1)

For purpose of SN initiated SCG deactivation without releasing the SN, a new SCG Indication (released, ...) IE was introduced over Xn and F1. However, it was left as an open item whether also the SCG activation should also be signaled over Xn/F1.

At the start of the meeting, companies make following declarations:

- In [14][1][11] it is declared that additional signaling to support an SN initiated SCG addition is not needed, on basis that only MN can initiate the SCG addition. Hence, no changes need be introduced to Xn/F1 for this purpose.

- In [5][10] it is declared that SCG addition can be easily introduced as an additional codepoint to SCG Indication IE to clearly inform M-gNB-DU that the SCG resources have been added and aid in band selection optimization.

- In [14] it is declared that for the SN initiated SCG release case, clarification is needed in TS 37.340 to indicate that SRB3 resources can also be released via an SN initiated procedure. Additionally, to avoid conflict with MCG fast recovery functionality, it is proposed to clarify that in case fast MCG recovery via SRB3 is configured, the SN shall not trigger SCG release.

Question 2-1: Should an SN initiated SCG addition function be introduced?

| Company | Comment |
| --- | --- |
| Nokia | Considering that RAN3 has already introduced the SN initiated SCG release, it is straightforward to also support the SCG addition via a new codepoint in the SCG Indication IE. |
| ZTE | Do not to introduce SCG addition. Currently, when receiving SCG release indicator, the MN cannot reject. How about the SCG addition indicator. Meanwhile, RAN2 does not ask RAN3 to introduce SCG addition. I do not see the evident motivation for this. |

Question 2-2: In case of MN initiated SCG addition, are there any changes needed in Xn and F1 interfaces to indicate the SCG addition?

| Company | Comment |
| --- | --- |
| Nokia | In case of F1, the SCG addition can be deduced via the settings in UL Configuration IE. However, given SCG Indication IE has already been introduced, we think utilizing the same IE for informing the SCG addition is clearer. In case of Xn, past discussions revolved only on SN initiated scenarios. However, for the MN initiated SCG addition, a new IE over Xn on MN to SN direction could be considered to clearly indicate the MN initiated SCG addition as well. |
| ZTE | Not sure, does the existing MN initiated SN modification procedure need to be enhanced to support SCG addition? |

Question 2-3: In regard to SN initiated SCG release, are any clarifications needed for TS 37.340 to indicate that (a) SRB3 resources can also be released via an SN initiated procedure, and (b) to clarify the interaction with fast MCG recovery function?

| Company | Comment |
| --- | --- |
| Nokia | Clarifications (a) and (b) are beneficial. |
| ZTE | I do not think the clarification is needed. If SRB3 is configured and being used (e.g., fast recovery), then the smart SN will not require to SCG release. |

## Response LS

Two companies proposed to send a response to RAN2 [4,15].

Question 3-1: Please, comment if the response is needed and possibly, under which conditions?

| Company | Comment |
| --- | --- |
| Nokia | All right to send, if RAN3 concludes the discussion. |
| ZTE | Yes, agree with Nokia. |

# Conclusion, Recommendations [if needed]

If needed

# References

- R3-213229, “Further discussion on NR-NR DC Configuration Release” (ZTE), RAN3 #113

- R3-213230, “Clarification for SgNB trigger SCG release in Rel-15” (ZTE), RAN3 #113

- R3-213231, “Clarification for SgNB trigger SCG release in Rel-16” (ZTE), RAN3 #113

- R3-213232, “[DRAFT] Reply LS on signalling SN initiated release of SCG” (ZTE), RAN3 #113

- R3-213254, “Consideration on SN-initiated SCG release in EN-DC” (Nokia, Nokia Shanghai Bell), RAN3 #113

- R3-213476, “SCG Event Indication over F1” (Nokia, Nokia Shanghai Bell), RAN3 #113

- R3-213477, “SCG Event Indication over F1” (Nokia, Nokia Shanghai Bell), RAN3 #113

- R3-213623, “SCG Event Indication over Xn” (Nokia, Nokia Shanghai Bell), RAN3 #113

- R3-213624, “SCG Event Indication over Xn” (Nokia, Nokia Shanghai Bell), RAN3 #113

- R3-213273, “Proposed Response LS to RAN2 onSignalling SN initiated release of SCG resource” (NEC), RAN3 #113

- R3-213885, “How to release SCG configuration between MN-CU and MN-DU” (Ericsson), RAN3 #113

- R3-213886, “How to release SCG configuration between MN-CU and MN-DU CR 36.423” (Ericsson), RAN3 #113

- R3-213887, “How to release SCG configuration between MN-CU and MN-DU CR 36.423” (Ericsson), RAN3 #113

- R3-213979, “Further discussion on SCG release” (Huawei, Deutsche Telekom), RAN3 #113

- R3-213980, “[DRAFT] LS on signalling SN initiated release of SCG” (Huawei), RAN3 #113

- R3-213981, “Clarification on SCG release indication” (Huawei, Deutsche Telekom), RAN3 #113

- R3-213982, “Clarification on SCG release indication” (Huawei, Deutsche Telekom), RAN3 #113

- R3-213983, “SN triggerred SCG release” (Huawei, Deutsche Telekom), RAN3 #113

- R3-213984, “SN triggerred SCG release” (Huawei, Deutsche Telekom), RAN3 #113