3GPP TSG-RAN WG3 #113-e												R3-213243

Online, 16 Aug – 26 Aug 2021

Agenda item:		21.3

Source:			ZTE

Title:	(TP for Introduction of the enhanced IIoT over E1) New QoS related parameters

Document for:		Approval

- Introduction

Based on the discussion in RAN3#112e meeting, it has been agreed that the unit of the parameter survival time is defined as “time”, and the minimum value of survival time is determined to be 0us. The maximum value of survival time needs to be further discussed. Furthermore, in order to support the new QoS-related parameters in IIoT, the maximum value of periodicity in TSC assistance information needs to be extended to 60s.

Detailed analysis can be seen in the related discussion paper (R3-213242).

Observation 1: The range of survival time depends on the scenario in periodic deterministic communication, and the maximum value of survival time is at least 180s.

Observation 2: The range of periodicity in TSCAI cannot meet the requirement of the maximum transmission interval in the plant asset management scenario.

Observation 3: There is no need for RAN3 to discuss the application of survival time under the TSN service of acknowledgment mode and provide the input to SA2.

Proposal 1: The maximum value of survival time is at least 180s.

Proposal 2: The maximum value of periodicity should be extended to 60000000 us.

- Text Proposal (TS38.463 16.6.1)

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Begin of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.77	Periodicity

This IE indicates the Periodicity of the TSC QoS flow as defined in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Periodicity | M |  | INTEGER (0..60000000, …) | Periodicity expressed in units of 1 us. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

9.3.1.XX	Survival Time

This IE indicates the Survival Time of the TSC QoS flow as defined in TS 23.501 [20].

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| Survival Time | M |  | INTEGER (0.180000000, …) | Survival Time expressed in units of 1 us. |

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< Next Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>

### 9.4.5	Information Element Definitions

#### -- S

//SKIP the unrelated part//

SubscriberProfileIDforRFP ::= INTEGER (1..256, ...)

SurvivalTime ::= INTEGER (0..180000000, ...)

#### -- T

//SKIP the unrelated part//

TSCAssistanceInformation ::= SEQUENCE {

periodicity				Periodicity,

burstArrivalTime		BurstArrivalTime												OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {TSCAssistanceInformation-ExtIEs} }	OPTIONAL,

...

}

TSCAssistanceInformation-ExtIEs NGAP-PROTOCOL-EXTENSION ::= {

{ID id-SurvivalTime		CRITICALITY ignore		EXTENSION SurvivalTime		PRESENCE OPTIONAL},

...

}

Periodicity					::= INTEGER	(1..60000000,	...)

BurstArrivalTime			::= OCTET STRING

<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<<< End of Change >>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>>