3GPP TSG-RAN WG3 #113-e						R3-213277

E-meeting, 16 – 26 August 2021

Agenda Item:  9.3.4.1

Source:       NEC

Title:		Including of NASC IE for EPS to 5GS handover

Document for:	dicision

- Introduction

This contribution discusses the optional presence of NASC IE in the HANDOVER REQUEST message and clarify the NASC IE is to be mandatory included for EPS to 5GS handover.

- 2. Discussion

The NASC IE represent the NAS Container that contains the information that is transparent to RAN node, this IE is mandatory present for the handover from EPS to 5GS, as quoted from 38.331:

RRCReconfiguration-v1530-IEs ::= SEQUENCE { masterCellGroup OCTET STRING (CONTAINING CellGroupConfig) OPTIONAL, -- Need M fullConfig ENUMERATED {true} OPTIONAL, -- Cond FullConfig dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message OPTIONAL, -- Cond nonHO masterKeyUpdate MasterKeyUpdate OPTIONAL, -- Cond MasterKeyChange dedicatedSIB1-Delivery OCTET STRING (CONTAINING SIB1) OPTIONAL, -- Need N dedicatedSystemInformationDelivery OCTET STRING (CONTAINING SystemInformation) OPTIONAL, -- Need N otherConfig OtherConfig OPTIONAL, -- Need M nonCriticalExtension RRCReconfiguration-v1540-IEs OPTIONAL } ... MasterKeyUpdate ::= SEQUENCE { keySetChangeIndicator BOOLEAN, nextHopChainingCount NextHopChainingCount, nas-Container OCTET STRING OPTIONAL, -- Cond securityNASC ... } ... securityNASC; This field is mandatory present in case of inter system handover. Otherwise the field is optionally present, need N.

However in the NGAP: HANDOVER REQUEST message, the correspondent NACS IE is optional. This may create inter-operability problem if the NACS IE is not present but it required in RRC to set the nas-Container for the EPS to 5GS handover.

We therefore propose to clarify in the NGAP: HANDOVER REQUEST message the NASC IE shall be present for the EPS to 5GS handover.

- 3. Proposal

Proposal 1: it is proposed to clarify in the NGAP HANDOVER REQUEST message the NASC IE shall be present for the EPS to 5GS handover.

The companion CRs are provided.