3GPP TSG-RAN WG3 Meeting #113-e	R3-213556

E-meeting, 16-26 Aug 2021

Title: 	(TP to TS 38.410 BL CR) Multicast Session Management

Source: 	Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

Agenda item:	22.2.2

Document Type: Other

1. Introduction

Based on the discussion in [1], there are several new NGAP procedures to be introduced:

Proposal 3: Introduce non UE associated class1 NGAP: Session activation/deactivation procedures for Multicast Session Management.

Proposal 4: Introduce NGAP: GROUP PAGING message to support group paging from CN to RAN. And the NGAP: GROUP PAGING message includes:

- MBS Session ID

- TAI List

- Service Area

Proposal 6: Introduce a non-UE associated class1 NGAP: Multicast Distribution Setup procedure, triggered by the gNB:

- in case the gNB decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

- in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the core network shall provide IP multicast DL tunnel info to the gNB, to enable IP multicast transport.

The Text Proposal to TS 38.410 BL CR is provided in section 3.

2. Reference

- R3-213555 Consideration on Multicast Session Management, Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

3. Text Proposal to BL CR of TS 38.410

-----------------Start of the Changes-------------------

5	Functions of the NG interface

5.1	General

The following clauses describe the functions supported over the NG interface.

//skip unchanged part

5.5	PDU Session Management function

The PDU Session Management function is responsible for establishing, modifying and releasing the involved PDU sessions NG-RAN resources for user data transport once a UE context is available in the NG-RAN node.

NGAP supports transparent relaying of PDU Session and MBS Session related information by the AMF as described in TS 23.502 [6].

//skip unchanged part

## 5.xx	NR MBS Session Management function

The MBS Session Management function is responsible for establishing, modifying and releasing the involved NR MBS sessions NG-RAN resources for user data transport once an MBS context is available in the NG-RAN node.

## 5.yy	Group Paging Function

The group paging function supports the sending of paging requests to the NG-RAN nodes involved in the group paging area i.e. the NG-RAN nodes of the TA(s) the UEs have joined the MBS Session.

6	Signalling procedures of the NG interface

//skip unchanged part

## 6.xx	NR MBS Session Management Procedures

The following list of MBS Session management procedures are used to establish, release, or modify NG-RAN resources for a NR MBS session:

-	Broadcast Session Resource Setup;

-	Broadcast Session Resource Release;

-	Multicast Session Activation;

-	Multicast Session Deactivation;

-	Multicast Distribution Setup;

-	Multicast Distribution Release

## 6.yy	Group Paging Procedures

The following group paging procedure is used to send group paging requests to the NG-RAN nodes involved in the group paging area:

-	Group Paging.

-----------------End of the Changes-------------------