3GPP TSG-RAN WG3 Meeting #113-e	R3-213601

# Online, August 16th – 26th 2021

Agenda Item:	13.3.1

Source:	Ericsson

Title:	(TP for IAB BL CR for TS 38.473): Congestion Mitigation in IAB Networks

Document for:	Agreement

# Introduction

At the RAN3#112-e meeting, the following was agreed:

The following two types of congestion indication are supported in CP-based congestion mitigation: 1) per child link; 2) per BH RLC CH ID. Which type of congestion indication to be reported could be up to implementation. FFS on per BAP routing ID.

This paper discusses the granularity of control plane (CP)-based congestion indication. A TP for IAB BL CR for TS 38.473 is presented in the Annex.

# The design of IAB Congestion Indication IE

At the RAN3#112-e meeting, the following congestion reporting granularities were agreed:

- Per child link.

- Per BH RLC channel.

The two open issues are:

- The presence (M/O) of child node ID in the congestion indication.

- Whether to support reporting per BAP routing ID.

## The presence of child ID in the IAB Congestion Indication IE

As of today, the presence of Child Node Identifier IE in the IAB Congestion Indication IE is FFS, as shown below excerpt from IAB BL CR for TS 38.473:

#### 9.3.1.x	IAB Congestion Indication

This IE contains the IAB downlink congestion indication.

Editor’s NOTE: FFS whether the congestion can be indicated per BAP routing ID as well.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB Congestion Indication List |  | 1 |  |  |
| >IAB Congestion Indication List Item |  | 1.. <maxnoofIABCongInd> |  |  |
| >>Child Node Identifier | FFS |  | 9.3.1.111 | This IE identifies the child node, the link to which is congested. |
| >>BH RLC CH List |  | 0..1 |  |  |
| >>>BH RLC CH List Item |  | 1.. <maxnoofBHRLCChannels> |  |  |
| >>>>BH RLC CH ID | M |  | 9.3.1.113 | This IE identifies the congested BH RLC channel over the link towards the node identified by the Child Node Identifier IE. |

In our view, the presence should be mandatory since the child identifier must be reported regardless of reporting granularity:

- If the reporting granularity is per BH RLC CH ID, the child ID must be reported because the BH RLC CH ID space is per parent-child pair.

- If the reporting granularity is per BAP routing ID, the child ID again needs to be reported because, in backhaul routing, for a certain BAP routing ID, the next hop BAP address and egress BH RLC CH ID depends on the previous hop BAP address and ingress BH RLC CH ID. This means that packets with the same BAP Routing ID may be transmitted over different sets of intermediate nodes and BH RLC channels between the donor DU and the destination node. Consequently, congestion indication per BAP routing ID would also require reporting the corresponding BH RLC CH ID. In other words, BAP PDUs carrying a BAP Routing ID may be mapped to and transmitted over multiple BH RLC channels on the same parent-child link and even via different physical paths.

Observation 1: The congestion reporting per BH RLC channel requires the reporting of the ID of the child terminating the indicated BH RLC CHs, since the BH RLC CH ID space is per parent-child pair.

Observation 2: The congestion reporting per BAP routing ID requires that child ID is reported because a BAP routing ID may pertain to more than one sequence of intermediate nodes and BH RLC CHs.

Proposal 1: RAN3 to make the presence of child ID mandatory in the IAB Congestion Indication IE.

## Reporting granularity per BAP routing ID

Given that two options for congestion indication granularity have been already agreed, we feel that the specification of per BAP routing ID granularity would introduce too many options without any tangible benefits. Moreover, any cause of congestion on a certain link will equally affect all BAP routing IDs, meaning that all BAP routing IDs will have to be reported, thus defeating the purpose of congestion indication reporting. Hence, we propose to preclude reporting per BAP routing ID.

Proposal 2: The congestion reporting per BAP routing ID is not supported.

Assuming the above, the maxnoofIABCongInd also needs to be aligned with the agreement on reporting granularity, meaning that it should be equal to the maximum number of child nodes (i.e., 1024).

Proposal 3: In the IAB BL CR for TS 38.473, set the value of maxnoofIABCongInd to 1024, i.e., the maximum number of child nodes.

# Conclusion

This paper discusses congestion mitigation in IAB networks. The following is proposed:

Proposal 1: RAN3 to make the presence of child ID mandatory in the IAB Congestion Indication IE.

Proposal 2: The congestion reporting per BAP routing ID is not supported.

Proposal 3: In the IAB BL CR for TS 38.473, set the value of maxnoofIABCongInd to 1024, i.e., the maximum number of child nodes.

# Annex: TP for IAB BL CR for TS 38.473

-------------------------------------------Change 1-------------------------------------------

#### 9.3.1.x	IAB Congestion Indication

This IE contains the IAB downlink congestion indication.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| IAB Congestion Indication List |  | 1 |  |  |
| >IAB Congestion Indication List Item |  | 1.. <maxnoofIABCongInd> |  |  |
| >>Child Node Identifier | M |  | 9.3.1.111 | This IE identifies the child node, the link to which is congested. |
| >>BH RLC CH List |  | 0..1 |  |  |
| >>>BH RLC CH List Item |  | 1.. <maxnoofBHRLCChannels> |  |  |
| >>>>BH RLC CH ID | M |  | 9.3.1.113 | This IE identifies the congested BH RLC channel over the link towards the node identified by the Child Node Identifier IE. |

| Range bound | Explanation |
| --- | --- |
| maxnoofIABCongInd | Maximum no. of congestion indications, the maximum value is 1024. |
| maxnoofBHRLCChannels | Maximum no. of BH RLC channels allowed towards one IAB-node, the maximum value is 65536. |

-------------------------------------------Change 2-------------------------------------------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

-- Information Element Definitions

--

-- **************************************************************

>>>>>>>>>>>>>>>>>Unchanged parts are skipped<<<<<<<<<<<<<<<

#### -- I

IAB-Barred	::=	ENUMERATED {barred, not-barred, ...}

IABCongestionIndication ::= SEQUENCE {

iAB-Congestion-Indication-List			IAB-Congestion-Indication-List,

iE-Extensions	ProtocolExtensionContainer { { IAB-Congestion-Indication-List-ExtIEs } } OPTIONAL

}

IAB-Congestion-Indication-List-ExtIEs	F1AP-PROTOCOL-EXTENSION ::= {

...

}

IAB-Congestion-Indication-List ::= SEQUENCE (SIZE(1..maxnoofIABCongInd)) OF IAB-Congestion-Indication-Item

IAB-Congestion-Indication-Item ::= SEQUENCE {

childNodeIdentifier				BAPAddress      M,

bHRLCCHList		                BHRLCCHList	    OPTIONAL,

iE-Extensions					ProtocolExtensionContainer { { IAB-Congestion-Indication-ItemExtIEs } }	OPTIONAL

}

IAB-Congestion-Indication-ItemExtIEs F1AP-PROTOCOL-EXTENSION ::= {

...

}

-------------------------------------------Change 3-------------------------------------------

### 9.4.7	Constant Definitions

-- ASN1START

-- **************************************************************

--

-- Constant definitions

--

-- **************************************************************

F1AP-Constants {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) f1ap (3) version1 (1) f1ap-Constants (4) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- **************************************************************

--

-- IE parameter types from other modules.

--

-- **************************************************************

IMPORTS

ProcedureCode,

ProtocolIE-ID

FROM F1AP-CommonDataTypes;

-- **************************************************************

--

#### -- Elementary Procedures

--

-- **************************************************************

id-Reset									ProcedureCode ::= 0

id-F1Setup									ProcedureCode ::= 1

id-ErrorIndication							ProcedureCode ::= 2

id-gNBDUConfigurationUpdate					ProcedureCode ::= 3

id-gNBCUConfigurationUpdate					ProcedureCode ::= 4

id-UEContextSetup							ProcedureCode ::= 5

id-UEContextRelease							ProcedureCode ::= 6

id-UEContextModification					ProcedureCode ::= 7

id-UEContextModificationRequired			ProcedureCode ::= 8

id-UEMobilityCommand						ProcedureCode ::= 9

id-UEContextReleaseRequest					ProcedureCode ::= 10

id-InitialULRRCMessageTransfer				ProcedureCode ::= 11

id-DLRRCMessageTransfer						ProcedureCode ::= 12

id-ULRRCMessageTransfer						ProcedureCode ::= 13

id-privateMessage							ProcedureCode ::= 14

id-UEInactivityNotification					ProcedureCode ::= 15

id-GNBDUResourceCoordination				ProcedureCode ::= 16

id-SystemInformationDeliveryCommand			ProcedureCode ::= 17

id-Paging									ProcedureCode ::= 18

id-Notify									ProcedureCode ::= 19

id-WriteReplaceWarning						ProcedureCode ::= 20

id-PWSCancel								ProcedureCode ::= 21

id-PWSRestartIndication						ProcedureCode ::= 22

id-PWSFailureIndication						ProcedureCode ::= 23

id-GNBDUStatusIndication 					ProcedureCode ::= 24

id-RRCDeliveryReport		 				ProcedureCode ::= 25

id-F1Removal								ProcedureCode ::= 26

id-NetworkAccessRateReduction				ProcedureCode ::= 27

id-TraceStart								ProcedureCode ::= 28

id-DeactivateTrace							ProcedureCode ::= 29

id-DUCURadioInformationTransfer				ProcedureCode ::= 30

id-CUDURadioInformationTransfer				ProcedureCode ::= 31

id-BAPMappingConfiguration					ProcedureCode ::= 32

id-GNBDUResourceConfiguration				ProcedureCode ::= 33

id-IABTNLAddressAllocation					ProcedureCode ::= 34

id-IABUPConfigurationUpdate					ProcedureCode ::= 35

id-resourceStatusReportingInitiation		ProcedureCode ::= 36

id-resourceStatusReporting					ProcedureCode ::= 37

id-accessAndMobilityIndication				ProcedureCode ::= 38

id-accessSuccess							ProcedureCode ::= 39

id-cellTrafficTrace 							ProcedureCode ::= 40

id-PositioningMeasurementExchange			ProcedureCode ::= 41

id-PositioningAssistanceInformationControl	ProcedureCode ::= 42

id-PositioningAssistanceInformationFeedback	ProcedureCode ::= 43

id-PositioningMeasurementReport				ProcedureCode ::= 44

id-PositioningMeasurementAbort				ProcedureCode ::= 45

id-PositioningMeasurementFailureIndication	ProcedureCode ::= 46

id-PositioningMeasurementUpdate				ProcedureCode ::= 47

id-TRPInformationExchange					ProcedureCode ::= 48

id-PositioningInformationExchange			ProcedureCode ::= 49

id-PositioningActivation					ProcedureCode ::= 50

id-PositioningDeactivation					ProcedureCode ::= 51

id-E-CIDMeasurementInitiation				ProcedureCode ::= 52

id-E-CIDMeasurementFailureIndication		ProcedureCode ::= 53

id-E-CIDMeasurementReport					ProcedureCode ::= 54

id-E-CIDMeasurementTermination				ProcedureCode ::= 55

id-PositioningInformationUpdate				ProcedureCode ::= 56

id-ReferenceTimeInformationReport			ProcedureCode ::= 57

id-ReferenceTimeInformationReportingControl	ProcedureCode ::= 58

-- **************************************************************

--

#### -- Extension constants

--

-- **************************************************************

maxPrivateIEs							INTEGER ::= 65535

maxProtocolExtensions					INTEGER ::= 65535

maxProtocolIEs							INTEGER ::= 65535

-- **************************************************************

--

#### -- Lists

--

-- **************************************************************

maxNRARFCN								INTEGER ::= 3279165

maxnoofErrors							INTEGER ::= 256

maxnoofIndividualF1ConnectionsToReset	INTEGER ::= 65536

maxCellingNBDU							INTEGER ::= 512

maxnoofSCells							INTEGER ::= 32

maxnoofSRBs								INTEGER ::= 8

maxnoofDRBs								INTEGER ::= 64

maxnoofULUPTNLInformation				INTEGER ::= 2

maxnoofDLUPTNLInformation				INTEGER ::= 2

maxnoofBPLMNs							INTEGER ::= 6

maxnoofCandidateSpCells					INTEGER ::= 64

maxnoofPotentialSpCells					INTEGER ::= 64

maxnoofNrCellBands						INTEGER ::= 32

maxnoofSIBTypes							INTEGER ::= 32

maxnoofSITypes							INTEGER ::= 32

maxnoofPagingCells						INTEGER ::= 512

maxnoofTNLAssociations					INTEGER ::= 32

maxnoofQoSFlows							INTEGER ::= 64

maxnoofSliceItems						INTEGER ::= 1024

maxCellineNB							INTEGER ::= 256

maxnoofExtendedBPLMNs					INTEGER ::= 6

maxnoofUEIDs							INTEGER ::= 65536

maxnoofBPLMNsNR							INTEGER ::= 12

maxnoofUACPLMNs							INTEGER ::= 12

maxnoofUACperPLMN						INTEGER ::= 64

maxnoofAdditionalSIBs					INTEGER ::= 63

maxnoofslots							INTEGER ::= 5120

maxnoofTLAs								INTEGER ::=	16

maxnoofGTPTLAs							INTEGER ::=	16

maxnoofBHRLCChannels					INTEGER ::= 65536

maxnoofRoutingEntries					INTEGER ::= 1024

maxnoofIABSTCInfo						INTEGER ::= 45

maxnoofSymbols							INTEGER ::= 14

maxnoofServingCells						INTEGER ::= 32

maxnoofDUFSlots							INTEGER ::= 320

maxnoofHSNASlots						INTEGER ::= 5120

maxnoofServedCellsIAB					INTEGER ::= 512

maxnoofChildIABNodes					INTEGER ::= 1024

maxnoofNonUPTrafficMappings				INTEGER ::= 32

maxnoofTLAsIAB							INTEGER ::= 1024

maxnoofMappingEntries					INTEGER ::= 67108864

maxnoofDSInfo							INTEGER ::= 64

maxnoofEgressLinks						INTEGER ::= 2

maxnoofULUPTNLInformationforIAB			INTEGER ::= 32678

maxnoofUPTNLAddresses					INTEGER ::= 8

maxnoofSLDRBs							INTEGER ::= 512

maxnoofQoSParaSets						INTEGER ::= 8

maxnoofPC5QoSFlows						INTEGER ::= 2048

maxnoofSSBAreas							INTEGER ::=	64

maxnoofPhysicalResourceBlocks			INTEGER ::= 275

maxnoofPhysicalResourceBlocks-1			INTEGER ::= 274

maxnoofPRACHconfigs						INTEGER ::= 16

maxnoofRACHReports						INTEGER ::= 64

maxnoofRLFReports						INTEGER ::= 64

maxnoofAdditionalPDCPDuplicationTNL		INTEGER ::=	2

maxnoofRLCDuplicationState				INTEGER ::=	3

maxnoofCHOcells							INTEGER ::= 8

maxnoofMDTPLMNs							INTEGER ::=	16

maxnoofCAGsupported						INTEGER ::= 12

maxnoofNIDsupported						INTEGER ::= 12

maxnoofNRSCSs							INTEGER ::= 5

maxnoofExtSliceItems					INTEGER ::= 65535

maxnoofPosMeas							INTEGER ::=	16384

maxnoofTRPInfoTypes						INTEGER ::=	64

maxnoofTRPs								INTEGER ::=	65535

maxnoofSRSTriggerStates					INTEGER ::= 3

maxnoofSpatialRelations					INTEGER ::= 64

maxnoBcastCell							INTEGER ::= 16384

maxnoofAngleInfo						INTEGER ::= 65535

maxnooflcs-gcs-translation				INTEGER ::= 3

maxnoofPath								INTEGER ::= 2

maxnoofMeasE-CID						INTEGER ::= 64

maxnoofSSBs								INTEGER ::= 255

maxnoSRS-ResourceSets					INTEGER ::= 16

maxnoSRS-ResourcePerSet					INTEGER ::= 16

maxnoSRS-Carriers						INTEGER ::= 32

maxnoSCSs								INTEGER ::= 5

maxnoSRS-Resources						INTEGER ::= 64

maxnoSRS-PosResources					INTEGER ::= 64

maxnoSRS-PosResourceSets				INTEGER ::= 16

maxnoSRS-PosResourcePerSet				INTEGER ::= 16

maxnoofPRS-ResourceSets					INTEGER ::= 2

maxnoofPRS-ResourcesPerSet				INTEGER ::= 64

maxNoOfMeasTRPs							INTEGER ::= 64

maxnoofPRSresourceSets					INTEGER ::= 8

maxnoofPRSresources						INTEGER ::= 64

maxnoofIABCongInd					    INTEGER ::= 1024

-------------------------------------------End of changes-------------------------------------------