3GPP TSG-RAN WG3 Meeting #113-e	R3-213652

Online, 16 - 27 August 2021

Agenda item:	  22.2.2

Source:	  Qualcomm Incorporated, Huawei, Lenovo, Motorola Mobility

Title:		(TP to TS 38.470 BL CR) Multicast Session Management

Document for:		Other

# 1	Introduction

- As discussed in [1], the following proposals are achieved:

Proposal 1: Introduce MBS Session List in PDU Session Level, and each MBS Session item should:

- includes MBS Session ID;

- includes list of MBS QoS Flow information (includes MBS QoS Flow ID, MBS QoS profile, and the mapped unicast QoS Flow ID);

- supports to add/modify/release MBS QoS flows;

- includes MBS Service Area Information.

Proposal 2: Use the accepted MBS QFI in the PDU Session Resource Modify Response Transfer IE to indicate the support of MBS by the gNB.

Proposal 3: Introduce non UE associated class1 NGAP: Session activation/deactivation procedures for Multicast Session Management.

Proposal 4: Introduce NGAP: GROUP PAGING message to support group paging from CN to RAN. And the NGAP: GROUP PAGING message includes:

- MBS Session ID

- TAI List

- Service Area

Proposal 5: Introduce non UE associated Class 2 F1AP: Group Paging procedure to support group paging from gNB-CU to gNB-DU. And the F1AP: GROUP PAGINGE message includes:

- MBS Session ID

- Paging Cell List

Proposal 6: Introduce a non-UE associated class1 NGAP: Multicast Distribution Setup procedure, triggered by the gNB:

- in case the gNB decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Setup Request.

- in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the core network shall provide IP multicast DL tunnel info to the gNB, to enable IP multicast transport.

Proposal 7: Introduce a non-UE associated Class1 E1AP procedure, e.g. named as Multicast Distribution Setup procedure, triggered by the gNB-CU-UP:

- in case the gNB-CU-UP decides or is configured to use unicast transport for multicast distribution sessions, it includes the DL GTP-U tunnel info in the Multicast Distribution Response Request.

- in case the DL GTP-U tunnel info is not included in the Multicast Distribution Setup Request, the cgNB-CU-CP shall provide IP multicast DL tunnel info to the gNB-CU-UP, to enable IP multicast transport.

- In this paper, we provide the text proposal to BL CR of TS 38.470 in Section 3.

# 2 Reference

- R3-213555	Consideration on Multicast Session Management	Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

- R3-213556	(TP to TS 38.410 BL CR) Multicast Session Management	Huawei, CBN, China Unicom, Lenovo, Motorola Mobility, Qualcomm Incorporated, China Telecom

- R3-213557	(TP to TS 38.413 BL CR) Multicast Session Management	Huawei, Lenovo, Motorola Mobility, Qualcomm Incorporated

- R3-213746	(TP to TS 38.460 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

- R3-213747	(TP to TS 38.463 BL CR) Support of Multicast Distribution Setup and Release	Lenovo, Motorola Mobility, Huawei, Qualcomm Incorporated

# 3	Text Proposal

-----------------Start of the Changes-------------------

### 6.1.x	NR MBS procedures

The F1 MBS procedures are listed below:

- Group Paging procedure