3GPP TSG-RAN WG3 Meeting #113-e	R3-213711

E-Meeting, 16-26 Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.425 | CR | 0123 | rev |  | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of F1-U delay measurement for QoS monitoring 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Verizon Wireless |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2021-8-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | F1-U delay measurement should be supported in RAN side. According to 28.552, the corresponding measurement method is The time when receiving a GTP packet delivery status message from the gNB DU at the egress GTP termination, minus time when sending the same packet to gNB DU at the GTP ingress termination, minus feedback delay time in gNB DU, obtained result is divided by two. But the feedback delay time in gNB DU is unavailable. Therefore, feedback delay time in gNB DU should be provided for F1-U delay measurement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add DU Feedback Delay For Highest Successfully Delivered NR PDCP Sequence Number, DU Feedback Delay For Highest Transmitted NR PDCP Sequence Number. Impact Analysis: Impact assessment towards the previous version of the specification (same release): This CR has isolate impact on because the change only affects DL DATA DELIVERY STATUS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The F1-U delay measurement for QoS monitoring can not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.1, 5.5.2.2, 5.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<<<<<<<<<<<<<<<<<<< Start of the First Change >>>>>>>>>>>>>>>>>>>>

### 5.4.2	Downlink Data Delivery Status

#### 5.4.2.1	Successful operation

……

In case of RLC AM, after the highest NR PDCP PDU sequence number successfully delivered in sequence is reported to the node hosting the NR PDCP entity, the corresponding node removes the respective NR PDCP PDUs. For RLC UM, the corresponding node may remove the respective NR PDCP PDUs after transmitting to lower layers.

The Downlink Data Delivery Status procedure may be invoked if the corresponding node is configured to perform the QoS monitoring and to send the DU feedback delay results to the node hosting the NR PDCP entity for F1-U delay measurement.

<<<<<<<<<<<<<<<<<<<< End of the First Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<< Start of the Second Change >>>>>>>>>>>>>>>>>>>>

#### 5.5.2.2	DL DATA DELIVERY STATUS (PDU Type 1)

This frame format is defined to transfer feedback to allow the receiving node (i.e. the node that hosts the NR PDCP entity) to control the downlink user data flow via the sending node (i.e. the corresponding node).

The following shows the respective DL DATA DELIVERY STATUS frame. The Figure shows an example of how a frame is structured when all optional IEs (i.e. those whose presence is indicated by an associated flag) are present.

Absence of such an IE changes the position of all subsequent IEs on octet level.

| Bits |  |  |  |  |  |  |  |  |  |  | Number of Octets |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7 |  | 6 |  | 5 |  | 4 | 3 | 2 | 1 | 0 |  |
| PDU Type (=1) |  |  |  |  |  |  | Highest Transmitted NR PDCP SN Ind | Highest Delivered NR PDCP SN Ind | Final Frame Ind. | Lost Packet Report | 1 |
| Spare | DU Feedback Delay For Highest Transmitted NR PDCP SN Packet Ind |  | DU Feedback Delay For Highest Delivered NR PDCP SN Packet Ind |  | Delivered NR PDCP SN Range Ind |  | Data rate Ind. | Retransmitted NR PDCP SN Ind | Delivered Retransmitted NR PDCP SN Ind | Cause Report | 1 |
| Desired buffer size for the data radio bearer |  |  |  |  |  |  |  |  |  |  | 4 |
| Desired Data Rate |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| Number of lost NR-U Sequence Number ranges reported |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Start of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |  |  | 0 or (6* Number of reported lost NR-U SN ranges) |
| End of lost NR-U Sequence Number range |  |  |  |  |  |  |  |  |  |  |  |
| Highest successfully delivered NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DU Feedback Delay For Highest Successfully Delivered NR PDCP Sequence Number Packet |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| Highest transmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| DU Feedback Delay For Highest Transmitted NR PDCP Sequence Number Packet |  |  |  |  |  |  |  |  |  |  | 0 or 4 |
| Cause Value |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Successfully delivered retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Retransmitted NR PDCP Sequence Number |  |  |  |  |  |  |  |  |  |  | 0 or 3 |
| Number of successfully delivered out of sequence PDCP Sequence Number range |  |  |  |  |  |  |  |  |  |  | 0 or 1 |
| Start of successfully delivered out of sequence PDCP Sequence Number range |  |  |  |  |  |  |  |  |  |  | 0 or (6* Number of successfully delivered out of sequence PDCP Sequence Number range) |
| End of successfully delivered out of sequence PDCP Sequence Number range |  |  |  |  |  |  |  |  |  |  |  |
| Padding |  |  |  |  |  |  |  |  |  |  | 0-3 |

Figure 5.5.2.2-1: DL DATA DELIVERY STATUS (PDU Type 1) Format

<<<<<<<<<<<<<<<<<<<< End of the Second Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<< Start of the Third Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

#### 5.5.3.X	DU Feedback Delay For Highest Successfully Delivered NR PDCP Sequence Number Packet

Description: This field indicates DU feedback delay measured at the corresponding node in milliseconds for the packet indicated by highest successfully delivered NR PDCP sequence number. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the F1-U delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8].

Value range: {0..232-1}.

Field length: 4 octets.

<<<<<<<<<<<<<<<<<< End of the Third Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<< Start of the Fourth Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

#### 5.5.3.X	DU Feedback Delay For Highest Transmitted NR PDCP Sequence Number Packet

Description: This field indicates DU feedback delay measured at the corresponding node in milliseconds for the packet indicated by highest transmitted NR PDCP sequence number. It is encoded as an Unsigned32 binary integer value. The node hosting PDCP entity shall, if supported, use this information to calculate the F1-U delay for the concerned DRB and report to the UPF for the purpose of QoS monitoring as specified in [8].

Value range: {0..232-1}.

Field length: 4 octets.

<<<<<<<<<<<<<<<<<< End of the Fourth Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<< Start of the Fifth Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

#### 5.5.3.X	DU Feedback Delay For Highest Transmitted NR PDCP SN Packet Ind

Description: This field indicates the presence of DU Feedback Delay For Highest Transmitted NR PDCP Sequence Number Packet.

Value range: {0 = DU Feedback Delay For Highest Transmitted NR PDCP Sequence Number Packet not present, 1 = DU Feedback Delay For Highest Transmitted NR PDCP Sequence Number Packet present}.

Field length: 1 bit.

<<<<<<<<<<<<<<<<<< End of the Fifth Change >>>>>>>>>>>>>>>>>>>>

<<<<<<<<<<<<<<<<<< Start of the Sixth Change >>>>>>>>>>>>>>>>>>>>

### 5.5.3	Coding of information elements in frames

#### 5.5.3.X	DU Feedback Delay For Highest Delivered NR PDCP SN Packet Ind

Description: This field indicates the presence of DU Feedback Delay For Highest Successfully Delivered NR PDCP Sequence Number Packet.

Value range: {0 = DU Feedback Delay For Highest Successfully Delivered NR PDCP Sequence Number Packet no present, 1 = DU Feedback Delay For Highest Successfully Delivered NR PDCP Sequence Number Packet present}.

Field length: 1 bit.

<<<<<<<<<<<<<<<<<< End of the Sixth Change >>>>>>>>>>>>>>>>>>>>