3GPP TSG-RAN WG3 #113-e								R3-213899

16-26 Aug 2021

Agenda Item:	9.3.4.1

Source:	China Telecom

Title: 	Correction on LTE UE RLF report

Document for:	Discussion

Introduction

For LTE UE RLF Report, there are two parts in UEInformationResponse message which are RLF-Report-r9 IE and RLF-Report-v9e0 IE. The contribution [1] proposed to include RLF-Report-v9e0 IE in LTE UE RLF Report in RAN3#115 meeting. This paper analyses the scenario on reporting RLF-Report-v9e0 in NR and gives some proposals.

Discussion

Scenario

RLF-report-r9 IE is used to report the measurement result during radio link failure. It includes the following information:

RLF-Report-r9 ::= 					SEQUENCE {

measResultLastServCell-r9				SEQUENCE {

rsrpResult-r9							RSRP-Range,

rsrqResult-r9							RSRQ-Range			OPTIONAL

},

measResultNeighCells-r9				SEQUENCE {

measResultListEUTRA-r9					MeasResultList2EUTRA-r9		OPTIONAL,

measResultListUTRA-r9					MeasResultList2UTRA-r9		OPTIONAL,

measResultListGERAN-r9					MeasResultListGERAN			OPTIONAL,

measResultsCDMA2000-r9					MeasResultList2CDMA2000-r9	OPTIONAL

}																OPTIONAL,

...

}

The measurement results of neighbour LTE cells MeasResultList2EUTRA-r9 as an optional IE contained in RLF reports. As we know, the range of ARFCN-ValueEUTRA is not enough to cover all bands defined in TS36.101. Thus, 3GPP introduce an extension value for EARFCN in release 9.

ARFCN-ValueEUTRA-v9e0 ::=			INTEGER (maxEARFCN-Plus1..maxEARFCN2)

As per TS36.331, the definition of RLF-Report-v9e0 IE is as follow:

UEInformationResponse-v9e0-IEs ::= SEQUENCE {

rlf-Report-v9e0						RLF-Report-v9e0					OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

RLF-Report-v9e0 ::= 				SEQUENCE {

measResultListEUTRA-v9e0			MeasResultList2EUTRA-v9e0

}

MeasResultList2EUTRA-v9e0 ::=			SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA-v9e0

MeasResult2EUTRA-v9e0 ::=				SEQUENCE {

carrierFreq-v9e0						ARFCN-ValueEUTRA-v9e0		OPTIONAL

}

The RLF-Report-v9e0 IE consists of multiple measurement results ARFCN-ValueEUTRA-v9e0. And the ARFCN-ValueEUTRA-v9e0 is an extension value for IE ARFCN-ValueEUTRA which used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) E-UTRA carrier frequency, as defined in TS 36.101. In addition, the measResultListEUTRA-v9e0 shall include the same number of entries, and listed in the same order, as in measResultListEUTRA-r9.

On this point, the RLF-Report-v9e0 can be regarded as an assistant information to indicate the extension value for the measurement results listed in RLF-Report-r9.

Observation: the RLF-Report-v9e0 indicates the extension value for the measurement results listed in RLF-Report-r9.

In order to support MRO in inter-system scenario, the following highlight information was agreed to be contained in the RLF report. It is natural to introduce the RLF-Report-v9e0 IE in NR scenario to indicate neighbouring cell which shall use ARFCN-ValueEUTRA-v9e0 to indicate its ARFCN value.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CHOICE type | M |  |  |  |
| >NR |  |  |  |  |
| >>NR UE RLF Report Container | M |  | OCTET STRING | nr-RLF-Report-r16 IE contained in the UEInformationResponse message defined in TS 38.331 [10]. |
| >LTE |  |  |  |  |
| >>LTE UE RLF Report Container | M |  | OCTET STRING | RLF-Report-r9 IE contained in the UEInformationResponse message defined in TS 36.331 [14] |

Proposal: It is proposed to include RLF-Report-v9e0 IE in LTE UE RLF Report.

Proposal

Based on the above analysis, we have the following observation and proposal:

Observation: the RLF-Report-v9e0 indicates the extension value for the measurement results listed in RLF-Report-r9.

Proposal: It is proposed to include RLF-Report-v9e0 IE in LTE UE RLF Report.

Reference

- R3-212509, Correction on LTE UE RLF Report ,China Telecom, CATT