3GPP TSG-RAN WG3 Meeting #113-e	R3-214073

E-meeting, 16-26 Aug 2021

Title: 	Further discussions on per-slice QoE measurement

Source: 	Huawei

Agenda item:	15.2.1.2

Document Type:	Discussion

1. Introduction

In the last meeting, RAN3 discussed the configuration and reporting of QoE and had the following agreements and FFS:

Liaise SA5 on the support of (de)activation of NR QoE, including concerns on whether current Trace Function could support QoE mechanism, decoupling of deactivation, failure handling and QoE Reference

Whether to reuse the TRACE START message for the activation of QoE or to introduce a new IE (QoE Activation IE) which is independent of Trace Function pending to the reply from SA5.

At least enhancements on the following messages over network interface are needed for the activation of NR QoE, no matter which solution is approved:

NGAP:

- INITIAL CONTEXT SETUP REQUEST

- HANDOVER REQUEST

- HANDOVER REQUIRED

XnAP:

- HANDOVER REQUEST

- RETRIEVE UE CONTEXT RESPONSE

Whether to reuse DEACTIVATE TRACE message for the deactivation message or to introduce a new IE (QoE deactivation IE) in UE CONTEXT MDIFICATION REQUEST message over NGAP.

Stage 2 description for NR QMC description to be introduced in TS 38.300.

To be continued...

Introduce a new IE "QoE Reference" explicitly over interfaces at least for s-based, whether it can be applied to m-based and whether it is per service type or per slice depends on feedback from SA5

Introduce a new IE "Measurement Collection Entity IP Address", FFS whether it is per service type or per "QoE Reference" depends on feedback from SA5

Include slice info as explicit IE in the configuration message over NG, FFS whether it should be also included inside the transparent configuration container; FFS whether slice info should be signalled as an explicit IE in the configuration message and in the report message over radio interface.

Introduce the following additional new IEs:

- a list of UE Application layer measurement configuration IE for each service type.

- inside each UE Application layer measurement configuration IE:

- Container.

- a numerated IE indicating service type (e.g., Streaming services, MTSI services, VR, MBMS, XR).

- Area scope (a list of cells/TA/TAI/PLMN).

- Slice scope (FFS a list of S-NSSAI).

Additional IEs are FFS, FFS on the detail of IE names

Further details on how slice info should be reflected in the configuration info and report message.

Stage 3 details, e.g. whether Measurement Collection Entity IP Address and QoE reference ID is per service type or not, slice scope, etc.

Whether and how to support of roaming UEs.

Whether a pause indication is needed, as guidance from OAM/CN to RAN for handling in case of RAN overload.

Whether a prioritization mechanism of different service types or slices is needed for the RAN to pause or release ongoing QoE measurements in case of RAN overload.

Whether a prioritization mechanism is needed for UE to send to RAN pending QoE reports when RAN overload is solved.

Whether to introduce any or all of these criteria, including one or more time-based, one or more threshold-based and one or more event-based, as conditions for triggering/stopping the QoE measurement.

To be continued...

This contribution will mainly discuss the remaining issues about per-slice QoE measurements.

2. Discussion

In the last meeting, RAN3 has agreed to introduce the slice info in the configuration message [1] [2] [3] [4] [5]. In our understanding, the motivation is to measure the QoE for these slices. The NG-RAN can send the slice info as an explicit IE and the transparent configuration container to the UE. Therefore the slice info does not need to be included in the transparent configuration container.

- The transparent configuration container does not need to include the slice info. The NG-RAN sends the slice info as an explicit IE and the transparent configuration container to the UE in Uu.

For the QoE report as a transparent reporting container, it is used by the OAM. Therefore, the NG-RAN does not need to know the slice ID of the QoE report, the slice ID is only needed to be included in the transparent reporting container.

For the RAN visible QoE metric reporting, it is used by the NG-RAN. RAN3 are still discussing whether to support the per slice RAN visible QoE metric. Therefore we think whether the slice ID as an explicit IE is included in the Uu depends on the progress in RAN visible QoE metric.

- For the legacy QoE measurement reporting, the slice ID is included in the transparent reporting container.

Another FFS is the format of the slice scope. Each network slice is uniquely identified by a S-NSSAI. The CN inform the RAN the relation of PDU session and slice. For both management and signalling based, the slice scope in the QoE measurement is originally configured by the OAM. In our understanding, the OAM does not know the relation of PDU session and slice, therefore we think the slice scope should be a list of S-NSSAI, so that the RAN could decide whether to configure the QoE measurement for the UE based on whether the UE has set up the service of these slice IDs and the UE could decide whether to perform the QoE measurement based on the relation between mapping of PDU session and slice ID to perform QoE measurement, which is pending on SA4 final decision.

- The slice scope is a list of S-NSSAI.

3. Conclusion

Based on the discussion in this paper, we have the following observations and proposal.

- The transparent configuration container does not need to include the slice info. The NG-RAN sends the slice info as an explicit IE and the transparent configuration container to the UE in Uu.

- For the legacy QoE measurement reporting, the slice ID is included in the transparent reporting container.

- The slice scope is a list of S-NSSAI.

4. Reference

- R3-212498, Per-slice QoE measurement (CMCC)	discussion

- R3-212518, Further analysis on spec impacts of the potential solutions to QoE measurement per slice (Huawei)	other

- R3-212591, On per-slice QoE measurement (Nokia Corporation, Nokia Shanghai Bell)	discussion

- R3-211734, Per slice QoE measurements (Qualcomm Incorporated)	discussion

- R3-211837, Discussion on per-slice QoE measurement (CATT)	discussion