3GPP TSG-RAN WG3 #113-e							                                              R3-214109

16-26 Aug 2021

Online

Agenda item:	15.3

Source:	CMCC

Title:	Further discussions on RAN visible QoE

Document for:	Discussion

1	Introduction

Last RAN3 meeting primarily discusses RVQoE, and a bunch of agreements are achieved.

This contribution further discusses RAN visible QoE for NR QMC.

2	Discussion

For configuration and reporting of RAN visible QoE metrics, TR 38.890 has captured some general principles related to configuration and reporting which are listed as follows,

The following holds:

-	The RAN is responsible for assembling the RAN-visible QoE measurement configuration.

-	The RAN is responsible for triggering i.e. activating the RAN-visible QoE measurement.

-	Whether the RAN can explicitly ask the UE to report certain RAN-visible QoE metrics, or just an indication to report the fixed set of RAN-visible QoE metrics predefined per service type, is to be studied in the normative phase.

-	The RAN should be able to configure RAN-visible QoE autonomously for a given service type only if the application layer QoE for the same service type is already configured.

-	The RAN is not allowed to change the existing configuration of legacy QoE metrics specified by SA4.

Firstly, if RAN decides to activate RAN visible QoE measurement and informs UE AS, then UE AS needs to indicate to UE APP that RAN visible QoE measurement has been activated, and UE APP needs to send an extra copy of the indicated RAN visible QoE metrics to UE AS. As a result, SA4 needs to be consulted to confirm the feasibility of implementing such mechanism, and enhance the functionality if needed.

Secondly, there could be two types of reporting for legacy QoE report, event-triggered and periodic. If legacy reporting is periodic, it is sufficient for RAN visible QoE reports to be transferred together with legacy QoE report container from UE APP to UE AS, and then UE AS transmits RAN visible QoE reports to RAN.

Proposal 1: Upon RAN visible QoE measurement activation, UE AS indicates to UE APP that RAN visible QoE measurement has been triggered, potentially with RAN visible QoE metrics needed to be collected at UE APP as requested by RAN.

Proposal 2: Upon RAN visible QoE measurement deactivation, UE AS indicates to UE APP that RAN visible QoE measurement has been terminated, and then UE APP stops to provide RVQOE measurement results to UE AS.

In addition, last meeting we captured the following agreement that might be easily overlooked,

The UE is assumed to indicate to the RAN its capability with respect to providing RVQOE metrics (LS to RAN2 seems needed).

During last RAN3 meeting, we have achieve consensus that if RVQOE functionality is enabled, a UE capability is deemed to be introduced. So our understanding is that if the discussion on RVQOE is mature enough, the LS needs to be sent sooner than later. So we provide a draft LS as in the Appendix, to tell RAN2 that the discussion on UE capability could be initiated.

Proposal 3: Agree to send the LS on UE capability to RAN2.

3	Conclusion

This contribution discusses NR QoE management, and provides following proposals,

Proposal 1: Upon RAN visible QoE measurement activation, UE AS indicates to UE APP that RAN visible QoE measurement has been triggered, potentially with RAN visible QoE metrics needed to be collected at UE APP as requested by RAN.

Proposal 2: Upon RAN visible QoE measurement deactivation, UE AS indicates to UE APP that RAN visible QoE measurement has been terminated, and then UE APP stops to provide RVQOE measurement results to UE AS.

Proposal 3: Agree to send the LS on UE capability to RAN2.

References

- RP-193256 Study on NR QoE management and optimizations for diverse services

- RP-202151

Appendix: draft LS

3GPP TSG-RAN WG3 #113-e							                                              R3-21xxxx

16-26 Aug 2021

Online

Title:	[Draft] LS on introducing RVQOE related UE capabilities

Response to:

Release:	Rel-17

Work Item:	NR_QoE-Core

Source:	CMCC [to be RAN3]

To:	RAN2

Cc:

Contact person:

#### Name:	Xingyu HAN

#### Tel. Number:

#### E-mail Address:	hanxingyu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

RAN3 has discussed the topic on RAN Visible QoE (RVQOE), and identifies there might be a need to introduce RVQOE related UE capability, i.e. the UE is assumed to indicate to the RAN its capability with respect to providing RVQOE metrics.

# 2	Actions

To RAN2

ACTION: RAN3 respectfully asks RAN2 to consider the need and how to introduce RVQOE related UE capability.

# 3	Dates of next RAN3 meetings

3GPP TSG RAN3#114e		11/2021	E-Meeting

3GPP TSG RAN3#114bis-e		01/2022	E-Meeting