3GPP TSG-RAN WG3 Meeting #114-e	R3-214814

E-meeting, 1.-11.11.2021

Agenda item:	10.2.1.6

Source:	Nokia, Nokia Shanghai Bell

Title:	[TP to SON BL CR to 38.423, TP to SON BL CR to 36.423, NR_ENDC_SON_MDT_enh]  MRO for PSCell change failure

Document for:	Endorsement

# 1	Introduction

Discussions on MRO for SN Change Failure have been made at RAN3#109-e, RAN3#110-e, RAN3#111-e, RAN3#112-e and RAN3#113-e meetings, and several agreements were made.

In this document, we provide explanations, observations and proposals for the open issues and items left FFS.

In section 2, we discuss insufficiencies of the current MRO analysis mechanism for pre-Rel-17 UEs due to PSCell change without MN involvement, content of the new message from MN to initiating SN to forward SCG failure information, which scenarios to support MRO for SN change failure in Rel-17, and we clarify the other SCG failure case that was claimed to create ambiguity in the last meeting.

In section 3, we conclude the document by presenting the summary of the main ideas.

# 2	Discussion

## 2.1	MRO for SN Change Failure for Pre-Rel-17 UEs

The following agreement has been made in the RAN3#112-e meeting:

A class 2 procedure is defined for transmitting SCGFailureInformation from the MN to the SN that caused the failure, unless class-1 is found needed to resolve the issue of intra-SN PSCell change.

In addition, the following was discussed in the RAN3#113-e meeting the last time:

FFS how to support Intra-SN PSCell change after MN/SN initiated SN change for pre-R17 UEs.

In our view, which has also been agreed by several companies in the last meeting, additional mechanisms are needed for the support of MRO for pre-Rel-17 UEs that would still send the conventional SCG failure information message in case of an SCG failure. In particular, intra-SN PSCell change without MN involvement creates problems and this issue needs to be solved.

In a scenario where the MN initiates an SN change that is successfully completed for a UE, or where the SN initiates an inter-SN PSCell change, the MN would be aware of the PSCell change, since the MN is directly involved in those procedures. After this PSCell change, the serving SN may initiate independently another intra-SN PSCell change via SRB3 without MN involvement. This may occur, even if it is assumed that only one of the nodes should be responsible for SCG mobility, because measurement-based mobility may not be the only trigger for PSCell changes. For example, if one layer (in this scenario the SN) is responsible for “normal” mobility, the other layer (MN here) may decide to execute load-based or service-based SN change.

After the UE connects to the PSCell that is indicated by the serving SN via SRB3, this PSCell may fail. Based on the measurements reported from the UE, this scenario would conventionally be identified as a “PSCell change failure to wrong cell”, “too early PSCell change failure” or a “too late PSCell change failure” that is caused by the last serving SN, and the SN would make own configuration changes for MRO purposes based on the below agreement from RAN3#109-e meeting:

In case of an SCG failure that is a result of an SN-initiated PSCell change, the SN initiating the last PSCell change (or the last serving SN, in case of too late SN change) is responsible to derive the needed correction for its SCG mobility configuration.

However, since the MN is not aware of the intra-SN PSCell change initiated by the serving SN, upon reception of the SCG failure information sent by the UE, MN would assume the failure is related to the SN change that MN triggered itself (or source SN) and would not forward the SCG failure information to the last serving SN based on the current agreements – it would consider the report as a consequence of the earlier SN change. Even if the MN forwards the message to the serving SN for purposes other than MRO, the MN would make own configuration changes for MRO, as it would think that the SCG failure was caused by itself (or source SN). On top of that, last serving SN would also make own configuration changes for MRO. One of the two corrective actions are not necessary and may thus worsen the performance instead of improving it (also, the statistics, if collected, would be badly impacted). The high-level signalling chart of the exemplary scenario is provided in Figure 1.

Figure 1 High-level signalling chart of the described problem caused by PSCell change without MN involvement.

Thus, the considered mechanism based on a single information message sent if the MN classifies the failure as “belonging to the SN” is not sufficient for successful MRO operation at least under the given scenario and enhancements are needed.

Observation 1: A failed intra-SN PSCell change for a pre-Rel-17 UE, if done without MN’s involvement, may lead to a wrong MRO analysis for SN change failure.

Proposal 1: RAN3 studies mechanisms for successful MRO operation for SN change failure of pre-Rel-17 UEs in all scenarios.

During the discussions in RAN3#113-e meeting, there were some companies proposing that even in case of intra-SN PSCell change without MN involvement, the “guilty” node can be the MN. However, based on the previous agreements made, the “guilty” is the node that initiated the PSCell change. If there was an intra-SN PSCell change that failed, it is the SN that shall resolve the issue – as agreed before. Some companies were claiming that the MN can correctly identify the node by looking at the measurements and “some other information” in the MN. Firstly, what “other information” can the MN use have not been clarified by the companies. Also, the measurements are not all, neither the MN may be able to interpret them (in EN-DC).

Observation 2: Based on the previous agreements, in case of an SCG failure that is a result of an SN-initiated PSCell change, the SN initiating the last PSCell change is responsible to derive the needed correction for its SCG mobility configuration.

Observation 3: If there is PSCell change without MN involvement, the SN that has initiated the PSCell change without MN involvement performs MRO, in case of a subsequent SCG failure.

Observation 4: MN cannot correctly identify the guilty node by looking at the measurements and “some other information” in the MN, in case an intra-SN PSCell change without MN involvement is followed by an SCG failure.

As we have explained in the last meeting, one of the below proposed solutions can be used to overcome the above problem. Although we lean towards the first option that uses a class-1 message, option 2 that uses class-2 messages can also be adopted.

- Option-1:

MN always forwards the newly introduced message including SCG failure information to the last serving SN, and the last serving SN replies this message (as a response to class-1 procedure) indicating whether a PSCell change without MN involvement had been performed. MN uses the information to perform initial MRO analysis and further forwarding of the SCG failure information.

A high-level signalling chart for an exemplary scenario for this option is provided in Figure 2. Herein, it is assumed that the last PSCell change initiating node is a source SN (other than the last serving SN). As illustrated, firstly, the MN forwards the Rel-16 SCG failure information message to the last serving SN upon reception of the message from the UE. This message is needed to notify the SN about the failure. Also, if the SN is a Rel-16 implementation, it would fail to receive the newly introduced Rel-17 message, which is another reason why the Rel-16 message is always needed. Afterwards, MN transmits the newly introduced message to the SN for SON purposes. SN replies this message indicating whether a PSCell change without MN involvement had been performed.

In case the last serving SN indicates an intra-SN PSCell change without MN involvement, the MN does not need to transmit the newly introduced message to the source SN, since the failure is caused by the last serving SN. The last serving SN can directly perform MRO. On the other hand, in case no PSCell change without MN involvement was made, the MN can transmit the newly introduced message to the source SN and the source SN can perform MRO. Note that a reply is not needed from the source SN at this step.

Figure 2 High-level signalling chart of the Option-1 for an exemplary scenario.

- Option-2:

Rel-16 RRC transfer message, including the SCG failure information, is enhanced. MN transmits the updated message to the last serving SN upon reception of the SCG failure information message from the UE. SN can indicate whether it has initiated any intra-SN PSCell change without MN involvement using a class-2 procedure. If there was an intra-SN PSCell change without MN involvement, the MN can further transmit the newly introduced message to the last serving SN for the SN to perform MRO. If there was not a PSCell change without MN involvement, MN uses the information to perform initial MRO analysis and further forwarding of the SCG failure information.

A high-level signalling chart for an exemplary scenario for this option is provided in Figure 3. Again, it is assumed that the last PSCell change initiating node is a source SN (other than the last serving SN).

Figure 3 High-level signalling chart of the Option-2 for an exemplary scenario.

Proposal 2: RAN3 adopts a solution (option-1 introduced within this contribution) where the MN always forwards the SCG failure information to the last serving SN (before making the initial analysis). Then, the last serving SN replies this message (as a response to class-1 procedure) indicating whether a PSCell change without MN involvement had been performed.

At RAN3#113-e meetings, following were agreed to be included in the new message to support pre-Rel-17 UEs:

b)	Source PSCell CGI, if available in MN

c)	Failed PSCell CGI, if available in MN

Regarding source / failed PSCell CGI, the MN is aware what is the PSCell for a given UE only when the SN informs about each PSCell change. This is possible, but dependent on another optional feature. Therefore, it can’t be assumed the MN knows the source PSCell always. It should be noted that the problem is not only there when intra-SN PSCell change without MN involvement occurs, as there had been some misunderstanding by the companies in the last meeting.

In case of NR-DC, if the MN is to know the PSCell, it may always read the SCG configuration provided by target SN. However, assuming such implementation would be abusive, because the design of the MR-DC is made so to spare the MN from this requirement. In case of EN-DC, the MN may not even be able to read the SCG configuration.

Observation 5: The MN is not aware what is the PSCell for a given UE, since the SN is the node that controls SCG mobility (independent from whether any intra-SN PSCell change happened).

Observation 6: For NR-DC, the assumption that the MN always reads the SCG configuration provided by target SN to learn about the PSCell for a given UE would be abusive.

Observation 7: For EN-DC, the MN may not even be able to read the SCG configuration.

In our view, what we proposed (Option-1 and Option-2) above regarding PSCell change without MN involvement, can perfectly be used also for the MN to learn about (c), i.e., failed PSCell CGI. The MN always forwards the SCG failure information to the last serving SN, and the last serving SN replies this message indicating the failed PSCell CGI, i.e., (c).

Proposal 3: In order for the MN to know about (c), i.e., failed PSCell CGI, RAN3 adopts the solution (option-1 introduced within this contribution) where the MN always forwards the SCG failure information to the last serving SN. Then, the last serving SN replies this message indicating the failed PSCell CGI.

In case of too late handover, (b) and (c) are already known by the source / last serving SN. For Rel-17 UEs, those IEs are already reported by the UE. In case of too early handover and handover to a wrong cell, if the UE context is still there in the source SN, (b) is not needed in the message from MN to the source SN. RAN3 should work on solutions for the MN to be informed about (b) in a scenario where the source SN removes the UE context.

Proposal 4: RAN3 works on mechanisms for the MN to be informed about (b), for the scenario where the source SN removes the UE context (and source SN should be informed about it by the MN).

Furthermore, at RAN3#113, following was agreed:

If the sufficient time has passed between the SN change and the report of SCG failure, the source SN may has released the UE context when it receives SCG Failure Information.

First of all, in our view, if the UE context is removed by the source SN, which is anyway a corner case, then MRO cannot be performed. Thus, we should assume that the UE context is there in the source SN, if the source SN performs MRO. In the LTE MRO, we enabled the reference to the UE context by C-RNTI even though the source could have deleted it.

Proposal 5: As it was required in LTE MRO, keeping the UE context for the short while after completing SN change may be a mandatory part of the MRO for SN change.

From the list of proposed information, following has not been discussed much at RAN3#113:

e) Mobility Information

g) Initiating node type i.e. MN or SN

h) S-NG-RAN node UE X2AP ID

i) M-NG-RAN node UE XnAP ID

As elaborated previously in this section, (g) is needed to support MRO for pre-Rel-17 UEs. The MN can include this information, and the last serving SN can indicate whether the understanding of the MN is correct, or whether any PSCell change without MN involvement has been performed.

Proposal 6: (g) is included in the new XnAP message for carrying SCGfailureinformation.

Moreover, in case the MN concludes after an initial analysis that this is a “too late PSCell change”, then it would mean that the UE context is still there in the last serving SN. On the other hand, it is not expected for the source SN and the MN to delete the UE’s XnAP IDs and assign to another UE, since the SCG failure and the following procedures will be occurring relatively fast in case of “PSCell change to wrong PSCell” and “too early PSCell change”. If the UE context is deleted by the source SN, as stated above, we should not expect the source SN to perform MRO. Therefore, (e) is not needed to be forwarded to the initiating SN, rather (h) and (i) are included in the new message.

Proposal 7: (e) is not forwarded to the initiating SN, whereas (h) and (i) are used to identify the UE.

## 2.2	Scenarios for MRO for SN Change Failure

At RAN3#113-e meeting, some companies proposed to limit the scope of the WI to NR-DC scenarios. The following was FFS after discussions:

FFS whether to support EN-DC and NR-DC scenarios for SN change failure in Rel-17.

In our view, both NR-DC and EN-DC scenarios shall be covered, where the priority can be on NR-DC. However, considering the time limitation, we are also fine to focus on a gNB / en-gNB first, but eventually, limiting the scenario to address should be consulted with the plenary.

Proposal 8: Limiting the scenario to address is consulted with the plenary.

## 2.3	Ambiguity in SCG Failure Cases

At RAN3#113-e meeting, [3] proposed a new scenario which presumably creates ambiguity:

After the RRC reconfiguration message for an SN change initiated by SN is received by a UE, a random access related procedure fails at the serving PSCell due to some other reason, e.g. for timing synchronization. So, an SCG failure report is produced. It is claimed that the report would confuse the source SN confused, as it presumably cannot determine whether the report is for SN change failure or other random access related procedure.

In our view, such problem does not exist, at least because of:

- If the UE context release request sent from the DU to the CU has been received by the CU, it is not an SN change failure, rather an RA failure.

- If the RRC reconfiguration message has been received by the UE, all other RA procedures should be stopped.

- For Rel-17 UEs, target PSCell CGI would be included in SCG failure report, which would indicate the procedure that failed.

- Failure type within SCG failure information would be set to synchReconfigFailureSCG, in case of SN change failure, whereas it would be set to randomAccessProblem, in case of RA related failure.

Proposal 9: The ambiguity proposed by [1] does not exist, because of several reasons mentioned in this document.

## 2.4	Scenarios for SCG MRO

In an LS sent to RAN3 [2], RAN2 asks about the scenarios where the SCG MRO may be used:

RAN2 is working on detailed signalling to support such report. Furthermore, RAN2 would like to ask RAN3’s confirmation on the necessary scenarios of such report. More specifically, RAN2 kindly requests RAN3 to confirm whether all (and if not which of) the following scenarios need to be supported for MRO in SCG Failure Report:

-	NR-DC,

-	NE-DC,

-	EN-DC, NGEN-DC.

RAN3 has not made any limitations in the work on the SCG MRO hitherto, so there is no reason to limit the scenarios at this moment. Neither the WI limits the scope of supportable scenarios.

Proposal 10: RAN3 responds to RAN2 that all of the listed scenarios are expected to be supported.

# 3	Conclusions

In this contribution, we have discussed various aspects of MRO for SN change failure. From those discussions we have the following observations:

Observation 1: A failed intra-SN PSCell change for a pre-Rel-17 UE, if done without MN’s involvement, may lead to a wrong MRO analysis for SN change failure.

Observation 2: Based on the previous agreements, in case of an SCG failure that is a result of an SN-initiated PSCell change, the SN initiating the last PSCell change is responsible to derive the needed correction for its SCG mobility configuration.

Observation 3: If there is PSCell change without MN involvement, the SN that has initiated the PSCell change without MN involvement performs MRO, in case of a subsequent SCG failure.

Observation 4: MN cannot correctly identify the guilty node by looking at the measurements and “some other information” in the MN, in case an intra-SN PSCell change without MN involvement is followed by an SCG failure.

Observation 5: The MN is not aware what is the PSCell for a given UE, since the SN is the node that controls SCG mobility (independent from whether any intra-SN PSCell change happened).

Observation 6: For NR-DC, the assumption that the MN always reads the SCG configuration provided by target SN to learn about the PSCell for a given UE would be abusive.

Observation 7: For EN-DC, the MN may not even be able to read the SCG configuration.

According to those observations we have the following proposals:

Proposal 1: RAN3 studies mechanisms for successful MRO operation for SN change failure of pre-Rel-17 UEs in all scenarios.

Proposal 2: RAN3 adopts a solution (option-1 introduced within this contribution) where the MN always forwards the SCG failure information to the last serving SN (before making the initial analysis). Then, the last serving SN replies this message (as a response to class-1 procedure) indicating whether a PSCell change without MN involvement had been performed.

Proposal 3: In order for the MN to know about (c), i.e., failed PSCell CGI, RAN3 adopts the solution (option-1 introduced within this contribution) where the MN always forwards the SCG failure information to the last serving SN. Then, the last serving SN replies this message indicating the failed PSCell CGI.

Proposal 4: RAN3 works on mechanisms for the MN to be informed about (b), for the scenario where the source SN removes the UE context (and source SN should be informed about it by the MN).

Proposal 5: As it was required in LTE MRO, keeping the UE context for the short while after completing SN change may be a mandatory part of the MRO for SN change.

Proposal 6: (g) is included in the new XnAP message for carrying SCGfailureinformation.

Proposal 7: (e) is not forwarded to the initiating SN, whereas (h) and (i) are used to identify the UE.

Proposal 8: Limiting the scenario to address is consulted with the plenary.

Proposal 9: The ambiguity proposed by [1] does not exist, because of several reasons mentioned in this document.

The above proposals are implemented in TPs for Xn and X2 below.

Furthermore, following proposal is made concerning the LS received from RAN2 [2]:

Proposal 10: RAN3 responds to RAN2 that all of the listed scenarios are expected to be supported.

# References

- R3-213817, RAN3 #113e

- R2-2109208, RAN2 #115e

# Text proposal for TS 38.423

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| S-NG-RAN node Addition Preparation | S-NODE ADDITION REQUEST | S-NODE ADDITION REQUEST ACKNOWLEDGE | S-NODE ADDITION REQUEST REJECT |
| M-NG-RAN node initiated S-NG-RAN node Modification Preparation | S-NODE MODIFICATION REQUEST | S-NODE MODIFICATION REQUEST ACKNOWLEDGE | S-NODE MODIFICATION REQUEST REJECT |
| S-NG-RAN node initiated S-NG-RAN node Modification | S-NODE MODIFICATION REQUIRED | S-NODE MODIFICATION CONFIRM | S-NODE MODIFICATION REFUSE |
| S-NG-RAN node initiated S-NG-RAN node CHANGE | S-NODE CHANGE REQUIRED | S-NODE CHANGE CONFIRM | S-NODE CHANGE REFUSE |
| M-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUEST | S-NODE RELEASE REQUEST ACKNOWLEDGE | S-NODE RELEASE REJECT |
| S-NG-RAN node initiated S-NG-RAN node Release | S-NODE RELEASE REQUIRED | S-NODE RELEASE CONFIRM |  |
| Xn Setup | XN SETUP REQUEST | XN SETUP RESPONSE | XN SETUP FAILURE |
| NG-RAN node Configuration Update | NG-RAN NODE CONFIGURATION UPDATE | NG-RAN NODE CONFIGURATION UPDATE ACKNOWLEDGE | NG-RAN NODE CONFIGURATION UPDATE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| Xn Removal | Xn REMOVAL REQUEST | Xn REMOVAL RESPONSE | Xn REMOVAL FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| SCG MRO Information | SCG MRO INFORMATION | SCG MRO CONFIRM |  |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| RAN Paging | RAN PAGING |
| Xn-U Address Indication | XN-U ADDRESS INDICATION |
| S-NG-RAN node Reconfiguration Completion | S-NODE RECONFIGURATION COMPLETE |
| S-NG-RAN node Counter Check | S-NODE COUNTER CHECK REQUEST |
| UE Context Release | UE CONTEXT RELEASE |
| RRC Transfer | RRC TRANSFER |
| Error Indication | ERROR INDICATION |
| Notification Control Indication | NOTIFICATION CONTROL INDICATION |
| Activity Notification | ACTIVITY NOTIFICATION |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| Failure Indication | FAILURE INDICATION |
| Handover Report | HANDOVER REPORT |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Access And Mobility Indication | ACCESS AND MOBILITY INDICATION |

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

### 8.3.A1	SCG MRO Information

#### 8.3.A1.1	General

The purpose of the SCG MRO Information procedure is to provide information needed for executing MRO analysis related to an SCG failure for a specific UE.

The procedure uses UE-associated signalling.

#### 8.3.A1.2	Successful Operation

Figure 8.3.A1.2-1: SCG MRO Information, successful operation

The M-NG-RAN node initiates the procedure by sending the SCG MRO INFORMATION message to the S-NG-RAN node.

The M-NG-RAN node includes the Initiating Node IE in the SCG MRO INFORMATION message to provide the information about the node that initiated the last PSCell change, according to the information available at the M-NG-RAN node. If the reported failure concerns a recently executed PSCell change initiated by the S-NG-RAN node that the M-NG-RAN node is not aware of, the S-NG-RAN node indicates it in the Initiating Node IE included in the SCG MRO CONFIRM message.

#### 8.3.A1.3	Unsuccessful Operation

Not applicable.

#### 8.3.A1.4	Abnormal Conditions

Void.

//////////////////////////////////////////////////////////////irrelevant operations skipped/////////////////////////////////////////////////////////////////////

#### 9.1.2.A2	SCG MRO INFORMATION

This message is sent by the M-NG-RAN node to the S-NG-RAN node to inform about an SCG failure and to provide information for MRO analysis.

Direction: M-NG-RAN node  S-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Initiating node | M |  | ENUMERATED (mn, sn, ...) |  | YES | ignore |
| UE Report |  | 0..1 |  |  | YES | reject |
| >RRC Container | M |  | OCTET STRING | For NGEN-DC and NR-DC, includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [10] containing the FailureInformation message. | – |  |

#### 9.1.2.A2	SCG MRO CONFIRM

This message is sent by the S-NG-RAN node to provide the M-NG-RAN with additional information related to the SCg MRO analysis.

Direction: S-NG-RAN node  M-NG-RAN node.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.3.1 |  | YES | reject |
| M-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the M-NG-RAN node | YES | reject |
| S-NG-RAN node UE XnAP ID | M |  | NG-RAN node UE XnAP ID 9.2.3.16 | Allocated at the S-NG-RAN node | YES | reject |
| Initiating node | M |  | ENUMERATED (mn, sn, ...) |  | YES | ignore |
| Failure cell ID | M |  | NG-RAN Cell Identity 9.2.2.9 | Cell in which UE failed | YES | Ignore |

============ Start of ASN.1 change ==============

[ASN.1 to be added once endorsed.]

# Text proposal for TS 36.423

The below text proposal is based on the endorsed BL CR in [R3-211498]. The ASN.1 will be added once the concept of the change is endorsed.

////////////////////////////////////////////////////////////////////////Change Start ///////////////////////////////////////////////////////////////////////////

## 8.1	Elementary procedures

In the following tables, all EPs are divided into Class 1 and Class 2 EPs.

Table 8.1-1: Class 1 Elementary Procedures

| Elementary Procedure | Initiating Message | Successful Outcome | Unsuccessful Outcome |  |
| --- | --- | --- | --- | --- |
|  |  | Response message | Response message |  |
| Handover Preparation | HANDOVER REQUEST | HANDOVER REQUEST ACKNOWLEDGE | HANDOVER PREPARATION FAILURE |
| Reset | RESET REQUEST | RESET RESPONSE |  |
| X2 Setup | X2 SETUP REQUEST | X2 SETUP RESPONSE | X2 SETUP FAILURE |
| eNB Configuration Update | ENB CONFIGURATION UPDATE | ENB CONFIGURATION UPDATE ACKNOWLEDGE | ENB CONFIGURATION UPDATE FAILURE |
| Resource Status Reporting Initiation | RESOURCE STATUS REQUEST | RESOURCE STATUS RESPONSE | RESOURCE STATUS FAILURE |
| Mobility Settings Change | MOBILITY CHANGE REQUEST | MOBILITY CHANGE ACKNOWLEDGE | MOBILITY CHANGE FAILURE |
| Cell Activation | CELL ACTIVATION REQUEST | CELL ACTIVATION RESPONSE | CELL ACTIVATION FAILURE |
| SeNB Addition Preparation | SENB ADDITION REQUEST | SENB ADDITION REQUEST ACKNOWLEDGE | SENB ADDITION REQUEST REJECT |
| MeNB initiated SeNB Modification Preparation | SENB MODIFICATION REQUEST | SENB MODIFICATION REQUEST ACKNOWLEDGE | SENB MODIFICATION REQUEST REJECT |
| SeNB initiated SeNB Modification | SENB MODIFICATION REQUIRED | SENB MODIFICATION CONFIRM | SENB MODIFICATION REFUSE |
| SeNB initiated SeNB Release | SENB RELEASE REQUIRED | SENB RELEASE CONFIRM |  |
| X2 Removal | X2 REMOVAL REQUEST | X2 REMOVAL RESPONSE | X2 REMOVAL FAILURE |
| Retrieve UE Context | RETRIEVE UE CONTEXT REQUEST | RETRIEVE UE CONTEXT RESPONSE | RETRIEVE UE CONTEXT FAILURE |
| SgNB Addition Preparation | SGNB ADDITION REQUEST | SGNB ADDITION REQUEST ACKNOWLEDGE | SGNB ADDITION REQUEST REJECT |
| MeNB initiated SgNB Modification Preparation | SGNB MODIFICATION REQUEST | SGNB MODIFICATION REQUEST ACKNOWLEDGE | SGNB MODIFICATION REQUEST REJECT |
| SgNB initiated SgNB Modification | SGNB MODIFICATION REQUIRED | SGNB MODIFICATION CONFIRM | SGNB MODIFICATION REFUSE |
| SgNB change | SGNB CHANGE REQUIRED | SGNB CHANGE CONFIRM | SGNB CHANGE REFUSE |
| MeNB initiated SgNB Release | SGNB RELEASE REQUEST | SGNB RELEASE REQUEST ACKNOWLEDGE | SGNB RELEASE REQUEST REJECT |
| SgNB initiated SgNB Release | SGNB RELEASE REQUIRED | SGNB RELEASE CONFIRM |  |
| EN-DC X2 Setup | EN-DC X2 SETUP REQUEST | EN-DC X2 SETUP RESPONSE | EN-DC X2 SETUP FAILURE |
| EN-DC Configuration Update | EN-DC CONFIGURATION UPDATE | EN-DC CONFIGURATION UPDATE ACKNOWLEDGE | EN-DC CONFIGURATION UPDATE FAILURE |
| EN-DC Cell Activation | EN-DC CELL ACTIVATION REQUEST | EN-DC CELL ACTIVATION RESPONSE | EN-DC CELL ACTIVATION FAILURE |
| E-UTRA - NR Cell Resource Coordination | E-UTRA - NR CELL RESOURCE COORDINATION REQUEST | E-UTRA - NR CELL RESOURCE COORDINATION RESPONSE |  |
| EN-DC X2 Removal | EN-DC X2 REMOVAL REQUEST | EN-DC X2 REMOVAL RESPONSE | EN-DC X2 REMOVAL FAILURE |
| EN-DC Resource Status Reporting Initiation | EN-DC RESOURCE STATUS REQUEST | EN-DC RESOURCE STATUS RESPONSE | EN-DC RESOURCE STATUS FAILURE |
| UE Radio Capability ID Mapping | UE RADIO CAPABILITY ID MAPPING REQUEST | UE RADIO CAPABILITY ID MAPPING RESPONSE |  |
| SCG MRO information | SCG MRO INFORMATION | SCG MRO CONFIRM |  |

Table 8.1-2: Class 2 Elementary Procedures

| Elementary Procedure | Initiating Message |
| --- | --- |
| Load Indication | LOAD INFORMATION |
| Handover Cancel | HANDOVER CANCEL |
| SN Status Transfer | SN STATUS TRANSFER |
| UE Context Release | UE CONTEXT RELEASE |
| Resource Status Reporting | RESOURCE STATUS UPDATE |
| Error Indication | ERROR INDICATION |
| Radio Link Failure Indication | RLF INDICATION |
| Handover Report | HANDOVER REPORT |
| X2 Release | X2 RELEASE |
| X2AP Message Transfer | X2AP MESSAGE TRANSFER |
| SeNB Reconfiguration Completion | SENB RECONFIGURATION COMPLETE |
| MeNB initiated SeNB Release | SENB RELEASE REQUEST |
| SeNB Counter Check | SENB COUNTER CHECK REQUEST |
| SgNB Reconfiguration Completion | SGNB RECONFIGURATION COMPLETE |
| SgNB Counter Check | SGNB COUNTER CHECK REQUEST |
| RRC Transfer | RRC TRANSFER |
| Secondary RAT Data Usage Report | SECONDARY RAT DATA USAGE REPORT |
| SgNB Activity Notification | SGNB ACTIVITY NOTIFICATION |
| Data Forwarding Address Indication | DATA FORWARDING ADDRESS INDICATION |
| gNB Status Indication | GNB STATUS INDICATION |
| EN-DC Configuration Transfer | EN-DC CONFIGURATION TRANSFER |
| Trace Start | TRACE START |
| Deactivate Trace | DEACTIVATE TRACE |
| Handover Success | HANDOVER SUCCESS |
| Conditional Handover Cancel | CONDITIONAL HANDOVER CANCEL |
| Early Status Transfer | EARLY STATUS TRANSFER |
| EN-DC Resource Status Reporting | EN-DC RESOURCE STATUS UPDATE |
| Cell Traffic Trace | CELL TRAFFIC TRACE |
| F1-C Traffic Transfer | F1-C TRAFFIC TRANSFER |

////////////////////////////////////////////////////////////////////////Next Start ///////////////////////////////////////////////////////////////////////////

### 8.7.A1	SCG MRO Information

#### 8.7.A1.1	General

The purpose of the SCG MRO Information procedure is to provide information needed for executing MRO analysis related to an SCG failure for a specific UE.

The procedure uses UE-associated signalling.

#### 8.7.A1.2	Successful Operation

Figure 8.7.A1.2-1: SCG MRO Information, successful operation

The MeNB initiates the procedure by sending the SCG MRO INFORMATION message to the en-gNB.

The MeNB includes the Initiating Node IE in the SCG MRO INFORMATION message to provide the information about the node that initiated the last PSCell change, according to the information available at the MeNB. If the reported failure concerns a recently executed PSCell change initiated by the en-gNB that the MeNB is not aware of, the en-gNB indicates it in the Initiating Node IE included in the SCG MRO CONFIRM message.

#### 8.7.A1.3	Unsuccessful Operation

Not applicable.

#### 8.7.A1.4	Abnormal Conditions

Void.

////////////////////////////////////////////////////////////////////////Next Start ///////////////////////////////////////////////////////////////////////////

#### 9.1.2.A2	SCG MRO INFORMATION

This message is sent by the MeNB to the en-gNB to inform about an SCG failure and to provide information for MRO analysis.

Direction: MeNB  en-gNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Initiating node | M |  | ENUMERATED (mn, sn, ...) |  | YES | ignore |
| UE Report |  | 0..1 |  |  | YES | reject |
| >RRC Container | M |  | OCTET STRING | Includes the UL-DCCH-Message as defined in subclause 6.2.1 of TS 38.331 [31] containing the FailureInformation message. | – |  |

#### 9.1.2.A2	SCG MRO CONFIRM

This message is sent by the en-gNB to provide the M-NG-RAN with additional information related to the SCg MRO analysis.

Direction: en-gNB  MeNB.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| Message Type | M |  | 9.2.13 |  | YES | reject |
| MeNB UE X2AP ID | M |  | eNB UE X2AP ID 9.2.24 | Allocated at the MeNB | YES | reject |
| SgNB UE X2AP ID | M |  | en-gNB UE X2AP ID 9.2.100 | Allocated at the en-gNB | YES | reject |
| MeNB UE X2AP ID Extension | O |  | Extended eNB UE X2AP ID 9.2.86 | Allocated at the MeNB. | YES | reject |
| Initiating node | M |  | ENUMERATED (mn, sn, ...) |  | YES | ignore |
| Failure cell ID | M |  | ECGI 9.2.14 | Cell in which UE failed | YES | Ignore |

////////////////////////////////////////////////////////////////////////Next Start ///////////////////////////////////////////////////////////////////////////

[ASN.1 to be added once endorsed.]